3GPP TSG-RAN WG2 Meeting #97bis	R2-1703952

Spokane, USA, 3 - 7 April 2017

Agenda item:	10.2.1

Source:	Rapporteur (Nokia)

Title:	NG-RAN Stage 2

WID/SID:	NR_newRAT - Release 15

Document for:	Discussion and Decision

# 1	Introduction

The following changes have been incorporated to TS 38.300:

-	Stage 2 Details of ARQ operation makred as FFS;

-	Placeholder for CU/DU Split overview added;

-	Outdated editor notes removes;

-	Protocol Architecture updated;

-	NG-RAN terminology aligned;

-	Header placement in the L2 overview put as FFS;

- 	Desciption of measurements for mobility clarified;

- 	Some cell reselection details put FFS;

-	Outdated references removed;

The following issues have been indentified:

-	RRC state description: NG-RAN is used (as in the TR) but it is not clear whether the agreements apply to E-UTRAN connected to 5GC or not:

## 7.2	Protocol States

RRC supports the following states which can be characterised as follows:

-	RRC_IDLE:

-	PLMN selection;

-	DRX configured by NAS;

-	Broadcast of system information;

-	Cell re-selection mobility;

-	Paging (initiated and area managed by 5GC);

FFS whether the UE AS context is not stored in any gNB or in the UE.

-	RRC_INACTIVE:

-	Cell re-selection mobility;

-	5GC - NG-RAN connection (both C/U-planes) is established for UE;

-	The UE AS context is stored in at least one gNB and the UE;

-	Paging is initiated by NG-RAN;

-	RAN-based notification area is managed by NG- RAN;

-	NG-RAN knows the RAN-based notification area which the UE belongs to;

FFS if data transmission in possible in INACTIVE

-	RRC_CONNECTED:

-	The UE has an NG-RAN RRC connection;

-	The UE has an AS context in NR;

-	NG-RAN knows the cell which the UE belongs to;

-	Transfer of unicast data to/from the UE;

-	Network controlled mobility including measurements.

-	Description of MAC functionalities: the text in 6.2.1 reads “Multiplexing/demultiplexing of MAC SDUs belonging to one or different logical channels into/from transport blocks (TB) delivered to/from the physical layer on transport channels;” but the figures describes concatenation separetly. We either need to remove concatenation from the figure or ammend the description in 6.2.1 to distinguish multiplexing of bearers from concatenation, for instance::

-	Concatenation of MAC SDUs belonging to one logical channel;

-	Multiplexing/demultiplexing of MAC SDUs belonging to different logical channels into/from transport blocks (TB) delivered to/from the physical layer on transport channels;