3GPP TS 38.323 V0.0.5 (2017-05)

Technical Specification

3rd Generation Partnership Project;

Technical Specification Group Radio Access Network;

NR;

Packet Data Convergence Protocol (PDCP) specification

(Release 15	)

The present document has been developed within the 3rd Generation Partnership Project (3GPP TM) and may be further elaborated for the purposes of 3GPP..
The present document has not been subject to any approval process by the 3GPP Organizational Partners and shall not be implemented.
This Specification is provided for future development work within 3GPP only. The Organizational Partners accept no liability for any use of this Specification.
Specifications and Reports for implementation of the 3GPP TM system should be obtained via the 3GPP Organizational Partners' Publications Offices.

Keywords

NR, radio

3GPP

Postal address

3GPP support office address

650 Route des Lucioles - Sophia Antipolis

Valbonne - FRANCE

Tel.: +33 4 92 94 42 00 Fax: +33 4 93 65 47 16

Internet

http://www.3gpp.org

Copyright Notification

No part may be reproduced except as authorized by written permission.
The copyright and the foregoing restriction extend to reproduction in all media.

© 2017, 3GPP Organizational Partners (ARIB, ATIS, CCSA, ETSI, TSDSI, TTA, TTC).

All rights reserved.

UMTS™ is a Trade Mark of ETSI registered for the benefit of its members

3GPP™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners
LTE™ is a Trade Mark of ETSI registered for the benefit of its Members and of the 3GPP Organizational Partners

GSM® and the GSM logo are registered and owned by the GSM Association

Contents

Foreword	5

1	Scope	6

2	References	6

3	Definitions and abbreviations	6

3.1	Definitions	6

3.2	Abbreviations	7

4	General	7

4.1	Introduction	7

4.2	Architecture	7

4.2.1	PDCP structure	7

4.2.2	PDCP entities	8

4.3	Services	9

4.3.1	Services provided to upper layers	9

4.3.2	Services expected from lower layers	9

4.4	Functions	10

5	Procedures	10

5.1	PDCP entity handling	10

5.1.1	PDCP entity establishment	10

5.1.2	PDCP entity re-establishment	10

5.1.3	PDCP entity release	11

5.2	Data transfer	11

5.2.1	Transmit operation	11

5.2.2	Receive operation	12

5.3	SDU discard	12

5.4	Status reporting	12

5.4.1	Transmit operation	12

5.4.2	Receive operation	12

5.5	Data recovery	12

5.6	Data volume calculation	13

5.7	Header compression and decompression	13

5.7.1	Supported header compression protocols and profiles	13

5.7.2	Configuration of header compression	13

5.7.3	Protocol parameters	13

5.7.4	Header compression	14

5.7.5	Header decompression	14

5.7.6	PDCP Control PDU for interspersed ROHC feedback	14

5.7.6.1	Transmit Operation	14

5.7.6.2	Receive Operation	14

5.8	Ciphering and deciphering	15

5.9	Integrity protection and verification	15

5.10	Handling of unknown, unforeseen, and erroneous protocol data	16

6	Protocol data units, formats, and parameters	16

6.1	Protocol data units	16

6.1.1	Data PDU	16

6.1.2	Control PDU	16

6.2	Formats	16

6.2.1	General	16

6.2.2	Data PDU	16

6.2.2.1	Data PDU with 12 bits PDCP SN	16

6.2.2.2	Data PDU with 18 bits PDCP SN	17

6.2.3	Control PDU	17

6.2.3.1	Control PDU for PDCP status report	17

6.2.3.2	Control PDU for interspersed ROHC feedback	18

6.3	Parameters	18

6.3.1	General	18

6.3.2	PDCP SN	18

6.3.3	Data	18

6.3.4	MAC-I	18

6.3.5	COUNT	18

6.3.6	R	19

6.3.7	D/C	19

6.3.8	PDU type	19

6.3.9	FMC	19

6.3.10	Bitmap	19

6.3.11	Interspersed ROHC feedback	20

6.3.12	P	20

7	State variables, constants, and timers	20

7.1	State variables	20

7.2	Constants	21

7.3	Timers	21

Annex A (informative): Change history	22

Annex B (informative; to be removed later):  Agreements related to PDCP specification	22

Foreword

This Technical Specification has been produced by the 3rd Generation Partnership Project (3GPP).

The contents of the present document are subject to continuing work within the TSG and may change following formal TSG approval. Should the TSG modify the contents of the present document, it will be re-released by the TSG with an identifying change of release date and an increase in version number as follows:

Version x.y.z

where:

x	the first digit:

1	presented to TSG for information;

2	presented to TSG for approval;

3	or greater indicates TSG approved document under change control.

y	the second digit is incremented for all changes of substance, i.e. technical enhancements, corrections, updates, etc.

z	the third digit is incremented when editorial only changes have been incorporated in the document.

1	Scope

The present document provides the description of the Packet Data Convergence Protocol (PDCP).

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 38.300: "NG Radio Access Network; Overall description".

[3]	3GPP TS 38.331: "NR Radio Resource Control (RRC); Protocol Specification".

[4]	3GPP TS 38.321: "NR Medium Access Control (MAC) protocol specification".

[5]	3GPP TS 38.322: "NR Radio Link Control (RLC) protocol specification".

[6]	3GPP TS 33.401: "3GPP System Architecture Evolution: Security Architecture".

[7]	IETF RFC 4995: "The RObust Header Compression (ROHC) Framework".

[8]	IETF RFC 3095: "RObust Header Compression (ROHC): Framework and four profiles: RTP, UDP, ESP and uncompressed".

[9]	IETF RFC 4815: "RObust Header Compression (ROHC): Corrections and Clarifications to RFC 3095".

3	Definitions and abbreviations

3.1	Definitions

For the purposes of the present document, the terms and definitions given in [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in [1].

AM DRB: a data radio bearer which utilizes RLC AM.

Non-split bearer: a bearer whose radio protocols are located in either the MgNB or the SgNB to use MgNB or SgNB resource, respectively.

Split bearer: in dual connectivity, a bearer whose radio protocols are located in both the MgNB and the SgNB to use both MgNB and SgNB resources.

UM DRB: a data radio bearer which utilizes RLC UM.

3.2	Abbreviations

For the purposes of the present document, the abbreviations given in [1] and the following apply. An abbreviation defined in the present document takes precedence over the definition of the same abbreviation, if any, in [1].

AM	Acknowledged Mode

ARP	Address Resolution Protocol

CID	Context Identifier

DRB	Data Radio Bearer carrying user plane data

gNB	NR Node B

HFN	Hyper Frame Number

IETF	Internet Engineering Task Force

IP	Internet Protocol

L2	Layer 2 (data link layer)

L3	Layer 3 (network layer)

MAC	Medium Access Control

MAC-I	Message Authentication Code for Integrity

MCG	Master Cell Group

NG-RAN	NR Radio Access Network

PDCP	Packet Data Convergence Protocol

PDU	Protocol Data Unit

RB	Radio Bearer

RFC	Request For Comments

RLC	Radio Link Control

ROHC	RObust Header Compression

RRC	Radio Resource Control

RTP	Real Time Protocol

SAP	Service Access Point

SCG	Secondary Cell Group

SDU	Service Data Unit

SN	Sequence Number

SRB	Signalling Radio Bearer carrying control plane data

TCP	Transmission Control Protocol

UDP	User Datagram Protocol

UE	User Equipment

UM	Unacknowledged Mode

X-MAC	Computed MAC-I

4	General

4.1	Introduction

The present document describes the functionality of the PDCP.

4.2	Architecture

4.2.1	PDCP structure

Figure 4.2.1.1 represents one possible structure for the PDCP sublayer; it should not restrict implementation. The figure is based on the radio interface protocol architecture defined in [2].

Figure 4.2.1-1: PDCP layer, structure view

The PDCP sublayer is configured by upper layers [3]. The PDCP sublayer is used for RBs mapped on DCCH and DTCH type of logical channels. The PDCP sublayer is not used for any other type of logical channels.

Each RB (except for SRB0) is associated with one PDCP entity. Each PDCP entity is associated with one or two RLC entities depending on the RB characteristic (e.g uni-directional/bi-directional or split/non-split) or RLC mode. For non-split bearers, each PDCP entity is associated with one UM RLC entity, two UM RLC entities (one for each direction), or one AM RLC entity. For split bearers, each PDCP entity is associated with two UM RLC entities (one for each direction), four UM RLC entities (two for each direction), or two AM RLC entities (for same direction).

4.2.2	PDCP entities

The PDCP entities are located in the PDCP sublayer. Several PDCP entities may be defined for a UE. Each PDCP entity carrying user plane data may be configured to use header compression.

Each PDCP entity is carrying the data of one radio bearer. In this version of the specification, only the robust header compression protocol (ROHC) is supported. Every PDCP entity uses at most one ROHC compressor instance and at most one ROHC decompressor instance.

A PDCP entity is associated either to the control plane or the user plane depending on which radio bearer it is carrying data for.

Figure 4.2.2.1 represents the functional view of the PDCP entity for the PDCP sublayer; it should not restrict implementation. The figure is based on the radio interface protocol architecture defined in [2].

For split bearers, routing or duplication is performed in the transmitting PDCP entity.

Figure 4.2.2-1: PDCP layer, functional view

4.3	Services

4.3.1	Services provided to upper layers

The PDCP layer provides its services to the RRC or SDAP layers. The following services are provided by PDCP to upper layers:

-	transfer of user plane data;

-	transfer of control plane data;

-	header compression;

-	ciphering;

-	integrity protection.

The maximum supported size of a PDCP SDU is [8188] bytes. The maximum supported size of a PDCP Control PDU is [8188] bytes.

Editor’s Note: The exact maximum sizes of PDCP SDU and PDCP Control PDU are FFS.

4.3.2	Services expected from lower layers

A PDCP entity expects the following services from lower layers per RLC entity (for a detailed description see [5]):

-	acknowledged data transfer service, including indication of successful delivery of PDCP PDUs;

-	unacknowledged data transfer service.

4.4	Functions

The PDCP layer supports the following functions:

-	transfer of data (user plane or control plane);

-	maintenance of PDCP SNs;

-	header compression and decompression using the ROHC protocol;

-	ciphering and deciphering of user plane data and control plane data;

-	integrity protection and integrity verification of control plane data;

-	timer based SDU discard;

-	for split bearers, routing or duplication;

-	reordering and in-order delivery;

-	duplicate discarding;

5	Procedures

5.1	PDCP entity handling

5.1.1	PDCP entity establishment

When upper layers request a PDCP entity establishment for a radio bearer, the UE shall:

-	establish a PDCP entity for the radio bearer;

-	set the state variables of the PDCP entity to initial values.

5.1.2	PDCP entity re-establishment

When upper layers request a PDCP entity re-establishment, the UE shall additionally perform once the procedures described in this section. After performing the procedures in this section, the UE shall follow the procedures in subclause 5.2.

When upper layers request a PDCP entity re-establishment, the transmitting PDCP entity shall:

-	for UM DRBs, reset the header compression protocol for uplink and start with an IR state in U-mode [8] [9] if drb-ContinueROHC is not configured [3];

-	for UM DRBs and SRBs, set PT(S) and TX_HFN to 0;

-	for SRBs, discard all stored PDCP SDUs and PDCP PDUs;

-	apply the ciphering algorithm and key provided by upper layers during the PDCP entity re-establishment procedure;

-	for SRBs, apply the integrity protection algorithm and key provided by upper layers during the PDCP entity re-establishment procedure;

-	for UM DRBs, for each PDCP SDU already associated with a PDCP SN but for which a corresponding PDU has not previously been submitted to lower layers:

-	consider the PDCP SDUs as received from upper layer;

-	perform transmission of the PDCP SDUs in ascending order of the COUNT value associated to the PDCP SDU prior to the PDCP re-establishment without restarting the discardTimer.

-	for AM DRBs, from the first PDCP SDU for which the successful delivery of the corresponding PDCP Data PDU has not been confirmed by lower layers, perform retransmission or transmission of all the PDCP SDUs already associated with PDCP SNs in ascending order of the COUNT values associated to the PDCP SDU prior to the PDCP entity re-establishment as specified below:

-	perform header compression of the PDCP SDU (if configured) as specified in the subclause 5.7.4;

-	perform ciphering of the PDCP SDU using the COUNT value associated with this PDCP SDU as specified in the subclause 5.8;

-	submit the resulting PDCP Data PDU to lower layer.

When upper layers request a PDCP entity re-establishment, the receiving PDCP entity shall:

-	for SRBs, discard all stored PDCP SDUs and PDCP PDUs

-	for UM DRBs, reset the header compression protocol for downlink and start with NC state in U-mode (if configured) [8] [9] if drb-ContinueROHC is not configured [3];

-	for UM DRBs and SRBs, set PR(H) and RX_HFN to 0;

-	apply the ciphering algorithm and key provided by upper layers during the PDCP entity re-establishment procedure;

-	for SRBs, apply the integrity protection algorithm and key provided by upper layers during the PDCP entity re-establishment procedure.

5.1.3	PDCP entity release

When upper layers request a PDCP entity release for a radio bearer, the UE shall:

-	for UM DRBs and AM DRBs, deliver the PDCP SDUs stored in the receiving PDCP entity to upper layers in ascending order of associated COUNT values;

-	release the PDCP entity for the radio bearer.

Editor’s Note: Delivery of stored PDCP SDUs at PDCP entity release is FFS.

5.2	Data transfer

5.2.1	Transmit operation

At reception of a PDCP SDU from upper layers, the transmitting PDCP entity shall:

-	start the discardTimer associated with this PDCP SDU (if configured);

For a PDCP SDU received from upper layers, the transmitting PDCP entity shall:

-	associate the COUNT value corresponding to PT(S) to this PDCP SDU;

NOTE:	Associating more than half of the PDCP SN space of contiguous PDCP SDUs with PDCP SNs, when e.g., the PDCP SDUs are discarded or transmitted without acknowledgement, may cause HFN desynchronization problem. How to prevent HFN desynchronization problem is left up to UE implementation.

-	perform header compression of the PDCP SDU (if configured) as specified in the subclause 5.5.4;

-	perform integrity protection (if applicable), and ciphering (if applicable) using the PT(S) as specified in the subclause 5.9 and 5.8, respectively;

-	set the PDCP SN of the PDCP Data PDU to PT(S) modulo 2[pdcp-SN-Size];

-	increment PT(S) by one;

-	submit the resulting PDCP Data PDU to lower layer.

5.2.2	Receive operation

Editor’s Note: To be filled later.

5.3	SDU discard

When the discardTimer expires for a PDCP SDU, or the successful delivery of a PDCP SDU is confirmed by PDCP status report, the transmitting PDCP entity shall discard the PDCP SDU along with the corresponding PDCP PDU. If the corresponding PDCP PDU has already been submitted to lower layers, the discard is indicated to lower layers.

NOTE:	Discarding a PDCP SDU already associated with a PDCP SN causes a SN gap in the transmitted PDCP PDUs, which increases PDCP reordering delay in the receiving PDCP entity. It is up to UE implementation how to minimize SN gap after SDU discard.

5.4	Status reporting

5.4.1	Transmit operation

For AM DRBs configured by upper layers to send a PDCP status report in the uplink (statusReportRequired [3]), the receiving PDCP entity shall trigger a PDCP status report when:

-	upper layer requests a PDCP entity re-establishment;

-	upper layer requests a PDCP data recovery;

-	a PDCP Data PDU with P bit set to 1 is received;

Editor’s Note: The triggers for PDCP status report are FFS.

If a PDCP status report is triggered, the receiving PDCP entity shall:

-	compile a PDCP status report as indicated below by:

-	setting the FMC field to the COUNT value of the first missing PDCP SDU;

-	if there is at least one out-of-sequence PDCP SDU stored, allocating a Bitmap field of length in bits equal to the number of COUNTs from and not including the first missing PDCP SDU up to and including the last out-of-sequence PDCP SDUs, rounded up to the next multiple of 8, or up to and including a PDCP SDU for which the resulting PDCP Control PDU size is equal to [8188] bytes, whichever comes first;

-	setting in the bitmap field as ‘0’ for all PDCP SDUs that have not been received, and optionally PDCP SDUs for which decompression have failed;

-	setting in the bitmap field as ‘1’ for all PDCP SDUs that have been received;

-	submit the PDCP status report to lower layers as the first PDCP PDU for transmission.

5.4.2	Receive operation

For AM DRBs, when a PDCP status report is received in the downlink, the transmitting PDCP entity shall:

-	consider for each PDCP SDU, if any, with the bit in the bitmap set to '1', or with the associated COUNT value less than the value of FMC field as successfully delivered, and discard the PDCP SDU as specified in the subclause 5.3.

5.5	Data recovery

For AM DRBs, when upper layers request a PDCP data recovery for a radio bearer, the transmitting PDCP entity shall:

-	if the radio bearer is configured by upper layers to send a PDCP status report in the uplink (statusReportRequired [3]), transmit a PDCP status report as described in subclause 5.4.1;

-	perform retransmission of all the PDCP Data PDUs previously submitted to re-established AM RLC entity in ascending order of the associated COUNT values from the first PDCP Data PDU for which the successful delivery has not been confirmed by lower layers.

After performing the above procedures, the transmitting PDCP entity shall follow the procedures in subclause 5.2.1.

5.6	Data volume calculation

For the purpose of MAC buffer status reporting, the transmitting PDCP entity shall consider PDCP Control PDUs, as well as the following as PDCP data volume:

For SDUs for which no PDU has been submitted to lower layers:

-	the SDU itself, if the SDU has not yet been processed by PDCP, or

-	the PDU if the SDU has been processed by PDCP.

Editor’s Note: Other SDUs and PDUs that are considered as PDCP data volume are FFS.

5.7	Header compression and decompression

5.7.1	Supported header compression protocols and profiles

The header compression protocol is based on the Robust Header Compression (ROHC) framework [7]. There are multiple header compression algorithms, called profiles, defined for the ROHC framework. Each profile is specific to the particular network layer, transport layer or upper layer protocol combination e.g. TCP/IP and RTP/UDP/IP.

The detailed definition of the ROHC channel is specified as part of the ROHC framework [7]. This includes how to multiplex different flows (header compressed or not) over the ROHC channel, as well as how to associate a specific IP flow with a specific context state during initialization of the compression algorithm for that flow.

The implementation of the functionality of the ROHC framework and of the functionality of the supported header compression profiles is not covered in this specification.

In this version of the specification the support of the following profiles is described:

Table 5.7.1-1: Supported header compression protocols and profiles

| Profile Identifier | Usage | Reference |
| --- | --- | --- |
| 0x0000 | No compression | RFC 4995 |
| 0x0001 | RTP/UDP/IP | RFC 3095, RFC 4815 |
| 0x0002 | UDP/IP | RFC 3095, RFC 4815 |
| 0x0003 | ESP/IP | RFC 3095, RFC 4815 |
| 0x0004 | IP | RFC 3843, RFC 4815 |
| 0x0006 | TCP/IP | RFC 4996 |
| 0x0101 | RTP/UDP/IP | RFC 5225 |
| 0x0102 | UDP/IP | RFC 5225 |
| 0x0103 | ESP/IP | RFC 5225 |
| 0x0104 | IP | RFC 5225 |

5.7.2	Configuration of header compression

PDCP entities associated with DRBs can be configured by upper layers [3] to use header compression.

5.7.3	Protocol parameters

RFC 4995 has configuration parameters that are mandatory and that must be configured by upper layers between compressor and decompressor peers [7]; these parameters define the ROHC channel. The ROHC channel is a unidirectional channel, i.e. there is one channel for the downlink, and one for the uplink. There is thus one set of parameters for each channel, and the same values shall be used for both channels belonging to the same PDCP entity.

Editor’s Note: Support for UL-only ROHC is FFS.

These parameters are categorized in two different groups, as defined below:

-	M:	Mandatory and configured by upper layers.

-	N/A: Not used in this specification.

The usage and definition of the parameters shall be as specified below.

-	MAX_CID (M): This is the maximum CID value that can be used. One CID value shall always be reserved for uncompressed flows. The parameter MAX_CID is configured by upper layers (maxCID [3]).

-	LARGE_CIDS: This value is not configured by upper layers, but rather it is inferred from the configured value of MAX_CID according to the following rule:

If MAX_CID > 15 then LARGE_CIDS = TRUE else LARGE_CIDS = FALSE.

-	PROFILES (M): Profiles are used to define which profiles are allowed to be used by the UE. The list of supported profiles is described in section 5.7.1. The parameter PROFILES is configured by upper layers (profiles for uplink and downlink [3]).

-	FEEDBACK_FOR (N/A): This is a reference to the channel in the opposite direction between two compression endpoints and indicates to what channel any feedback sent refers to. Feedback received on one ROHC channel for this PDCP entity shall always refer to the ROHC channel in the opposite direction for this same PDCP entity.

-	MRRU (N/A): ROHC segmentation is not used.

5.7.4	Header compression

The header compression protocol generates two types of output packets:

-	compressed packets, each associated with one PDCP SDU

-	standalone packets not associated with a PDCP SDU, i.e. interspersed ROHC feedback

A compressed packet is associated with the same PDCP SN and COUNT value as the related PDCP SDU.

Interspersed ROHC feedback are not associated with a PDCP SDU. They are not associated with a PDCP SN and are not ciphered.

NOTE:	If the MAX_CID number of ROHC contexts are already established for the compressed flows and a new IP flow does not match any established ROHC context, the compressor should associate the new IP flow with one of the ROHC CIDs allocated for the existing compressed flows or send PDCP SDUs belonging to the IP flow as uncompressed packet.

5.7.5	Header decompression

If header compression is configured by upper layers for PDCP entities associated with user plane data, the PDCP PDUs are decompressed by the header compression protocol after performing deciphering as explained in the subclause 5.8.

5.7.6	PDCP Control PDU for interspersed ROHC feedback

5.7.6.1	Transmit Operation

When an interspersed ROHC feedback is generated by the header compression protocol, the transmitting PDCP entity shall:

-	submit to lower layers the corresponding PDCP Control PDU as specified in subclause 6.2.5 i.e. without associating a PDCP SN, nor performing ciphering.

5.7.6.2	Receive Operation

At reception of a PDCP Control PDU for interspersed ROHC feedback from lower layers, the receiving PDCP entity shall:

-	deliver the corresponding interspersed ROHC feedback to the header compression protocol without performing deciphering.

5.8	Ciphering and deciphering

The ciphering function includes both ciphering and deciphering and is performed in PDCP. For the control plane, the data unit that is ciphered is the data part of the PDCP PDU (see subclause 6.3.3) and the MAC-I (see subclause 6.3.4). For the user plane, the data unit that is ciphered is the data part of the PDCP PDU (see subclause 6.3.3); ciphering is not applicable to PDCP Control PDUs.

The ciphering algorithm and key to be used by the PDCP entity are configured by upper layers [3] and the ciphering method shall be applied as specified in [6].

The ciphering function is activated by upper layers [3]. When security is activated, the ciphering function shall be applied to all PDCP PDUs indicated by upper layers [3] for the downlink and the uplink, respectively.

For downlink and uplink ciphering and deciphering, the parameters that are required by PDCP for ciphering are defined in [6] and are input to the ciphering algorithm. The required inputs to the ciphering function include the COUNT value, and DIRECTION (direction of the transmission: set as specified in [6]).The parameters required by PDCP which are provided by upper layers [3] are listed below:

-	BEARER (defined as the radio bearer identifier in [6]. It will use the value RB identity –1 as in [3]);

-	KEY (the ciphering keys for the control plane and for the user plane are [KRRCenc] and [KUPenc], respectively).

Editor’s Note: Exact name of ciphering keys are FFS.

5.9	Integrity protection and verification

The integrity protection function includes both integrity protection and integrity verification and is performed in PDCP for PDCP entities associated with SRBs. The data unit that is integrity protected is the PDU header and the data part of the PDU before ciphering.

The integrity protection algorithm and key to be used by the PDCP entity are configured by upper layers [3] and the integrity protection method shall be applied as specified in [6].

The integrity protection function is activated by upper layers [3]. When security is activated, the integrity protection function shall be applied to all PDUs including and subsequent to the PDU indicated by upper layers [3] for the downlink and the uplink, respectively.

NOTE:	As the RRC message which activates the integrity protection function is itself integrity protected with the configuration included in this RRC message, this message needs first be decoded by RRC before the integrity protection verification could be performed for the PDU in which the message was received.

For downlink and uplink integrity protection and verification, the parameters that are required by PDCP for integrity protection are defined in [6] and are input to the integrity protection algorithm. The required inputs to the integrity protection function include the COUNT value, and DIRECTION (direction of the transmission: set as specified in [6]). The parameters required by PDCP which are provided by upper layers [3] are listed below:

-	BEARER (defined as the radio bearer identifier in [6]. It will use the value RB identity –1 as in [3]);

-	KEY ([KRRCint]).

Editor’s Note: Exact name of integrity key is FFS.

At transmission, the UE computes the value of the MAC-I field and at reception it verifies the integrity of the PDCP PDU by calculating the X-MAC based on the input parameters as specified above. If the calculated X-MAC corresponds to the received MAC-I, integrity protection is verified successfully.

5.10	Handling of unknown, unforeseen, and erroneous protocol data

When a PDCP PDU that contains reserved or invalid values is received, the receiving PDCP entity shall:

-	discard the received PDU.

6	Protocol data units, formats, and parameters

6.1	Protocol data units

6.1.1	Data PDU

The PDCP Data PDU is used to convey one or more of followings:

-	a PDCP SDU SN;

-	user plane data;

-	control plane data;

-	a MAC-I.

6.1.2	Control PDU

The PDCP Control PDU is used to convey one of followings:

-	a PDCP status report;

-	an interspersed ROHC feedback.

6.2	Formats

Editor’s Note: Exact PDU formats are not decided yet. The length of PDCP SN, use of D/C field, and use of P field need to be discussed.

6.2.1	General

A PDCP PDU is a bit string that is byte aligned (i.e. multiple of 8 bits) in length. In the figures in subclause 6.2, bit strings are represented by tables in which the most significant bit is the leftmost bit of the first line of the table, the least significant bit is the rightmost bit on the last line of the table, and more generally the bit string is to be read from left to right and then in the reading order of the lines. The bit order of each parameter field within a PDCP PDU is represented with the first and most significant bit in the leftmost bit and the last and least significant bit in the rightmost bit.

PDCP SDUs are bit strings that are byte aligned (i.e. multiple of 8 bits) in length. A compressed or uncompressed SDU is included into a PDCP PDU from the first bit onward.

6.2.2	Data PDU

6.2.2.1	Data PDU with 12 bits PDCP SN

Figure 6.2.2.1-1 shows the format of the PDCP Data PDU with 12 bits PDCP SN. This format is applicable for UM DRBs, AM DRBs, and SRBs. The MAC-I field is included only for SRBs.

Figure 6.2.2.1-1: PDCP Data PDU format with 12 bits PDCP SN

6.2.2.2	Data PDU with 18 bits PDCP SN

Editor’s Note: 18 bits PDCP SN length is FFS.

Figure 6.2.2.2-1 shows the format of the PDCP Data PDU with 18 bits PDCP SN. This format is applicable for UM DRBs and AM DRBs. The MAC-I field is included only for SRBs.

Figure 6.2.2.2-1: PDCP Data PDU format with 18 bits PDCP SN

6.2.3	Control PDU

6.2.3.1	Control PDU for PDCP status report

Figure 6.2.3.1-1 shows the format of the PDCP Control PDU carrying one PDCP status report. This format is applicable for AM DRBs.

Figure 6.2.3.1-1: PDCP Control PDU format for PDCP status report

6.2.3.2	Control PDU for interspersed ROHC feedback

Figure 6.2.3.2-1 shows the format of the PDCP Control PDU carrying one interspersed ROHC feedback. This format is applicable for UM DRBs and AM DRBs.

Figure 6.2.3.2-1: PDCP Control PDU format for interspersed ROHC feedback

6.3	Parameters

6.3.1	General

If not otherwise mentioned in the definition of each field then the bits in the parameters shall be interpreted as follows: the left most bit string is the first and most significant and the right most bit is the last and least significant bit.

Unless otherwise mentioned, integers are encoded in standard binary encoding for unsigned integers. In all cases the bits appear ordered from MSB to LSB when read in the PDU.

6.3.2	PDCP SN

Length: 12, or 18 bits as indicated in table 6.3.2.1. The length of the PDCP SN is configured by upper layers (pdcp-SN-Size [3]).

Table 6.3.2-1: PDCP SN length

| Length | Description |
| --- | --- |
| 12 | UM DRBs, AM DRBs, and SRBs |
| 18 | UM DRBs, and AM DRBs |

Editor’s Note: 18 bits PDCP SN length and other values of PDCP SN length are FFS.

6.3.3	Data

Length: Variable

This field includes one of the followings:

-	Uncompressed PDCP SDU (user plane data, or control plane data);

-	Compressed PDCP SDU (user plane data only).

6.3.4	MAC-I

Length: 32 bits

This field carries a message authentication code calculated as specified in subclause 5.9.

For control plane data that are not integrity protected, the MAC-I field is still present and should be padded with padding bits set to 0.

6.3.5	COUNT

Length: 32 bits

The COUNT value is composed of a HFN and the PDCP SN. The size of the HFN part in bits is equal to 32 minus the length of the PDCP SN.

Figure 6.3.5-1: Format of COUNT

NOTE:	When performing comparison of values related to COUNT, the UE takes into account that COUNT is a 32-bit value, which may wrap around (e.g., COUNT value of 232 - 1 is less than COUNT value of 0).

6.3.6	R

Length: 1 bit

Reserved. In this version of the specification reserved bits shall be set to 0. Reserved bits shall be ignored by the receiver.

6.3.7	D/C

Length: 1 bit

This field indicates whether the corresponding PDCP PDU is a PDCP Data PDU or a PDCP Control PDU.

Table 6.3.7-1: D/C field

| Bit | Description |
| --- | --- |
| 0 | Control PDU |
| 1 | Data PDU |

6.3.8	PDU type

Length: 3 bits

This field indicates the type of control information included in the corresponding PDCP Control PDU.

Table 6.3.8-1: PDU type

| Bit | Description |
| --- | --- |
| 000 | PDCP status report |
| 001 | Interspersed ROHC feedback |
| 010-111 | Reserved |

6.3.9	FMC

Length: 32 bits

First Missing COUNT. This field indicates the COUNT value of the first missing PDCP SDU.

6.3.10	Bitmap

Length: Variable. The length of the bitmap field can be 0.

This field indicates which SDUs are missing and which SDUs are correctly received in the receiving PDCP entity. The bit position of Nth bit in the Bitmap is N, i.e., the bit position of the first bit in the Bitmap is 1.

Table 6.3.10-1 Bitmap

| Bit | Description |
| --- | --- |
| 0 | PDCP SDU with COUNT = (FMC + bit position) modulo 232 is missing. |
| 1 | PDCP SDU with COUNT = (FMC + bit position) modulo 232 is correctly received. |

6.3.11	Interspersed ROHC feedback

Length: Variable

This field contains one ROHC packet with only feedback, i.e. a ROHC packet that is not associated with a PDCP SDU as defined in subclause 5.7.4.

6.3.12	P

Length: 1 bit

Polling indication. This field indicates whether the UE is requested to send a PDCP status report.

Editor’s Note: Whether the P field is applicable to PDCP Data PDU is FFS.

Table 6.3.12-1: P field

| Bit | Description |
| --- | --- |
| 0 | Status report is not requested |
| 1 | Status report is requested |

7	State variables, constants, and timers

7.1	State variables

This sub clause describes the state variables used in PDCP entities in order to specify the PDCP protocol. The state variables defined in this subclause are normative.

All state variables are non-negative integers.

All state variables can take values from 0 to [232 – 1]. All arithmetic operations contained in the present document on state variables are affected by the PDCP modulus (i.e. final value = [value from arithmetic operation] modulo 232).

PDCP Data PDUs are numbered integer sequence numbers (SN) cycling through the field: 0 to [2[pdcp-SN-Size] – 1].

When performing arithmetic comparisons of state variables or COUNT values, a modulus base shall be used.

PR(H) – Window_Size shall be assumed as the modulus base at the receiving PDCP entity. This modulus base is subtracted from all the values involved, and then an absolute comparison is performed (e.g. (PR(H) – Window_Size) <= COUNT < PR(H) is evaluated as [(PR(H) – Window_Size) – (PR(H) – Window_Size)] modulo 232 <= [COUNT – (PR(H) – Window_Size)] modulo 232 < [PR(H) – (PR(H) – Window_Size)] modulo 232).

Editor’s Note: The description of modulo operation could be removed if COUNT does not wrap-around.

The transmitting PDCP entity shall maintain the following state variables:

a)	TX_HFN

This state variable indicates the HFN value for the generation of the COUNT value used for the transmitted PDCP PDUs. The initial value is 0.

Editor’s Note: The state variable TX_HFN may not need to be defined.

b)	PT(S)

This state variable indicates the COUNT value of the next PDCP SDU to be transmitted. The initial value is 0.

The receiving PDCP entity shall maintain the following state variables:

a)	RX_HFN

This state variable indicates the HFN value for the generation of the COUNT value used for the received PDCP PDUs. The initial value is 0.

Editor’s Note: The state variable RX_HFN may not need to be defined.

b)	PR(H)

This state variable indicates the COUNT value of the next PDCP SDU expected to be received. The initial value is 0.

c)	PR(R)

This state variable indicates the COUNT value of the last PDCP SDU delivered to the upper layers. The initial value is 232 – 1.

d)	PR(X)

This state variable indicates the COUNT value following the COUNT value associated with the PDCP PDU which triggered t-Reordering.

7.2	Constants

a) Window_Size

This constant indicates the size of the reordering window. The value equals to 2[pdcp-SN-Size] – 1.

7.3	Timers

The transmitting PDCP entity shall maintain the following timers:

a) discardTimer

This timer is configured only for DRBs. The duration of the timer is configured by upper layers [3]. In the transmitter, a new timer is started upon reception of an SDU from upper layer.

The receiving PDCP entity shall maintain the following timers:

b) t-Reordering

The duration of the timer is configured by upper layers [3]. This timer is used to detect loss of PDCP PDUs as specified in the subclause 5.2.2. If t-Reordering is running, t-Reordering shall not be started additionally, i.e. only one t-Reordering per receiving PDCP entity is running at a given time.

Annex A (informative):
Change history

| Change history |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Date | Meeting | TDoc | CR | Rev | Cat | Subject/Comment | Version |
| 2017.03 | RAN2#97bis | R2-1703512 | - | - | - | First version. | x.y.z |
| 2017.04 | RAN2#97bis | R2-1703916 | - | - | - | Change section name “Retransmission” to “Data recovery” | 0.0.1 |
| 2017.05 | RAN2#98 | R2-1704076 | - | - | - | Initial draft TS capturing outcome of e-mail discussion [97bis#24] | 0.0.5 |

Annex B (informative; to be removed later): 
Agreements related to PDCP specification

Editor’s Note: This section collects the agreements that may have impact on PDCP specification. This section will be removed when the PDCP specification is officially released.

NR AH (Jan. 2017)

- PDCP and RLC layers have separate PDCP SN and RLC SN.

- Packet duplication is supported for user plane and control plane in NR-PDCP (This agreement does not preclude discussion of other mechanisms to improve mobility robustness)

- The PDCP function in the transmitter supports packet duplication and the PDCP function in the receiver supports duplicate packet removal.

RAN2#97 (Athens, Feb. 2017)

- Split bearers support RLC UM mode besides RLC AM mode.

- For DL and UL, duplication solution for CA case uses PDCP duplication to more than 1 logical channel so that the duplicated PDCP PDUs are sent over different carriers.

RAN2#97bis (Spokane, Mar. 2017)

- Duplicate detection and discard functionalities for SRBs should be introduced in LTE PDCP to support duplication via split SRB in LTE-NR tight interworking scenarios where LTE is the MN.

- RRC configures PDCP for duplication and the radio protocols of the UE with separate RLC entities and logical channels to handle duplicates (referred to as “legs”)

- only one additional leg is configured for PDCP duplicates.

- In NR, SRBs (except SRB0) should be always mapped on RLC AM.

- In NR, DRB can be mapped on either RLC AM or RLC UM.

- Confirmed that RoHC will remain in PDCP.

- We will not consider concatenation in PDCP or SDAP further in Rel-15.

- We will not look at TCP optimisations further in Rel-15.

- 2  handling of handover for AM mode:

  - 1: LTE- like handover

  - 2: No Key change, Data Recovery, RLC re-establishment

- 2 handling of handover for UM mode:

  - 1: LTE- like handover

  - 2: No Key change, RLC re-established

- 2 handling of handover for SRB:

  - 1: LTE- like handover

  - 2: No Key change, RLC re-established

- NR PDCP aims at defining a single procedure for reception for a AM and UM, as well as DRB and SRB (will be concluded in stage 3 discussions)

- NR PDCP procedures are based on COUNT instead of SN, which is determined in the beginning of the procedure. All NR PDCP state variables are based on COUNT.

- Adopt modulo operation for arithmetic operation.

- Some PDCP recovery mechanism based of PDCP status report is supported at least for the handover case and DC.  FFS if there are other cases in which this may be performed.

- PDCP SN length should be up to at least up 18 bits (for data bearer). FFS if there are use cases in which larger value is needed and which value for AM/UM.

- PDCP SN length 12 bits is supported for RLC UM and AM and for both DRB(s) and SRB(s)

- LTE PDCP PDU format for DRBs will be the reused

- NR should support jumbo frame (9KB)

- FFS NR UE can support super jumbo frame (65KB) and is optional.

- PDCP reordering should always enabled for SRBs

- PDCP duplicate discarding should be done at least when duplication is enabled for SRBs/DRBs(s).

- A unified re-ordering schemes is used for DRB(s)/SRB(s) and UM and AM, with LTE as baseline.

- It is desirable to disable PDCP reordering.  FFS how to signal it

- Use First Missing COUNT (FMC) instead of FMS in the PDCP Status Report.

- PDCP SDU is discarded upon the expiry of PDCP discard timer.

- PDCP SDU is discarded when successful delivery is confirmed by PDCP status report.