3GPP TSG-RAN WG2 #98	Tdoc R2-1704173

Hangzhou, P.R. of China, 15th – 19th May 2017	(update of R2-1702557)

Agenda Item:	9.7.2

Source:	Ericsson

Title:	QoS for LTE when connected to 5G-CN

Document for:	Discussion, Decision

# Introduction

During RAN2-NR#1 and RAN2#97 the new user plane (UP) QoS framework for New RAN (i.e., LTE and NR connected to NGC) was discussed and the following agreements were achieved.

Agreements:

1   	A new user plane AS protocol layer (e.g. PDAP) above PDCP should be introduced to accommodate all the functions introduced in AS for the new QoS framework, including:

- QoS flow->DRB routing;

- QoS-flow-id marking in DL/UL packets;

2.	The new protocol layer is applicable for all cases connecting to the 5G-CN.

3.	Single protocol entity is configured for each individual PDU session.

4.	RAN2 to confirm that the timing of non-default DRB establishment (RAN to UE) for QoS Flow configured during PDU Session Establishment could be done NOT at the same time as PDU Session Establishment (up to eNB implementation).

5.	Working assumption from RAN2#96 is confirmed, i.e., first UL packet that doesn't have a mapping to a DRB, is mapped to a default DRB.

6.	“Lossless HO”, that is, lossless, in sequence without duplication to upper layers, should be supported in specification for intra-NR.

- FFS whether we support QoS flow remapping at handover and, if supported, whether the handover is lossless for this case.

Considering these agreements, in this contribution we further discuss how the new QoS framework can be applied to LTE when connected to 5G-CN.

# QoS framework in EPS and 5GS

LTE nodes connected to 5G-CN (LTE-5G-CN) shall employ the new QoS framework, which is currently being discussed for New RAN and is different from the EPS bearer framework used in LTE-EPC. In the following we provide a brief description of the EPS bearer framework and the current understanding of the new QoS framework to highlight their differences.

## EPS bearer framework in LTE-EPC

In LTE-EPC to support multiple QoS requirements, different bearers are set up within EPS, each being associated with a QoS [1]. Broadly bearers can be classified as guaranteed bit-rate (GBR) or non-GBR.  Service data flow (SDF) packets mapped to the same EPS bearer receive the same bearer-level packet forwarding treatment. Providing different bearer-level QoS thus requires that a separate EPS bearer is established for each QoS flow, and user IP packets must be filtered into the different EPS bearers. In the access network, there is a 1:1 mapping between EPS and radio bearers (DRBs) and it is the eNB responsibility to ensure that the necessary QoS for a bearer over the radio interface is met. Each bearer has an associated Class Identifier (QCI) and an Allocation and Retention Priority (ARP). Each QCI is characterized by priority, packet delay budget and acceptable packet loss rate. The QCI label for a bearer determines the way it is handled in the eNB. A limited number of QCIs have been standardized so that different vendors can have the same understanding of the service characteristics and provide the corresponding treatment over the air interface, including RLC mode operation, queue management, conditioning, and policing strategy. The ARP of a bearer is used for call admission and congestion control, e.g., to indicate whether a bearer can be pre-empted in congestion situations.

## Current understanding of QoS framework in LTE-5G-CN

In LTE-5G-CN, the QoS-flow concept replaces the EPS bearer framework [2]. A QoS-flow requires a given end-to-end QoS level. QoS-flows can be either GBR or non-GBR. Each QoS-flow is assigned a QoS-flow ID (QFI) which is used to identify the flow in the 5G system. The number of QoS-flows per UE might be larger than the number of EPS bearers per UE, thus several QoS-flows with similar QoS requirements may be mapped over the same DRB over the Uu interface. Therefore, there is a two-level mapping between SDF-flows to QoS-flows to DRBs. The RAN and the UE are responsible for the QoS-flows to DRBs mapping and in case RAN decides that there is a flexible (e.g., other than 1:1) mapping between QoS-flows and DRBs, this mapping shall be transparent to the upper layers and shall have no impact on the NG3 marking. Each QoS-flow has an associated 5G QoS indicator (5QI) and ARP parameter. A limited number of 5QI indicators is expected to be standardized (the role of 5QI in 5GS framework is expected to be similar as the role of QCI in the EPS bearer framework). It is up to the RAN to define the QoS-flows to DRBs mapping based on the 5QI. There are two types of QoS-profiles, namely “A-type QoS profiles” which have predefined QoS characteristics and “B-type QoS profiles” which have QoS characteristics dynamically signalled to the RAN over the NG2 interface. The mapping between QoS-flows and DRBs is performed by a new AS layer protocol (e.g., PDAP) on top of PDCP.

An LTE node connected to both EPC and 5G-CN shall support both the EPS bearer framework and the new QoS framework. UEs that are connected to EPC shall employ the EPS bearer framework, while UEs connected to 5G-CN shall employ the new QoS framework. The impact of supporting the new QoS framework on LTE-RRC and LTE-PDCP protocols should be minimized. On the other hand, we expect that the new QoS framework will not impact the LTE lower-layer protocols (i.e., RLC, MAC and PHY).

Proposal 1	An LTE node connected to both EPC and 5G-CN shall support both the EPS bearer framework and the new QoS framework. The impact of supporting the new QoS framework on LTE-RRC and LTE-PDCP protocols should be minimized. No impact is expected on the LTE lower-layer protocols.

# 5G QoS characteristics

SA2 has decided that the 5QI should specify the following 5G QoS characteristics [2], which describe the packet forwarding treatment that a QoS-flow shall receive between the UE and the UPF:

- Resource Type (GBR or Non-GBR);

- Priority level;

- Packet delay budget;

- Packet error rate.

The 5G QoS characteristics should be understood as guidelines for setting node-specific parameters for each QoS-flow, e.g., for deciding the scheduling policy in the RAN. These QoS characteristics are the same that are currently specified by the QCI in the EPS bearer framework. Therefore, QoS-flows will have the same QoS characteristics as EPS bearers. On one hand, this is beneficial because it allows to (at least partly) reuse current traffic engineering and QoS enforcement policies. On the other hand, in the current EPS framework, observability of QoS target fulfilment is difficult. In addition, we believe that some parameters are missing in the EPS bearer framework to help the RAN in taking optimal scheduling decisions. With optimal scheduling decisions, we intend that the RAN should provide each QoS-flow with the required service level while maximizing the usage of the radio resource. Therefore, we believe that there is a need for more well-defined QoS characteristics definitions that map directly to service performance. The new QoS framework offers the possibility of introducing these new QoS characteristics.

Based on the above discussion, we submitted an SA2 contribution [3] in which we propose the introduction of new optional 5G QoS characteristics as part of the 5QI profile (in addition to the QoS parameters above which are already agreed in SA2):

- Preferred bit-rate for non-GBR flows

- Averaging window size

- Relative priority

- Periodicity

These parameters were described in detail in [4] and further discussed in a RAN2 e-mail-discussion, summarized in [5]. Additional 5G QoS characteristics will provide better observability of the QoS level, from which network operators will benefit, as it allows a better controllability of the SLA and a more consistent estimation of the performance of equipment from different vendors. It will also offer the possibility of performing a more optimal scheduling in the RAN without affecting the LTE-MAC operation, which will lead to an overall better QoE for the end-users and a more efficient radio resource usage.

When agreement on additional 5G QoS characteristics for NR is reached we think this is important to consider any additions made for NR should also apply for LTE connected to 5G-CN.

Observation 1	New 5G QoS characteristics offer better observability of the QoS levels and the possibility of performing a more optimal scheduling in the RAN, without affecting the LTE-MAC protocol.

Based on the above consideration, we suggest that RAN2 agrees that new QoS characteristics, beneficial for improved QoS observability and RAN performance should also be introduced for LTE.

Proposal 2	RAN2 agrees to that new 5G QoS characteristics should be introduced for LTE when connected to 5G-CN, to increase QoS level observability and RAN performance.

# Conclusion

In this contribution we discussed the new QoS framework in LTE connected to 5G-CN and we made the following observations and proposals.

Proposal 1	An LTE node connected to both EPC and 5G-CN shall support both the EPS bearer framework and the new QoS framework. The impact of supporting the new QoS framework on LTE-RRC and LTE-PDCP protocols should be minimized. No impact is expected on the LTE lower-layer protocols.

Observation 1	New 5G QoS characteristics offer better observability of the QoS levels and the possibility of performing a more optimal scheduling in the RAN, without affecting the LTE-MAC protocol.

Proposal 2	RAN2 agrees to that new 5G QoS characteristics should be introduced for LTE when connected to 5G-CN, to increase QoS level observability and RAN performance.

# References

TS 36.300, “Evolved Universal Terrestrial Radio Access (E-UTRA) and Evolved Universal Terrestrial Radio Access Network (E-UTRAN); Overall Description; Stage 2”, December 2016.

TS 23.501 v3.0, “System Architecture for the 5G System; Stage 2”, February 2017.

S2-171501, “QoS parameters and characteristics needed in 5G Systems”, February 2017.

R2-1702755, “RAN aspects of QoS parameters”, Spokane, USA, April, 2017.

R2-1704378, Email discussion report on additional QoS Parameters, 3GPP RAN WG2 #98, Hangzhou, P.R. of China, 15th – 19th May 2017