3GPP TSG-RAN WG2 Meeting #98                                                                      R2-1704265

Hangzhou, China, 15th - 19th May 2017

Source:	CATT

Title:	SDAP PDU format

Agenda Item:	10.3.4.2

Document for:	Discussion and Decision

# Introduction

In RAN2#97bis meeting, the following agreements regarding SDAP PDU were reached [1]:

Agreements on QoS layer:

-	New AS layer PDU is PDCP SDU

-	AS layer header is byte-aligned

-	DL packets over Uu are not marked with “Flow ID” at least for cases where UL AS reflective mapping and NAS reflective QoS is not configured for DRB.

-	AS layer header include the UL “Flow ID” depending on network configuration

In this contribution, we discuss the detailed formats of SDAP data PDU and QFI presence, and give our proposals accordingly.

# Discussion

In SDAP layer, UL and DL have different characteristics. First of all, DL PDU should carry RQI (Reflective QoS Indicator) field as UP activation indicator of reflective QoS function. However UL PDU doesn’t need it. Secondly, UL PDU should always carry QFI (QoS Flow Indicator) information either by explicit header field or by implicit configuration with 1-to-1 mapping. However DL PDU does not require QFI for cases where AS reflective mapping and NAS reflective QoS are not activated.

Proposal 1:  NR SDAP layer may have different PDU formats for UL and DL.

In the following subsections, detailed PDU formats design will be discussed.

## UL SDAP PDU

For UL SDAP, if there is 1-to-1 mapping between QoS flow and DRB, it does not need to be marked with QFI in principle.  Network can derive the QFI from DRB tunnel. But the issue of mapping reconfiguration caused by QoS flow addition and deletion arises, where PDUs without QFI field and PDUs with QFI field may be mixed in buffers and in the air. It brings extra complexity and confusion for SDAP layer processing.

In our view, there are two typical cases and their corresponding expected operations:

- First one is for some special services, e.g. VoIP, which requires guaranteed efficient transmission with minimum overhead. In this case, always ensuring 1-to-1 mapping is the easiest way.

- Second one is for ordinary services, e.g. best effort services, where overhead is not a big problem and flexibility and multiplexing is more necessary. For example, when PDU session establishes, there is only one QoS flow in a DRB. Later some other QoS flow(s) activation or arrival, mapping relationship between QoS flow(s) and that DRB can be changed from 1-to-1 to many-to-1 mapping. In this case, it is an easy way that network configures UE always carrying QFI from the beginning of DRB establishment, even during the 1-to-1 mapping interval. This can avoid re-organization of PDUs and other re-establishment operation triggered by mapping relationship reconfiguration.

For the above cases, the detailed configuration and decision is left to network implementation. But from the perspective of specification, explicit UL QFI presence indicator needs to be supported even in 1-to-1 mapping.

Proposal 2:  It is proposed to support explicit UL QFI presence indicator in RRC signaling and allow carrying UL QFI even in 1-to-1 mapping.

As a result, there are two UL PDU formats: one is without any header field and the other is carrying QFI field in header. Following figures give the detailed formats.

| Figure 1 UL PDU without header | Figure 2 UL PDU with QFI |
| --- | --- |

Note: The length of QFI is up to CT1 progress and agreements.

## DL SDAP PDU

The biggest difference between DL PDU and UL PDU is that DL PDU may carry the indicators of NAS reflective QoS or AS reflective mapping activation/deactivation. The indicator of NAS reflective QoS activation/deactivation is up to CN decision. An LS needs to ask SA2 and CT1 about whether there are some QoS flows, e.g. VoIP service, that are never marked with RQI (Reflective QoS Indicator in NAS layer). For these flows that never use RQI, RAN may configure 1-to-1 mapping between flow and DRB and omit any header similar to Figure 1.

Proposal 3:  An LS to SA2 and CT1 is needed to ask whether there are some QoS flows that are never marked with RQI.

For those flows that can use reflective function either in NAS layer or in AS layer, the potential activation/deactivation methods of reflective function are listed as in the following table:

|  | Activation/ deactivation via UP | Activation/ deactivation via CP |
| --- | --- | --- |
| NAS reflective QoS | RQI in N3 header and carried by SDAP header to UE | UE NAS signaling/N2 signaling notification per PDU session or per flow level |
| AS reflective Mapping | If supported, a specific indicator in SDAP header is needed | If supported, via RRC signaling per DRB or per UE level |

SA2 already agreed to support both UP and CP methods of NAS reflective QoS activation [2]. It is not yet discussed how to activate AS reflective mapping function, i.e. via UP or via CP or both. But from our view, since RQI is now present and there is at least 1 byte of SDAP header, another bit can be used as AS RQI without increasing any overhead. Furthermore, activation of AS reflective mapping via UP can have more flexibility than activation via CP.

Proposal 4: In addition to RQI field for NAS reflective QoS activation/deactivation, 1 bit ARQI for AS layer is also needed in SDAP DL PDU.

The fields of RQI and ARQI cannot be directly used to indicate whether QFI field follows up or not. When neither NAS reflective function nor AS reflective mapping are activated via UP, QFI is still needed in support of activation of NAS reflective function via CP. Hence a separate bit is needed to indicate whether QFI field follows or not.

Proposal 5: A separate bit is needed to indicate whether QFI field follows or not.

An example of DL SDAP PDU is shown in the following figure:

Figure 3 DL SDAP PDU

# Conclusion

Based on the analysis in section 2, the following is proposed:

Proposal 1:  NR SDAP layer may have different PDU formats for UL and DL.

Proposal 2:  It is proposed to support explicit UL QFI presence indicator in RRC signaling and allow carrying UL QFI even in 1-to-1 mapping.

Proposal 3:  An LS to SA2 and CT1 is needed to ask whether there are some QoS flows that are never marked with RQI.

Proposal 4: In addition to RQI field for NAS reflective QoS activation/deactivation, 1 bit ARQI for AS layer is also needed in SDAP DL PDU.

Proposal 5: A separate bit is needed to indicate whether QFI field follows or not.

# Reference

- RAN2#97bis, Chairman reports;

- TS 23.501, System Architecture for the 5G System, Stage 2, 2017-03;