3GPP TSG-RAN WG2 Meeting #97bis	R2-1704274

Spokane, USA, 3 - 7 April 2017	R2-1700813

Agenda item:	10.3.2.4

Source:	Nokia, Alcatel-Lucent Shanghai Bell

Title:	SN for RLC UM

WID/SID:	NR_newRAT - Release 15

Document for:	Discussion and Decision

# 1	Introduction

This contribution discusses the need for using a sequence number in RLC UM mode of operation.

# 2	Unacknowledged Mode

In order to support the 5G high bit rates and minimise the duplications of functions in the NR radio protocols, it was agreed to move concatenation to MAC and reordering to PDCP, leaving to RLC the main functions of duplication detection, error correction and segmentation/re-assembly [38.804].

Duplicates in RLC-UM operation can only happen when the MAC layer experiences ACK-NAK errors and a transport block is successfully received twice. Such duplicates can either be handled directly in MAC or at PDCP with the duplication detection function. As a result, there is no need for duplication detection in RLC.

Proposal 1: duplication detection is not performed at RLC for UM mode of operation.

As a result, the only purpose of an SN for RLC UM is to support segmentation. In other words, whenever segmentation does not occur, the RLC SN serves no purpose for RLC UM operation.

Observation 1: in RLC UM, whenever segmentation does not occur, the RLC SN serves no purpose.

The coverage of services with low bit rates such as VoIP is of paramount importance and until recently, RAN2 has still been discussing means to improve their support [RP-160664]:

-	Voice and Video over LTE (VoLTE/ViLTE) are key features for LTE to provide voice and video service. With the quick migration from 2G/3G to LTE and the increasing marketing requirement for high-quality voice/video services, Voice and video over LTE has being deployed and launched by operators over the world, which makes Voice and video over LTE capability extremely important for operators. In RAN#71, the study item “Study on enhancement of VoLTE” [1] was setup and concluded three aspects for VoLTE/video to be enhanced: 1)  the codec mode/rate selection and adaptation; 2) the signalling optimization; 3) VoLTE/video quality and coverage enhancements.

Additionally, RAN2 has agreed as baseline to support SO-based segmentation for segmentation and re-segmentation. With this, the same SN is assigned/used for every segment of a single RLC SDU and SO-field is used to point to the position of the original SDU the segment begins at.

Observation 1 allows us to reduce the overhead in a simple way: limit the use of a sequence number to the PDUs where segmentation occurs. On the receiver side, PDUs without sequence numbers can be passed to PDCP for re-ordering while PDUs with a sequence number need all their segments to be reassembled before doing so. The number of bits for the sequence number will determine how many RLC PDUs can be segmented on the fly. For instance, with 2 bits, RLC-UM can handle the segmentation of 4 RLC PDUs on the fly.

Proposal 3: in RLC UM, limit the use of a sequence number to the PDUs where segmentation occurs.

Typically for VoIP with header compression, no segmentation occurs and thus proposal 1 allows RLC UM to minimise overhead by not including any SN along with the individual speech frames. When a full IP header needs to be sent, segmentation occurs and an SN can then be used to handle the segments.

# 3	Conclusion

To minimise overhead in NR, two proposals were made:

Proposal 1: duplication detection is not performed at RLC for UM mode of operation.

Proposal 2: in RLC UM, limit the use of a sequence number to the PDUs where segmentation occurs.