3GPP TSG-RAN WG2 Meeting #98	R2-1704348

Hangzhou, China, 15 - 19 May 2017

Agenda item:	10.2.3

Source:	Nokia, Alcatel-Lucent Shanghai Bell

Title:	Additional messages to be supported using the SCG SRB

WID/SID:	NR_newRAT (Release 15)

Document for:	Discussion and Decision

# 1	Introduction

During the RAN2#97-bis meeting, the following messages were agreed to be sent on the SCG SRB using RRC signalling from secondary node.

Agreement

1:	The following RRC messages can be sent via the SRB in the SCG.

•	RRCConnectionReconfiguration and RRCConnectionReconfigurationComplete

•	MeasurementReport

In this contribution, we propose a few additional messages that are relevant to be sent on this interface.

# 2	Discussion

We show in [2], that the latency in using the SCG SRB is much lower compared to using the MCG path. When the SCG SRB is configured, the SgNB should be able to query the UE with its NR capability.

- UE CAPABILITY ENQUIRY

- UE CAPABILITY INFORMATION

In addition, for the split SCG and SCG bearers, the counter check messages could be sent with the SCG SRB.

- COUNTER CHECK

- COUNTER CHECK RESPONSE

Observation 1: The messages listed above do not need any coordination with the MN (i.e. are only a querying the status of the UE).

# 4	Conclusion

Based on the discussion above, we propose the following:

Proposal 1: Allow the following RRC messages via. the SRB in the SCG: UE CAPABILITY ENQUIRY, UE CAPABILITY INFORMATION, COUNTER CHECK and COUNTER CHECK RESPONSE

# References

- R2-1704349: Comparison of RRC reconfiguration latency for SCG SRB with MCG routed SRB, Nokia, Alcatel-Lucent Shanghai Bell

- R2-1702775: Specification impacts for direct RRC signalling from secondary node, Nokia, Alcatel-Lucent Shanghai Bell