3GPP TSG-RAN WG2 #98	Tdoc R2-1704363

Hangzhou, P.R. of China, 15th – 19th May 2017

Agenda Item:	10.3.1.3

Source:	Ericsson

Title:	MAC PDU Design

Document for:	Discussion, Decision

# Introduction

In RAN2#96-AH, the following agreements were made:

Agreements

1 MAC sub-headers are interleaved with MAC SDUs.

Agreements

1	As in LTE the UEs shall not send padding if there is data available and the remaining TB size is greater than X bytes (actual number can be discussed later when header sizes are known. In LTE X = 7 bytes)

2	MAC CEs are not placed in the middle of the MAC PDU but at the beginning or at the end (placement can be decided later)

3	Working assumption on no RLC concatenation taken at RAN2#96 is confirmed (i.e. concatenation of RLC PDUs is performed in MAC)

-	As in LTE the UEs shall not send padding if there is data available and the remaining TB size is greater than X bytes (actual number can be discussed later when header sizes are known. In LTE X = 7 bytes)

-	MAC CEs are not placed in the middle of the MAC PDU but at the beginning or at the end (placement can be decided later)

RAN2-97bis:

Agreements:

-	MAC SDUs, MAC subheaders, and MAC PDU are byte aligned (i.e., multiple of 8 bits).

-	MAC subheaders are placed immediately in front of the corresponding MAC SDUs, MAC CEs, or padding.  The possibility to parse the MAC PDU from the back is not precluded.

-	MAC CEs are grouped together

-	UL MAC CE(s) is placed after all the MAC SDUs.  For DL the placement will be deterministic (i.e. it should not be up to the network to decide).  FFS if we have the same behaviour for both or for DL the MAC CE is placed at the front

This contribution discusses about DL and UL MAC CE placement as well as padding and padding sub-header placement in MAC PDU.

# Discussion

## DL MAC CE placement

In the RAN2-97bis meeting, an agreement was made to place UL MAC CEs after the SDUs. It was left for FFS whether to place DL MAC CEs in the beginning of the PDU or at the end of the PDU. Placing the MAC CEs before the SDUs enables UE receiver to extract the MAC PDU information early, before the processing of MAC SDUs.

Placing the MAC CEs after the SDUs requires UE to process all SDUs before it can process important control elements. To speed up the processing when DL MAC CEs would be placed to the end, it is possible to introduce a mechanism which allows MAC CEs to be processed from the end to the beginning. In the solution where MAC CEs would be processed from the end towards the beginning, the sub-header placement would need to be changed and there would be major impact on header design in this solution. When compared to the solution where DL MAC CEs are placed in the beginning, there is no proven advantage in the solution, which would justify the added complexity.

When DL MAC CEs are placed after the SDUs, there could be a solution where the starting point of the sub-header is indicated with offset indicator placed at the very end of the PDU. In this solution, the impact on header design would be minimal as the DL CE sub-headers and CEs could be processed sub-header first and from left to right. It would also allow receiver to start CE processing before completing the SDU processing. This solution however, has added complexity and overhead caused by the indicator at the end when compared to the solution where DL MAC CEs are in the beginning.

Some companies argued to have indication field in the beginning of the PDU to imply whether the MAC CEs exist in the end. This solution would create undesirable data dependency between beginning and end of the PDU by requiring the transmitter to update the beginning of the PDU based on the result from the end. This solution would reduce the advantages gained from pre-processing and thus is undesirable option.

Placing DL MAC CEs before the SDUs enables UE to get the DL CE information earlier than placing it after the SDUs

Placing DL MAC CEs after the SDUs increases overhead and UE implementation complexity

Based on the reasoning above, placing the DL MAC sub-headers and corresponding DL MAC CEs to the beginning of the PDU allows transmitter pre-processing, fast receiver processing while minimizing the implementation complexity.

Figure 1: MAC CEs at the beginning

DL MAC CEs are placed at the beginning of PDU

## Padding and padding sub-header

Purpose of the padding sub-header is to provide information about the padding to the receiver. Padding sub-header allows receiver to identify that the padding will follow and on the transmitter side, it reduces the processing burden as there is no need to update the CE or SDU sub-headers after the headers are (pre) generated.

Alternative option to the padding sub-header is to place indication, such as ‘E’ bit to the sub-header prior to padding which indicates that padding follows. This option requires that the ‘E’ bit is always present in the SDU or CE sub-headers and it requires updating of ‘E’-bit when the position of the padding is known. Having padding sub-header removes the need for the ‘E’-bit and has advantage in sub-header pre-processing.

There is no need for ‘E’-bit in sub-header of CEs or SDUs when padding sub-header is always placed before padding

Padding sub-header is always placed before the padding

Padding sub-header can be static or dynamic, depending on if the length of the padding is included in the sub-header.  Having static padding sub-header allows transmitter to pre-process the padding and padding sub-header. For instance, padding sub-header and padding can be placed to the known memory location from where padding block is read based on the amount of padding required. Conversely, having dynamic sub-header with length field requires that the padding sub-header is updated separately after the amount of padding is known. This solution is not optimal for UE or gNB transmitter as it reduces the processing efficiency by requiring transmitter to update length field always in the padding sub-header. Need for the length field in the padding sub-header depends on the position of the padding and PDU design.

When padding sub-header and padding are placed at the very end of the PDU (Fig. 2), encountering the padding sub-header allows receiver to assume that the rest of the PDU is padding, Clearly, one of the advantages in this approach is that padding sub-header does not need to include the length field.

Figure 2: DL MAC CE with Padding and padding sub-header

Placing padding and padding sub-header after the SDUs but before the UL CEs (in the middle) does not make the padding sub-header length field mandatory. It is possible to indicate the length of the padding in the padding sub-header, but this solution would prohibit the pre-processing of the padding + padding sub-header as stated above. Alternative solution for this problem is to have two byte offset field always in the end of the PDU, which indicates the starting position of the UL MAC CEs (Figure 3). This will allow UE transmitter to pre-generate the padding and include the offset in the very end, when the UL CE start position is known. The solution allows receiver to extract the MAC CEs after the whole PDU is received but before all MAC SDUs have been parsed.

When padding sub-header has no length field, pre-processing of the padding sub-header and padding is possible

Placement of the padding sub-header and padding has impact whether padding sub-header length field is required

Padding sub-header does not include length-field

## UL Padding placement

Figure 3: UL MAC CE with Padding and padding sub-header

As discussed above, placing the padding sub-header after the MAC SDUs and indicating the position of the first UL CE sub-header with offset indicator has processing benefits in both, receiver, and transmitter side.

For UL, padding sub-header and padding are placed after the MAC SDUs and before the UL MAC CE sub-header. UL MAC CE follows the UL MAC CE sub-header immediately.

Position of the first UL MAC CE sub-header is indicated with indicator field of static size, which is positioned at the very end of the PDU (last field). The exact format of the indicator is FFS.

# Conclusion

In section 2 we made the following observations:

Observation 1	Placing DL MAC CEs before the SDUs enables UE to get the DL CE information earlier than placing it after the SDUs

Observation 2	Placing DL MAC CEs after the SDUs increases overhead and UE implementation complexity

Observation 3	There is no need for ‘E’-bit in sub-header of CEs or SDUs when padding sub-header is always placed before padding

Observation 4	When padding sub-header has no length field, pre-processing of the padding sub-header and padding is possible

Observation 5	Placement of the padding sub-header and padding has impact whether padding sub-header length field is required

Based on the discussion in section 2 we propose the following:

Proposal 1	DL MAC CEs are placed at the beginning of PDU

Proposal 2	Padding sub-header is always placed before the padding

Proposal 3	Padding sub-header does not include length-field

Proposal 4	For UL, padding sub-header and padding are placed after the MAC SDUs and before the UL MAC CE sub-header. UL MAC CE follows the UL MAC CE sub-header immediately.

Proposal 5	Position of the first UL MAC CE sub-header is indicated with indicator field of static size, which is positioned at the very end of the PDU (last field). The exact format of the indicator is FFS.

# References

R2-1702738, MAC PDU Design, Ericsson