3GPP TSG-RAN WG2 #98	R2-1704367

Hangzhou, P.R. of China, 15th – 19th May 2017

Agenda Item:	10.3.3.2

Source:	Ericsson

Title:	PDCP SN length

Document for:	Discussion, Decision

# Discussion

In table 1 and table 2, the PDCP bit rates are calculated for different PDCP PDU sizes, SN length, and RTTs.

Currently, it is agreed that NR supports at least a SN length value of 12 and 18 bits. It has been proposed to introduce a SN length value of 22 bits. However, the needs are still unclear. As shown in table 1, with a PDCP PDU size of a IP packet and a RTT of 30 ms, NR can achieve around ~50 Gbps, which above the expected NR bitrates.

While it can be argued that it is not possible to achieve the highest bit rates in conditions with poor RTTs and using a small PDCP PDU, we think that largest peak rates should be achieved for reasonable PDU sizes and good RTTs. In other words, the SN length should not have to be increased to cope with any kind of PDCP PDU size or any kind of RTT delay.

In Rel-15, the maximum PDCP SN length is 18 bits

Table 1	PDCP bit rates (Gbps) for different PDCP PDU sizes and SN lengths for RTT = 30 ms

|  |  | PDCP SN length (bits) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 7 | 12 | 15 | 18 |
| PDCP PDU (bytes) | 512 | 0,008 | 0,28 | 2,24 | 17,9 |
|  | 1502 | 0,025 | 0,82 | 6,56 | 52,5 |
|  | 8190 | 0,139 | 4,47 | 35,78 | 286,3 |

Table 2	PDCP bit rates (Gbps) for different PDCP PDU sizes and SN lengths for RTT = 50 ms

|  |  | PDCP SN length (bits) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 7 | 12 | 15 | 18 |
| PDCP PDU (bytes) | 512 | 0,005 | 0,17 | 1,34 | 10,74 |
|  | 1502 | 0,015 | 0,49 | 3,94 | 31,50 |
|  | 8190 | 0,084 | 2,68 | 21,47 | 171,76 |

For eMTC and NB-IoT, 7-bit PDCP SN length were specified in LTE. One of the reasons to have such small number was to minimize the overhead and, thus, improve coverage. Having 7-bits PDCP SN allows to reduce 1 byte.

If NR is supposed to support such services, it is important to already now discuss whether a PDCP SN length of 7 bits should be introduced and used only for UEs reporting the corresponding capability.

Discuss if 7-bit PDCP SN length is supported in Release 15. Support is indicated via a capability bit.

# Conclusion

Here there are two lists, one listing observations, and one listing proposals. Listing observations are not always needed, but listing proposals are always useful. To update a list, place the cursor inside the list and press F9.

Based on the discussion in section 2 we propose the following:

Proposal 1	In Rel-15, the maximum PDCP SN length is 18 bits

Proposal 2	Discuss if 7-bit PDCP SN length is supported in Release 15. Support is indicated via a capability bit.