3GPP TSG-RAN WG2 #98	Tdoc R2-1704373

Hangzhou, P.R. of China, 15th – 19th May 2017

Agenda Item:	10.3.3.1

Source:	Ericsson

Title:	PDCP unified reception algorithm

Document for:	Discussion, Decision

# Introduction

In RAN2#97bis, NR PDCP design was discussed and the following agreements were reached:

Agreements

-	A unified re-ordering schemes is used for DRB(s)/SRB(s) and UM and AM, with LTE as baseline.

-	It is desirable to disable PDCP reordering.  FFS how to signal it

-	Use First Missing COUNT (FMC) instead of FMS in the PDCP Status Report.

=> Use short name for PDCP state variables.

Agreements

1	NR PDCP aims at defining a single procedure for reception for a AM and UM, as well as DRB and SRB (will be concluded in stage 3 discussions)

2	NR PDCP procedures are based on COUNT instead of SN, which is determined in the beginning of the procedure. All NR PDCP state variables are based on COUNT.

3	Adopt modulo operation for arithmetic operation.

In this contribution, we discuss the design of the unified reception algorithm and provide a text proposal.

# Discussion

## Design principles for reception window operation

Generally, a reception algorithm can make use of PUSH based reception window operation and PULL based reception window operation.

- Push:

  - The lower window edge is pushed up by the highest delivered PDU (in order).

  - Data outside the window is considered old – and thus discarded

- Pull:

  - The upper window edge is pulled up by the highest received PDU (out of order)

  - Data outside the window is considered new – thus pulls up

These principles are illustrated in Figure 1.

Figure 1: SN space visualized for PUSH based reception window (left) and PULL based reception window (right)

In both schemes, a reordering timer could be employed, which would do out of order delivery at its expiry thus moving up the highest delivered PDU, which in turn would mean a push of the reception window in case of a PUSH window operation.

In LTE, a PUSH based reception operation is employed for bearers mapped on RLC AM, while a PULL based reception window operation is employed when mapped on RLC UM.

In the following we analyse how these principles are applicable to NR bearer types using RLC AM, RLC UM, both without or with split/duplication operation.

For AM, AM split, and AM duplication: following LTE, it is obvious that PUSH based operation should be employed.

For UM, one might argue that PULL reception mode is required due to the missing RLC acknowledgements to the receiver and risk that more than the reception window of data is brought in flight, which would have led to HFN desynch. However, it should be noted, that also for UM operation, this should be avoided, since also in PULL-based window operation, bringing more than the reception window of data in flight leads to HFN desynch and data loss. In UM, transmitter operation to limit the data in flight is implementation-specific and can e.g. be based on simply observing which data was transmitted or HARQ ACKs. In LTE PDCP, a NOTE had been specified indicating the implementation-specific nature of the HFN desynch avoidance by the UE in UL. Furthermore, it can be noted that in LTE WLAN aggregation the AM+reordering PUSH-based reception algorithm is employed where the WLAN-leg can be considered UM-like as well.

For UM with split, a reordering timer is required to reorder the PDUs received among the legs. This is part of both push and pull based reception algorithm. Hence, no further requirements on the used window type come from split operation.

For UM with duplication, duplicates may arrive late and must be discarded. This does not put any requirements on the need for push and pull either. However, considering that duplicates may arrive very late (due to these duplicates not discarded properly in the transmitter), they might be considered new in pull window operation and move the window falsely upwards, instead of being discarded, as in push window operation. Thus push window is preferably in this case.

For SRB and duplication, the AM push based approach can readily be reused. The only additional functionality of SRB is integrity verification, which can be simply inserted in the reception algorithm.

Both PUSH and PULL based reception window algorithms have the requirement that the transmitter must not bring more than the reordering window of data in flight. Otherwise, in both options, HFN desynch may occur.

In both RLC AM and RLC UM with and without split/duplication, the transmitter has sufficient (implementation specific) means to avoid HFN desynch.

## NR PDCP

From the discussion above it can be deduced that the requirements on the reception algorithm in PDCP are the same for both RLC AM and RLC UM operation with and without data split. With respect to SRB, where routing and duplication is considered in NR, SRB operation comes in NR with the same requirements as DRB operation on the PDCP reception algorithm. Thus, we propose:

Joint reception algorithm for DRBs and SRBs on RLC AM and RLC UM is employed, and uses current PDCP AM with reordering algorithm as baseline.

It had been agreed to base the PDCP procedures in NR on COUNT, as well as the state variables. Therefore, after determining COUNT of the received PDCP PDU from the SN in the PDU header, all subsequent operations can be based on COUNT instead of SN.

For arithmetic operations based on COUNT, modulo operation is not required. All PDCP state variables and related variables in reception algorithm are based on COUNT instead of SN. Modulo operation is thus not required after COUNT has been determined.

With the agreement to use short state variable names, we believe the following variables name provide both sufficient conciseness as well as descriptiveness. Their initial values are also given:

- RX_COUNT_NEXT: indicates COUNT of the next expected PDCP PDU to be received. Initial value is set to 0.

- RX_COUNT_DELIV: indicates COUNT of the last PDCP SDU delivered to upper layers. Initial value is MAX_COUNT-1, i.e. 2^32 -1.

- RX_COUNT_REORD: indicates COUNT+1 of the PDCP PDU that triggered t-Reordering.

Use only the following short receiver variables: RX_COUNT_NEXT, RX_COUNT_DELIV, RX_COUNT_REORD.

Note that the COUNT of the received PDU can be considered a local/temporary variable and is not a state variable. Note that with the usage of COUNT, no HFN-related state variables are needed anymore.

It had been agreed that disabling of PDCP reordering is desirable in NR. We believe that this behaviour should be configurable by RRC. This conditional behaviour can simply be integrated into the joint reception algorithm. If out of order delivery is activated, the received PDU is delivered to upper layers immediately at reception (if not discarded due to out of window or duplicate reception). The window status should however not be changed at this out of order delivery.

Out of order delivery from PDCP is configurable by RRC, and considered in the joint reception algorithm. Out of order delivery does not affect window status, i.e. RX_COUNT_DELIV.

The window status, i.e. the RX_COUNT_DELIV could be maintained following the legacy operation of the reordering timer (as implemented in text proposal below). However, as analysed in [2], a decoupled reordering time from the reception/discard-window could also be considered in NR, which would be beneficial for modern TCP algorithms. For further details, please see [2].

Legacy ROHC specifications expect in-order delivery of header compressed PDUs, while never versions support some degree of out of order delivery. Please refer to [1] for more discussion on ROHC in NR.  In the NR reception algorithm design, ROHC decompression can be considered to be applied after reordering, and with this also higher degrees of out of order delivery are supportable.

ROHC header decompression, if applicable, is applied after reordering when a PDCP SDU is delivered to higher layers.

For SRBs, integrity verification is applied, if applicable. This requirement can be readily integrated in the joint reception algorithm. When integrity verification fails, it should be indicated to higher layers, following LTE principles. For SRB mapped on split bearer with potentially using duplication, PDCP PDUs may arrive out of window and/or as duplicates. To be able to identify a potential threat of an attacker inserting duplicates, beside discarding those duplicates, the integrity of the duplicates should be verified and failure indicated to higher layers in this case (apparent duplicates were no real duplicates, but modified).

Integrity verification, if applicable, is applied at reception of a PDU. Following LTE, it is applied also for PDUs received out of the window and/or duplicates.

A text proposal for the NR PDCP unified reception algorithm is provided in the annex. It includes all proposals made in this document. We propose to incorporate it as the baseline algorithm in NR PDCP.

Agree to text proposal for joint reception algorithm (annex).

# Conclusion

In section 2 we made the following observations:

Observation 1	Both PUSH and PULL based reception window algorithms have the requirement that the transmitter must not bring more than the reordering window of data in flight. Otherwise, in both options, HFN desynch may occur.

Observation 2	In both RLC AM and RLC UM with and without split/duplication, the transmitter has sufficient (implementation specific) means to avoid HFN desynch.

Observation 3	For arithmetic operations based on COUNT, modulo operation is not required. All PDCP state variables and related variables in reception algorithm are based on COUNT instead of SN. Modulo operation is thus not required after COUNT has been determined.

Based on the discussion in section 2 we propose the following:

Proposal 1	Joint reception algorithm for DRBs and SRBs on RLC AM and RLC UM is employed, and uses current PDCP AM with reordering algorithm as baseline.

Proposal 2	Use only the following short receiver variables: RX_COUNT_NEXT, RX_COUNT_DELIV, RX_COUNT_REORD.

Proposal 3	Out of order delivery from PDCP is configurable by RRC, and considered in the joint reception algorithm. Out of order delivery does not affect window status, i.e. RX_COUNT_DELIV.

Proposal 4	ROHC header decompression, if applicable, is applied after reordering when a PDCP SDU is delivered to higher layers.

Proposal 5	Integrity verification, if applicable, is applied at reception of a PDU. Following LTE, it is applied also for PDUs received out of the window and/or duplicates.

Proposal 6	Agree to text proposal for joint reception algorithm (annex).

A text proposal with the discussed unified reception algorithm is provided in Section 5.

# References

R2-1704371, Header compression in NR, Ericsson, RAN#98, Hangzhou, P.R. of China, 15th – 19th May 2017

R2-1705318, PDCP reception and reordering window, Ericsson, RAN#98, Hangzhou, P.R. of China, 15th – 19th May 2017

# Text proposal

### 5.2.2	Receive operation

#### 5.2.2.1	General

The procedures and most variables used herein use the sequence number space of the full COUNT, i.e., 32 bit. The sequence number in the PDCP PDU header has a sequence number length shorter than or equal to Maximum_PDCP_SN in accordance with the DRB configuration. Hence, the received PDCP SN is converted to the full sequence number space prior to any other processing.

The following definitions apply in this section:

PDCP_WINDOW_SIZE is half the PDCP Sequence Number space and hence derived as follows:

-	PDCP_WINDOW_SIZE = (Maximum_PDCP_SN + 1) / 2

#### 5.2.2.2	Procedures when a PDCP PDU is received from the lower layers

Upon reception of a PDCP Data PDU with PDCP Sequence Number SN from lower layers, the UE shall:

-	set COUNT to SN + (RX_COUNT_NEXT & (Maximum_COUNT - Maximum_PDCP_SN));

-	if (COUNT <= RX_COUNT_NEXT - PDCP_WINDOW_SIZE)

[SN wraparound happened: assuming transmitter did not bring more than PDCP_WINDOW_SIZE in flight, COUNT value more than PDCP_WINDOW_SIZE behind RX_COUNT_NEXT is not possible, indicating that the previous estimated COUNT based on most significant bits of RX_COUNT_NEXT (HFN) is incorrect, i.e. must be incremented by one SN space]

-	set COUNT to (COUNT + (Maximum_PDCP_SN + 1))

-	Process the received PDCP PDU with COUNT in accordance with section 5.2.2.3.

NOTE: (Maximum_COUNT - Maximum_PDCP_SN) is used to determine the Most Significant Bits of the full-size COUNT based sequence numbers (i.e. HFN) by performing a binary AND function (“&” operator).

#### 5.2.2.3	Procedures for processing a received PDCP PDU with COUNT

For processing of a PDCP Data PDU with COUNT, the UE shall:

-	if (COUNT <= RX_COUNT_DELIV), or;

-	if a PDCP PDU with the same COUNT has been received before:

-	perform deciphering and verify integrity (if applicable) of the PDCP PDU using COUNT;

-	if integrity verification is applicable and integrity verification fails:

-	indicate the integrity verification failure to upper layer;

- 	discard the PDCP PDU;

-	else:

-	perform deciphering and verify integrity (if applicable) of the PDCP PDU using COUNT;

-	if integrity verification is applicable and integrity verification fails:

-	indicate the integrity verification failure to upper layer;

-	discard the received PDCP Data PDU;

- 	else:

-	if COUNT >= RX_COUNT_NEXT:

-	set RX_COUNT_NEXT to COUNT + 1;

-	store the PDCP SDU;

-	if outOfSequenceDelivery is configured

-	perform header decompression (if configured);

-	deliver the SDU to higher layers;

-	Update RX_COUNT_DELIV in accordance with section 5.2.2.4.

#### 5.2.2.4	Updating RX_COUNT_DELIV

-	if the SDU with COUNT = RX_COUNT_DELIV + 1 has been received:

-	if the SDU has not yet been delivered to higher layers:

-	perform header decompression (if configured);

-	deliver the SDU to upper layers;

-	set RX_COUNT_DELIV to RX_COUNT_DELIV + 1;

-	Update RX_COUNT_DELIV in accordance with section 5.2.2.4 (recursively);

-	else

-	maintain t-Reordering in accordance with section 5.2.2.5.

NOTE: The recursive execution of the function in this sub-section advances RX_COUNT_DELIV beyond the sequence number of the last PDCP PDU that has been received in sequence.

NOTE: Whether to stop and or (re-)start t-Reordering should be verified when RX_COUNT_DELIV cannot be advanced further.

#### 5.2.2.5	Maintaining t-Reordering

-	if t-Reordering is running:

-	if the PDCP SDU with RX_COUNT_REORD - 1 has been delivered to upper layers:

-	stop and reset t-Reordering;

-	if t-Reordering is not running (includes the case when t-Reordering is stopped due to actions above):

-	if there is at least one stored PDCP SDU that has not been delivered to upper layers:

-	set RX_COUNT_REORD to RX_COUNT_NEXT;

-	start t-Reordering;

##### 5.2.2.5.1	Procedures when t-Reordering expires

When t-Reordering expires, the UE shall:

-	perform header decompression (if configured) and deliver to upper layers in ascending order of the associated COUNT value:

-	all stored PDCP SDU(s) with COUNT value(s) less than RX_COUNT_REORD;

-	all stored PDCP SDU(s) with consecutive COUNT value(s) starting from RX_COUNT_REORD;

-	set RX_COUNT_DELIV to the COUNT of the last PDCP SDU delivered to upper layers;

-	if there is at least one stored PDCP SDU:

-	set RX_COUNT_REORD to RX_COUNT_NEXT.

-	start t-Reordering;

##### 5.2.2.5.2	Procedures when the value of t-Reordering is reconfigured

When the value of the t-Reordering is reconfigured by upper layers while the t-Reordering is running, the UE shall:

-	stop and restart t-Reordering;

-	set RX_COUNT_REORD to RX_COUNT_NEXT.