3GPP TSG-RAN2 Meeting #98			     R2-1704472

Hangzhou, China, 15th – 19th May, 2017

Agenda item:	   10.3.4.2 (NR_newRAT-Core)

Source:	LG Electronics Inc.

Title: 		Further considerations on QoS flow ID

Document for:	Discussion and Decision

1.	Introduction

According to TR 38.804, the main services and functions of SDAP layer, which is newly defined for the NR QoS framework, include:

- Mapping between a QoS flow and a DRB;

- Marking QoS flow ID in both DL and UL packets.

This document discusses QoS flow ID to be included in the SDAP header.

2.	Discussion

QoS flow is the finest granularity of QoS differentiation, and is identified by QoS Flow ID in the NR QoS framework. Traffic with the different QoS Flow ID value within a PDU session requires the different traffic forwarding treatment. In the downlink, the QoS Flow ID is carried in N3 encapsulation header with DL packet, and gNB maps the DL packet to a DRB based on the received QoS Flow ID and the associated QoS profiles. If AS or/and NAS-level reflective QoS is triggered, the QoS flow ID is transmitted in SDAP header with the DL packet. In the uplink, QoS flow ID is sent in SDAP header with UL packet so that gNB can determine the QoS marking which is carried in the N3 encapsulation header of the UL packet. Therefore, the QoS flow ID, which is included in the SDAP header as well as the N3 encapsulation header, needs to be unique within a PDU session.

Proposal 1:	QoS flow ID in SDAP header shall uniquely identify QoS flow within a PDU session.

As mentioned above, QoS flow ID is included in the DL and UL SDAP header which are attached to DL and UL packets, respectively. What is the “acceptable” amount of overhead introduced by such QoS flow ID in air interface? If it is smaller than the size of QoS flow ID defined by SA2, RAN2 should consider a shorten QoS flow ID. Therefore, we think that RAN2 need to know the size of the full QoS flow ID to decide whether to consider the shorten QoS flow ID.

Proposal 2:	RAN2 need to send LS to SA2 in order to ask the size of QoS flow ID.

3.	Proposal

In this document, we present our view on QoS flow ID. We have following proposals:

Proposal 1: QoS flow ID in SDAP header shall uniquely identify QoS flow within a PDU session.

Proposal 2: RAN2 need to send LS to SA2 in order to ask the size of QoS flow ID.