3GPP TSG-RAN WG2 NR #98 Meeting                                                    R2-1704503

Hangzhou, China, 15th-19th May 2017

Agenda item:	10.3.1.10

Source:	Samsung

Title:	RAN2 impacts from bandwidth part in NR

Document for:	Discussion & Decision

# 1. Introduction

RAN#75 in March approved a 5G WID [1] on New Radio access technology, which targets a unified framework for traffic with diverse QoS requirements e.g. enhanced mobile broadband (eMBB), massive machine-type-communications (mMTC), ultra-reliable and low latency communications (URLLC).

During NR SI/WI phases for Rel-15, the following agreements were achieved regarding bandwidth considerations for NR

RAN1 Agreements (#85)

- NR should support of flexible NW and UE channel bandwidth

- The NR physical-layer design should allow for fine granularity in terms of NR carrier bandwidth

- The NR physical-layer design should be such that devices with different bandwidth capabilities can efficiently access the same NR carrier regardless of the NR carrier bandwidth

RAN1 Agreements (Jan. AH)

- From RAN1 specification perspective, maximum channel bandwidth per NR carrier is [400, 800, 1000] MHz in Rel-15

  - RAN1 recommends RAN4 to consider at least 100 MHz maximum channel bandwidth per NR carrier in Rel-15 considering carrier frequency bands

    - RAN1 asks the feasibility of at least followings

      - For sub-6 GHz, 100 MHz is considered and for above-6 GHz, wider than 100 MHz is considered

      - Other cases can be considered by RAN4, e.g., 40 MHz, 200 MHz

  - Note that RAN1 will specify all details for channel bandwidth at least up to 100 MHz per NR carrier in Rel-15

  - Also note that RAN1 will consider scalable design(s) for up to maximum channel bandwidth per NR carrier

- From RAN1 specification perspective, the maximum number of NR carriers for CA and DC is [8, 16, 32]

- The maximum FFT size is not larger than [8192, 4096, 2048]

RAN1 Working Assumption (#88bis)

- One or multiple bandwidth part configurations for each component carrier can be semi-statically signalled to a UE

  - A bandwidth part consists of a group of contiguous PRBs

    - Reserved resources can be configured within the bandwidth part

  - The bandwidth of a bandwidth part equals to or is smaller than the maximal bandwidth capability supported by a UE

  - The bandwidth of a bandwidth part is at least as large as the SS block bandwidth

    - The bandwidth part may or may not contain the SS block

  - Configuration of a bandwidth part may include the following properties

    - Numerology

    - Frequency location (e.g. center frequency)

    - Bandwidth (e.g. number of PRBs)

  - Note that it is for RRC connected mode UE

  - FFS how to indicate to the UE which bandwidth part configuration (if multiple) should be assumed for resource allocation at a given time

  - FFS neighbour cell RRM

8.1 Numerologies and frame structure from TR 38.912

At least for single carrier operation, NR should allow a UE to operate in a way where it receives at least downlink control information in a first RF bandwidth and where the UE is not expected to receive downlink control information or data in a second RF bandwidth that is larger than the first RF bandwidth within less than X µs.

This contribution investigates design considerations to support wider BW in NR; especially about flexible BW support based on bandwidth part.

# 2. 	Design Issues for Bandwidth Part

  - Configuration of bandwidth part

At last meeting, RAN1 agreed to make a working assumption about BW part which can be configured to a UE semi-statically for RRC connected mode UE. The motivation of BW part configuration is to support wider BW compared to maximum BW capability of UE. The configured BW part is regarded as the operating BW of the UE at the instance. Network can configure multiple different BW part to RRC connected UEs to utilize wider BW efficiently. From UE side, at least one BW part should be configured and multiple BW parts can be configured by network decision based on UE BW capability. Multiple BW parts configuration may be useful for BW adaptation to save UE power consumption or for flexible scheduling mechanism in single CC.

Figure 1: The concept of bandwidth part

Observation 1: Bandwidth part is beneficial for both network and UE from the spectrum utilization, scheduling flexibility and UE power saving.

According to the configuration of BW part, the UE understands the PHY channel structure and other related parameters based on indication in the BW part configuration. RRC connection setup message would be a right place to deliver UE BW capability so that network can decide proper BW part configuration to the UE. L1 signaling to configure the BW part is not a favorable option since the information size for BW part may be too much in L1 signal. Furthermore the configured BW part may impact to other RRC related procedures such as RRM measurement. So reconfiguration of the BW part should be involved by RRC.

Proposal 1: One or multiple bandwidth part should be configured to UE per CC by RRC dedicated signaling.

The behavior of UE is restricted and segmented into the configured BW part, like CC in LTE. So UE monitors DL control channel which is defined in the BW part and measure CSI-RS for CSI feedback. If UE is assigned for DL in the BW part, UE performs HARQ procedure for the DL assignment. Since NR basically supports TDD, UE performs HARQ procedure for the UL grant in the BW part. In case of FDD, it needs further discussion how to configure UL resource.

Observation 2: Like LTE CC, UE can perform DL control channel monitoring, CSI-RS measurement and HARQ procedure in per-bandwidth part basis.

The location of NR-SS in frequency is in center frequency similar to LTE. 5/10 MHz for sub 6 GHz and 40/80 MHz for above 6 GHz has considered for BW of NR-SS. On the other hand, the maximum BW of single CC can be wide to at least 400 MHz. Therefore some functions may be limitedly available due to the huge gap between NR-SS BW and CC BW. For instance, if a UE has 20 MHz maximum BW capability and configured at the location that is far from the center by 100 MHz, the UE may not receive NR-SS for synchronization and RRM measurement purposes. Therefore at least one bandwidth part of UE should contain the bandwidth where NR-SS locates.

Observation 3: UE may not perform synchronization or RRM measurement in the configured bandwidth part, when NR-SS is not located within the configured BW part of the UE.

Proposal 2: At least one bandwidth part should contain NR-SS for synchronization or RRM measurement.

  - Scheduling based on bandwidth part

As identified above, at least one BW part should contain NR-SS and the BW part would be located in the center frequency of the CC. If the BW part of UE is configured only near the center frequency, network can’t use other frequency resources which are not contained in the BW part, and spectrum utilization is low. Therefore dynamic change of BW part at UE side is needed between 1st BW part nearby the center and 2nd BW part far from the center. The indication to change of BW part may be L1 signal, or MAC/RRC messages. L1 signal-based indication to change of BW part is the most dynamic among them by minimizing interval from indication to completion of BW change. Therefore it may be beneficial for some time-stringent NR functions to support L1 signal-based indication for dynamic change of BW part. For details about how to configure 1st and 2nd BW part and related issues, you can refer to the companion paper [2].

Observation 4: Dynamic change of bandwidth part is beneficial for higher spectrum utilization subject to UE requirement to monitor NR-SS.

Figure 2: Bandwidth part-based scheduling

Scheduling is one of time-stringent functions since it performs every scheduling opportunity and is sensitive to latency from the capacity and service requirement perspectives. In case of scheduling, as shown in Figure 2, the indication of BW part change could be realized by indication via control channel at one BW part for data transmission at another BW part. From the analogy to cross-carrier scheduling, we call indication of BW part change for scheduling by cross-bandwidth part scheduling or shortly cross-BWP scheduling. For details on resource indication for self/cross-BWP scheduling, please refer to the companion paper [3].

Proposal 3: RAN2 supports the cross-bandwidth part scheduling to efficiently use bandwidth of the network subject to the maximum bandwidth capability of UE.

On the other hand, some UE may not be capable to operate with low latency during RF retuning. If such limited capable UE is scheduled across BW part, the scheduling performance will be severely degraded due to higher overhead for every indication for scheduling. So it seems natural to support self-bandwidth part scheduling.

Proposal 4: RAN2 basically supports the self-bandwidth part scheduling for limited capable UE for RF retuning latency.

From last RAN2 meeting, an issue of higher power drain due to monitoring wider BW was raised during DRX discussion on DRX parameters. Since DRX concerns just time domain, DRX configuration is not sufficient to control the bandwidth adaptation. The bandwidth part configuration by RRC would be rather useful for BW adaptation, because the configured BW part information is commonly required for both BW adaptation for DL monitoring and BW part-based scheduling. By the way, it needs further discussion how BW part configuration is related to DRX operation.

Proposal 5: The bandwidth part configuration supports bandwidth adaptation during DL control channel monitoring for connected mode UE from the battery lifetime aspect.

  - Other issues

Latency for RF retuning

Since UE has a limited size of BW, UE can’t monitor larger BW than nor monitor different BW from that of itself at the same time. Due to physical processing in RF and BB, the latency is expected for completing change of operating BW. From the companion paper [3], we investigated possible value of latency from several cases. If BW is changed while keeping the same center frequency, the possible latency is 1s of us. However if BW is changed totally with changing the center frequency, the possible latency is 100s of us. It requests RAN2 to consider the latency for BW retuning as a design constraint. So the following operation was agreed during NR SI phase (refer to TR38.912):

“At least for single carrier operation, NR should allow a UE to operate in a way where it receives at least downlink control information in a first RF bandwidth and where the UE is not expected to receive downlink control information or data in a second RF bandwidth that is larger than the first RF bandwidth within less than X µs.”

Proposal 6: RAN2 considers RF retuning latency to design dynamic change of bandwidth part.

# 3. Summary

Based on the discussion and identified observations in this contribution, we propose followings:

Observation 1: Bandwidth part is beneficial for both network and UE from the spectrum utilization, scheduling flexibility and UE power saving.

Observation 2: Like LTE CC, UE can perform DL control channel monitoring, CSI-RS measurement and HARQ procedure in per-bandwidth part basis.

Observation 3: UE may not perform synchronization or RRM measurement in the configured bandwidth part, when NR-SS is not located within the configured BW part of the UE.

Observation 4: Dynamic change of bandwidth part is beneficial for higher spectrum utilization subject to UE requirement to monitor NR-SS.

Proposal 1: One or multiple bandwidth part should be configured to UE per CC by RRC dedicated signaling.

Proposal 2: At least one bandwidth part should contain NR-SS for synchronization or RRM measurement.

Proposal 3: RAN2 supports the cross-bandwidth part scheduling to efficiently use bandwidth of the network subject to the maximum bandwidth capability of UE.

Proposal 4: RAN2 basically supports the self-bandwidth part scheduling for limited capable UE for RF retuning latency.

Proposal 5: The bandwidth part configuration supports bandwidth adaptation during DL control channel monitoring for connected mode UE from the battery lifetime aspect.

References

[1] RP-170823, New WID Proposal: Work Item on New Radio (NR) Access Technology

[2] R1-1708060, “Wider Bandwidth  Operations”, Samsung, May 2017

[3] R1-1708017, “DL Resource  Allocation Aspects”, Samsung, May 2017

[4] R2-1704508, “RRM considerations for adaptive BW”, Samsung, May 2017