3GPP TSG-RAN WG2 #98	Tdoc R2-1704683

Hangzhou, P.R. of China, 15th – 19th May 2017	update of R2-1702666

Agenda Item:	10.3.1.8

Source:	Ericsson

Title:	General HARQ aspects of SPS UL

Document for:	Discussion, Decision

# Introduction

In RAN2#97bis, the following agreements were made in the chairman’s notes[1]:

Agreements on grant-free

=>	From RAN2 point of view it would be beneficial to be able to share “SPS/grant free” UL resources amongst different UE.  Mechanism to identify the UE for collision resolution purpose may be needed.   The details can be discussed in RAN1.

Agreements on SPS:

-	Like in legacy LTE, at least SPS period is configured by RRC. FFS how frequency resources, MCS, etc., for SPS are provided to the UE depends on RAN1 discussion.

-	UL skipping for dynamic grant should be configurable.  FFS if UL skipping for SPS is configurable

-	Working assumption:  Like in LTE, DRX behaviour with SPS UL should be to restart inactivity timer when UL data is transmitted, and not to restart when SPS UL grant is not used.  This behaviour depends on outcome of DRX design.

In this paper, we discuss general HARQ aspects of SPS UL in NR, related with the asynchronous HARQ operation. An enhancement for HARQ feedback mode for data with high reliability requirement is provided in [2], our views on skip uplink is in [3] and some further SPS enhancemets are in [4].

# Discussion

In this paper, we discuss how to incorporate the asynchronous HARQ operation in NR SPS. In light of reliability requirement, an enhanced HARQ feedback mode to tackle lost initial transmission can be found in the companion paper [2].

## Asynchronous UL feedback – assuming ACK

Asynchronous HARQ feedback has been agreed in RAN1#85 that "NR should support at least asynchronous hybrid ARQ in the DL and UL to avoid fixed timing relationship between initial transmission and re-transmission”. In LTE, a fixed timing is configured so that UE knows implicitly when the feedback will be sent and proper actions are taken if no feedback is received. In NR, this agreement means that the feedback might be transmitted from gNB at any given time, and UE does not have a well-defined reference point on assuming no feedback is sent. To limit this uncertainty in NR, a maximum feedback time T can be explicitly configured for the UE to wait for feedback. As a starting point, the time T can be configured to be equal to the expected HARQ round trip time. Additionaly, this timer should be reset after a transmission or retransmission.

With asynchronous HARQ feedback in SPS UL, a configurable maximum feedback time T is beneficial for a UE to wait for feedback after an UL transmission.

To always send an explicit positive or negative feedback for each UL transmission induces a large signalling load in the downlink. For reduced load of the DL control channel, in some scenarios certain feedback transmissions can be skipped without substantially degrading the UL transmission performance. For instance, UE can interpret whether the UL grant is received or not as a form of HARQ feedback. If a UL grant for retransmission is received, then this indicates a failure at the receiver. Otherwise if nothing is received, then the UE considers this as a successful transmission. Since the UL BLER value usually would be kept within a small value, this procedure saves most of signalling overhead for feedback. This is the case for LTE.

- Proposal 1	As in LTE SPS UL, retransmission for SPS UL transmission based on a dynamic UL grant is supported.

Furthermore, the UE keeps the data in the buffer, and waits for the UL-grant for a maximum time T. If no UL-grant is received when the same HARQ process within the maximum time T, then the UE assumes that the data is successfully delivered. Then the UE proceeds and can reuse the HARQ process for new data, and the buffer is overwritten by the new data. In other words, UE assumes ACK as long as there is no feedback received. In addition, the data transmision buffer is overwritten by new data transmission unless a UL grant for retransmission is received within the maximum time T.

With UL grant-indicated feedback in SPS, UE assumes ACK and data buffer is overwritten by new transmission unless a UL grant for retransmission is received within the maximum feedback time T.

## Multiple HARQ processes

During the maximum feedback time T, data has to be kept in the HARQ buffer in case if UL grant for retransmission is received, and no new data can be transmitted on the same HARQ process. Hence, unless the UE is configured with multiple SPS UL HARQ processes it can’t send new data during T.

If the UE is configured with only one SPS UL HARQ process it is prevented from sending new data until the time T after a transmission.

If new SPS UL transmissions are possible during the time T the latency can be reduced. Therefore, multiple SPS UL HARQ processes should be supported.

- Proposal 2	As in LTE SPS UL, multiple HARQ processes are supported in NR.

## HARQ process ID (PID)

In LTE SPS and fast UL framework with multiple HARQ processes, the HARQ PID is not indicated. Formulas have been specified (TS 36.321) to derive the HARQ PID from the absolute system frame number (SFN) and subframe number (which is known in both eNB and UE). In other words, the HARQ PID is synchronized, and eNB and UE are in-sync on which process ID to use on each TTI. As follows from the discussion above, this synchronicity restricts either gNBs flexibility in scheduling retransmissions, or UE’s ability to send new data. In NR, this approach becomes even more complicated with the mixure of slots/mini-slots and different numerologlies. A more flexible solution of process ID determination for SPS UL in NR would therefore be desirable.

Deriving SPS HARQ PID from the frame number, as in LTE SPS UL, might restrict the re-scheduling flexibility offered by asynchronous HARQ and may become more complicated with mini-slots and different numerologies.

One approach is to extend UCI format, and transmit the HARQ process IDs in the UCI.  Furthermore, the UCI could be sent together with the data, however, it must be separately decoded by gNB. The most flexible approach is that UE decides which HARQ process ID to use. However, the UCI has to be decoded sucesfully by gNB thereby the gNB can schedule the re-transmissions for the recevied process ID. The succesfull decoding might be significantly important for data with high reliability requirement, such as URLLC data. This also puts a limitation on the number of bits in the UCI. Moreover, this extended and reliable UCI format may lead to extra overhead in SPS UL configured with small data and mini-slots.

- Proposal 3	In SPS UL, it is beneficial to convey HARQ PID in the initial transmission.

In another approach, the HARQ feedback contains a backwards-counting pointer that refers to which initial transmission this HARQ feedback is about. The pointer can mean the HARQ feedback is related with, for example four or five, configured UL transmission resources before. The UL transmission resources can be in either mini-slots or slots. The pointer can use the same UL-grant DCI format and put the numerical value in the process ID field. The DCI can be further scrambled by SPS-RNTI to reduce the mis-interpretation risks.

The main advantage of this approach is that it works under the current framework. In other words, it does not require extra physical channel design and also does not incur extra UL overhead. However, such an optimized approach may run into the risk that any further extension for SPS UL might be difficult.

As another approach, if no HARQ PID is signaled in the initial transmissions for SPS UL, the UL-grant from gNB for retransmission contain a pointer pointing to the initial transmission.

# Conclusion

For general HARQ aspects of SPS UL in NR, related with asynchronous HARQ operation we made the following observations and proposals in section 2:

Observation 1	With asynchronous HARQ feedback in SPS UL, a configurable maximum feedback time T is beneficial for a UE to wait for feedback after an UL transmission.

Observation 2	With UL grant-indicated feedback in SPS, UE assumes ACK and data buffer is overwritten by new transmission unless a UL grant for retransmission is received within the maximum feedback time T.

Observation 3	If the UE is configured with only one SPS UL HARQ process it is prevented from sending new data until the time T after a transmission.

Observation 4	Deriving SPS HARQ PID from the frame number, as in LTE SPS UL, might restrict the re-scheduling flexibility offered by asynchronous HARQ and may become more complicated with mini-slots and different numerologies.

Observation 5	As another approach, if no HARQ PID is signaled in the initial transmissions for SPS UL, the UL-grant from gNB for retransmission contain a pointer pointing to the initial transmission.

Proposal 1	As in LTE SPS UL, retransmission for SPS UL transmission based on a dynamic UL grant is supported.

Proposal 2	As in LTE SPS UL, multiple HARQ processes are supported.

Proposal 3	In SPS UL, it is beneficial to convey HARQ PID in the initial transmission.

# References

RAN2-97bis Spokane Chairman notes

R2-1704684, Enhanced HARQ feedback mode in SPS

R2-1704382, SPS UL skipping in NR

R2-1704383, SPS Enhancements in NR