3GPP TSG RAN WG2 Meeting #98	R2-1704685

Hangzhou, China, 15-19 May, 2017

Agenda Item:	9.3.1

Source: 	CATT

Title:  	Summary of email discussion [97bis#28][LTE/UDC] Continued simulation and comparison of solutions on UDC (CATT)

Document for:	Discussion and Decision

Introduction

This document summarizes the following email discussion:

[97bis#28][LTE/UDC] Continued simulation and comparison of solutions on UDC (CATT)

-	Based on simulation assumptions agreed in this meeting

-	The continued simulation should focus on RoHC

-	Mix traffic should be considered in the simulation

Intended outcome: email discussion report

Deadline: 04/27/2017

According to the online discussion during RAN2#97bis meeting, the following aspects should be discussed and considered in this email discussion:

- Long duration ftp file should be provided for simulation;

- Mixed traffic should be considered in the simulation;

- Simulation results for RoHC should be provided

These aspects are identified in each section.

Description

Long duration ftp file

During RAN2#97bis meeting, it is proposed to use long duration ftp data for simulation. Companies are encouraged and appreciated to provide long duration ftp data for simulation in a zip file with file name,ftp data-company name.zip.The data file format should be “.pcap” which only includes UL traffic. Please don’t insert the zip file(s) in this document, just indicate the file name in the table below.

Question 1: Companies can provide ftp data for simulation

| Company name | file name of the data for simulation |
| --- | --- |
| CATT | Thanks MTK for providing a long duration ftp file in ftp data-MediaTek.pcap. |

Mixed traffic file

According to the online discussion, mixed traffic should be considered in the simulation. Mixing of already available traffic data file could be used to emulate mix traffic environment. How to mix the traffic files can be discussed first.

- Mixed 2 files, which files?

- Mixed 3 files, which files?

Currently, we have the following data for simulation:

| 1 | FTP- Client (CMCC) |
| --- | --- |
| 2 | FTP- Server (CMCC) |
| 3 | Online video (CMCC) |
| 4 | Long period video (CMCC) |
| 5 | SIP UE1(CMCC) |
| 6 | SIP UE2 (CMCC) |
| 7 | SIP UE3 (CMCC) |
| 8 | Web surfing (CMCC) |
| 9 | Video data (MediaTek) |
| 10 | Long duration FTP (Media Tek) |
| 11 | Multiple IP flows (Qualcomm) |

Companies can select two or three traffic data files from these 10 files or use the 11th file directly.

Question 2a: Companies are requested to express their views on which files to be mixed.

| Company name | Comments/Suggestions |
| --- | --- |
| Ericsson | The purpose of this is to study UDC behaviour when it is applied to Default bearer (which carries mix of Internet Traffic with different destination IP addresses). How would the UDC handle that and what is the impact on UDC packet buffer and the compression gain? It would be good to have a new PCAP file from real network where UE is basically using the default bearer. Otherwise, 8 +9 or 8 + 10 can be used |
| Qualcomm | We agree with Ericsson that it is better to use input PCAP file captured from real networks. As we know, traffic pattern has big impact on the evaluation results, so we should use real traffic and not artificiallycreated traffic. To keep ourselves neutral, we have been waiting for operators andother companiesto provide pcap files from real networks. However, we have not seen it until 4PM 4/26 Pacific time (one day before deadline 4/27/2017). Therefore, Qualcomm would like to provide a web-browsing PCAP file with multiple destination IP addresses. It was captured in regular web-browsing. We welcome operators and other companies to provide similar PCAP files with multiple destination IP addresses. Our PCAP file name is: BrowseMultipleIpFlow-Qualcomm.zip The UDC simulator should skip the first 16 Bytes of each packet. The first 16 Bytes are the headers below IP layer. |
| CATT | Thanks Qualcomm for providing the multiple flows file. We can use this input file for simulation of mixed traffic rather than emulating the mixed traffic affect by mixing different traffic inputs. Also we think it is reasonable to show the impacts on the comparison efficiency by mixing of the traffic inputs, e.g. mixed traffic input 8+4, 8+9 or 8+10. |
| MediaTek | We also agree to evaluate the mixed traffic scenario. In the following, we provide the evaluation results of “8+10” and “multiple IP flows (Qualcomm)”. |

How to mix the selected data files should be considered. The following options can be considered in our simulation:

Option 1): average mixed: messages in one file will be distributed evenly among all messages in another data file, e.g. assume one data file has 6 messages, another file has 3 messages, following pattern illustrates mixed traffic.

Option 2): inserts data from one file in the middle of data from another file. This may emulate the data arrival in a typical mix service scenario..

Option 3): random mix: the compressor randomly selects one of these files, one packet each time.

Question 2b: Companies are requested to express their views onhow to mix the selected files.

| Company name | Comments/Suggestions |
| --- | --- |
| Ericsson | Good to have real network PCAP file. Otherwise; Option 1 or 2 is fine. |
| Qualcomm | Agree with Ericsson. Qualcomm would like to provide a web-browsing PCAP file with multiple destination IP addresses. It was captured in regular web-browsing. We welcome and appreciate operators and other companies to provide similar PCAP files with multiple destination IP addresses. |
| CATT | Same as the comments above, input file 11 can be used. Additionally the simulation result of e.g. mixed 8+4, 8+9 or 8+10 with option 1 and option 2 could also be considered. |
| MediaTek | We agree that Option 1 or 2 are possible evaluation method. |

Question 2c: Companies could provide simulation results on mixed traffic below.

Simulation results on mixed traffic from CATT

Both the multiple IP flow (input traffic file 11) provided by Qualcomm and mixed Web surfing (input traffic file 8) and Long period video (input traffic file 4) traffic provided by CMCC with option 1, 2 and 3 are used in the simulation. Corresponding simulation results are:

|  | Original Size（Byte） | 8K buffer |  | 32K buffer |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Compressed Size（Byte） | Compression Efficiency | Compressed Size（Byte） | Compression Efficiency |
| Input file 4+8 Option 1 | 3753581 | 1220693 | 67.48% | 1071819 | 71.45% |
| Input file 4+8 Option 2 |  | 1151601 | 69.32% | 1026762 | 72.65% |
| Input file 4+8 Option 3 |  | 1220407 | 67.49% | 1067292 | 71.57% |
| Multiple IP flows (QC) | 5319100 | 1434672 | 73.03% | 1336519 | 74.87% |

Simulation results on mixed traffic from MediaTek

| Scenario \ window size | Zlib 8K | Zlib 32K | RoHC |
| --- | --- | --- | --- |
| Input file 8 | 66.3% | 70.1% | 23.1% |
| Input file 10 | 63.6% | 60.0% | 83.4% |
| Input file 8+10\Option-2 | 65.6% | 67.4% | 39.3% |

Simulation results on mixed traffic from Qualcomm for Solution 4

The same simulation results are obtained for setups with 8K and 32K compression buffer sizes. The results are shown below.

| PCAP File # | PCAP File | Original Size (Bytes) | Compressed Size (Bytes) | Compression Efficiency (%) |
| --- | --- | --- | --- | --- |
| 11 | Multiple IP flows (Qualcomm) | 5319100 | 1312299 | 75.32 |

Simulation results for RoHC

During the email discussion after RAN2#97 meeting, only one company provided simulation results of RoHC. According to the online discussion in RAN2#97bis meeting, more simulation results for RoHC should be provided.

Question 3: Companies could provide more simulation results on RoHC below.

Simulation results on RoHC from CATT

The simulation results of ROHC are provided in the table below:

|  | Original Size（Byte） | RoHC Compressed Size（Byte） | Compression Efficiency |
| --- | --- | --- | --- |
| FTP – Client (CMCC) | 1211 | 538 | 55.57% |
| Video data (MTK) | 2453749 | 842739 | 65.66% |
| SIP UE1 (CMCC) | 51020 | 48677 | 4.59% |
| SIP UE2 (CMCC) | 32680 | 31225 | 4.45% |
| SIP UE3 (CMCC) | 46688 | 45755 | 2.00% |

Simulation results on RoHC from MediaTek

| Scenario | RoHC | Size of TCP/IPheaders |
| --- | --- | --- |
| FTP- Client (CMCC) | 73.3% | 90.8% |
| FTP- Server (CMCC) | 59.7% | 73.4% |
| Online video (CMCC) | 21.7% | 29.1% |
| Long period video (CMCC) | 45.1% | 58.1% |
| SIP UE1(CMCC) | 5.4% | 7.5% |
| SIP UE2 (CMCC) | 5.1% | 7.1% |
| SIP UE3 (CMCC) | 4.4% | 6.2% |
| Web surfing (CMCC) | 23.1% | 31.3% |
| Video data (MediaTek) | 80.7% | 95.9% |
| FTP data-MTK | 83.4% | 99.96% |

….

Updated or More simulation results of each solutions

In this section, companies can provide more simulation results on new files by using proposed solutions.

New simulation results from CATT

The simulation results of solution 3 for all of the input traffic files are updated (new results are highlighted in yellow):

|  | Original Size（Byte） | 8K buffer |  | 32K buffer |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Compressed Size（Byte） | Compression Efficiency | Compressed Size（Byte） | Compression Efficiency |
| FTP- Client (CMCC) | 1211 | 585 | 51.69% | 585 | 51.69% |
| FTP- Server (CMCC) | 1782 | 962 | 46.02% | 962 | 46.02% |
| Online video (CMCC) | 13450 | 4632 | 65.56% | 4633 | 65.55% |
| Long period video (CMCC) | 1371861 | 365346 | 73.37% | 337360 | 75.41% |
| SIP UE1(CMCC) | 51020 | 6639 | 86.99% | 5997 | 88.25% |
| SIP UE2 (CMCC) | 32680 | 4921 | 84.94% | 4791 | 85.34% |
| SIP UE3 (CMCC) | 46688 | 5927 | 87.31% | 5313 | 88.62% |
| Web surfing (CMCC) | 2381720 | 786295 | 66.99% | 689638 | 71.04% |
| Video data (MediaTek) | 2453749 | 950644 | 61.26% | 983524 | 59.92% |
| Long period FTP (MediaTek) | 879630 | 317485 | 63.91% | 347815 | 60.46% |
| Multiple IP flow (QC) | 5319100 | 1434672 | 73.03% | 1336519 | 74.87% |

New simulation results from MediaTek

- Based on the discussions in RAN2#97bis meeting, we would like to further clarify our evaluation assumptions. Zlib v1.2.11 is used in our evaluation (http://zlib.net/). The relevant parameter settings follows http://zlib.net/manual.html, which are

    - level(0-9): Z_DEFAULT_COMPRESSION(6)

    - method: Z_DEFLATED

    - windowBits(9-15): 13(8K), 15(32K)

    - memLevel(1-9): 8

    - strategy: Z_DEFAULT_STRATEGY

- Our evaluation results can be found in the following table.

|  | Zlib-based UDC (8K) | Zlib-based UDC (32K) |
| --- | --- | --- |
| FTP- Client (CMCC) | 50.5% | 50.5% |
| FTP- Server (CMCC) | 45.1% | 45.1% |
| Online video (CMCC) | 65.1% | 65.1% |
| Long period video (CMCC) | 72.9% | 73.9% |
| SIP UE1(CMCC) | 86.7% | 88.1% |
| SIP UE2 (CMCC) | 84.2% | 85.3% |
| SIP UE3 (CMCC) | 87.2% | 88.5% |
| Web surfing (CMCC) | 66.3% | 70.1% |
| Video data (MediaTek) | 60.7% | 59.1% |
| FTP (MediaTek) | 63.6% | 60.0% |
| Multiple IP flow (QC) | 72.3% | 74.3% |

New simulation results from Qualcomm for Solution 4

The same simulation results are obtained for setups with 8K and 32K compression buffer sizes. The results are shown below. New results are highlighted.

| PCAP File # | PCAP File | Original Size (Bytes) | Compressed Size (Bytes) | Compression Efficiency (%) |
| --- | --- | --- | --- | --- |
| 1 | FTP data-CMCC(UL-client) | 1211 | 548 | 54.74 |
| 2 | FTP data-CMCC(UL-server) | 1782 | 884 | 50.39 |
| 3 | Video data-CMCC(UL) | 13450 | 5105 | 62.04 |
| 4 | long period Video data-CMCC(UL) | 1371861 | 295658 | 78.44 |
| 5 | SIP signalling-CMCC 01(UL) | 51020 | 7337 | 85.61 |
| 6 | SIP signalling-CMCC 02(UL) | 32680 | 5827 | 82.16 |
| 7 | SIP signalling-CMCC 03(UL) | 46688 | 6561 | 85.94 |
| 8 | web surfing-CMCC(UL) | 2381720 | 767990 | 67.75 |
| 9 |  |  |  |  |
| 10 | Long duration FTP (MediaTek) | 879630 | 216910 | 75.34 |
| 11 | Multiple IP flows (Qualcomm) | 5319100 | 1312299 | 75.32 |

Any other topic for discussion

[MediaTek] We suggest companies can provide enough parameter configurations and compression libraries. It can facilitate cross-check performance results among different compression methods.

[Qualcomm] As requested in some companies, we have updated Solution 4 details and shared it in this email discussion thread to address the comments. We will also submit these detailed descriptions to the next RAN2 meeting.

Summary of the email discussion

Four companies participated in the discussion while three companies provided the simulation results. The simulations were carried out with the use of input files from MediaTek (long ftp file) and Qualcomm (mixed traffic file).

Mixed traffic profile:

Significant compression gain is shown with all UDC solutions when applying on mixed traffic profile. Simulation results for mixed traffic were derived based on combination of input traffic profiles as well as mixed traffic profile captured in a practical system. It can be observed that the compression gain of mixed traffic is between the compression gains resulted from individual traffic profiles.

Mixed traffic profile provided by Qualcomm was simulated with solution 3 and solution 4. Both solutions resulted around 75% of compression gain on the considered mixed traffic input.

Observation 1: It can be concluded that mixed traffic could also be compressed using UDC solution and a significant compression gain can be achieved.

Use of UL RoHC:

Simulation results using UL RoHC were provided by two companies. Compression gain of SIP signalling is in the range of 5% while compression gain is achieved using RoHC in the order of 50 to 80% on other traffic. It was noted that the compression gain by UL RoHC has correlation to the size of TCP/IP header ratio. If the TCP/IP header ratio is high, the compression gain achieved with RoHC is also high as expected from a header compression scheme.

Observation 2: compression gain of UL RoHC depends on the size of TCP/IP header ratio and depending on the traffic input, RoHC could achieve up to 80% compression gain.

Long ftp traffic profile:

Consistence results are shown for simulation of long ftp traffic input, where around 60% compression gain is achieved with solution 2 and solution 3 while compression gain of 75% achieved with solution 4.

Observation 3: it could conclude that significant compression gain of ftp input profile can be achieved with UDC solutions.

Additionally, possibility to facilitate cross-checking of different UDC solutions was highlighted by one company.

Detail of solution 4 was also provided during the email discussion.

Conclusion/Proposals

Based on summary of the email discussion the following proposals were made:

Proposal 1: Mixed traffic, representing a nature of traffic in a practical system could also be compressed using UDC solution and a significant compression gain can be achieved.

Proposal 2: Compression gain of UL RoHC depends on the size of TCP/IP header ratio and depending on the traffic input, RoHC could achieve up to 80% compression gain.

Proposal 3: Significant compression gain of ftp input traffic profile can be achieved with UDC solutions.

Proposal 4: To capture additional results provided for each solution in the TR 36.754.

Reference

[1]	R2-1702943		Summary of Email Discussion [97#61][LTE/UDC]  (CATT)	CATT(Rapporteur)