3GPP TSG-RAN WG2 Meeting #98	R2-1704750

Hangzhou, P. R. China, 15-19 May 2017

Agenda Item:	9.8.3

Source: 	Intel Corporation

Title:	IMU sensor based positioning

Document for:	Discussion and decision

# 1.	Introduction

3GPP has approved the WI [2] on support for Inertial Measurement Unit (IMU) based positioning for LTE with the intention of supporting IMU sensors, namely, accelerometer and gyroscope.

In this discussion paper we propose the importance of IMU sensors as well as the enhancements needed to LPP to support IMU sensors as envisaged in the WID [2].

# 2.	Discussion

Positioning methods like OTDOA and ECID can be used for positioning. However, the horizontal accuracy of these methods is in tens of meters. Moreover these positioning methods are not very efficient in detecting sudden, minor movements of UE thereby leading to  inaccurate measurements and hence an inacurate user position computation, especially in indoor scenarios. These positioning methods need to be augmented with measurements from other sources.

Rel-13 indoor positioning introduced WLAN and BT based measurements. However these rely on external entities like WLAN Access points or BT beacons to measure the RSSI and RTT values. In geolocations that lack this infrastructure, the UE will be handicapped and may have to rely only on OTDOA and ECID measurements. Moreover, in case of WiFi and BT, the UE may have to constantly measure the RSSI values with respect to the access points and beacons to accurately predict the movement of users which is a strain on resources for battery operated UE.

Observation 1: There is a need to augment the position computation process with measurements from additional sensors, for example, IMU sensors like accelerometer and gyroscope.

An accelerometer measures the change in velocity of the UE. When these measurements are used along with measurements from one or more of OTDOA, ECID, WiFi, BT, a very accurate position of the UE can be computed. Also, an accelerometer is capable of generating measurements within short intervals of time without involving interactions with external entities.

A gyroscope measures angular velocity with respect to a known/predefined fixed axis. When accelerometer is used in conjunction with gyroscope it results in prediction of linear movement as well as rotation of the UE. The E-SMLC server can then use these measurements to compute the accurate position of the user.

Proposal 1:  to introduce accelerometer and gyroscope sensor measurements that measure linear acceleration and angular velocity respectively.

The Text Proposal to Stage-2 LTE Positioning spec TS 36.05 is added as Annexure A.

# Annexure A

# Text Proposal to TS 36.305

## 4.3	Standard UE Positioning Methods

The standard positioning methods supported for E-UTRAN access are:

-	network-assisted GNSS methods;

-	downlink positioning;

-	enhanced cell ID method;

-	uplink positioning;

-	barometric pressure sensor method;

-	WLAN method;

-	Bluetooth method;

-	Terrestrial Beacon System method;

-	Accelerometer sensor method;

-	Gyroscope sensor method.

Hybrid positioning using multiple methods from the list of positioning methods above is also supported.

Standalone mode (e.g. autonomous, without network assistance) using one or more methods from the list of positioning methods above is also supported.

These positioning methods may be supported in UE-based, UE-assisted/E-SMLC-based, eNB-assisted, and LMU-assisted/E-SMLC-based versions. Table 4.3-1 indicates which of these versions are supported in this version of the specification for the standardised positioning methods.

Table 4.3-1: Supported versions of UE positioning methods

| Method | UE-based | UE-assisted, E-SMLC-based | eNB- assisted | LMU-assisted/ E-SMLC-based | SUPL |
| --- | --- | --- | --- | --- | --- |
| A-GNSS | Yes | Yes | No | No | Yes (UE-based and UE-assisted) |
| Downlink Note1 | No | Yes | No | No | Yes (UE-assisted) |
| E-CID | No | Yes | Yes | No | Yes (UE-assisted) |
| Uplink | No | No | No | Yes | No |
| Barometric | Yes | Yes | No | No | No |
| WLAN | Yes | Yes | No | No | Yes |
| Bluetooth | No | Yes | No | No | No |
| TBS Note 2 | Yes | Yes | No | No | Yes (MBS) |
| Accelerometer | No | Yes | No | No | No |
| Gyroscope | No | Yes | No | No | No |
| NOTE 1: This includes TBS positioning based on PRS signals. NOTE 2: In this version of the specification only for TBS positioning based on MBS signals. |  |  |  |  |  |

Barometric pressure sensor, Accelerometer sensor, Gyroscope sensor, WLAN, Bluetooth, and TBS positioning methods based on MBS signals are also supported in standalone mode, as described in the corresponding sections.

### 4.3.x	Accelerometer sensor positioning

# The accelerometer sensor method makes use of accelerometer sensors to determine the change in linear velocity of the UE. The UE measures the linear acceleration of the UE to send the measurements to the positioning server for position calculation.

# This method should be combined with other positioning methods to determine the 3D position of the UE.

# The operation of the accelerometer sensor positioning method is described in clause 8.x.

### 4.3.y	Gyroscope sensor positioning

# The gyroscope sensor method makes use of gyroscope sensor to determine the angular velocity of the UE. The UE measures the angular velocity of the UE to send the measurements to the positioning server for position calculation.

# This method should be combined with other positioning methods to determine the 3D position of the UE.

# The operation of the accelerometer sensor positioning method is described in clause 8.y.

### 5.3.1	User Equipment (UE)

The UE may transmit the needed signals for uplink-based UE Positioning measurements and may make measurements of downlink signals from E-UTRAN and other sources such as different GNSS and TBS systems, WLAN access points, Bluetooth beacons, UE barometric sensors, accelerometer sensors and gyroscope sensors. The measurements to be made will be determined by the chosen positioning method.

## 8.x	Accelerometer sensor positioning methods

### 8.x.1 General

In accelerometer positioning method, the acceleration of the UE is estimated. This is accomplished by the accelerometer sensor. Using the measured acceleration and measurements from other positioning methods, the position of the UE is computed.

The measurements computed by the accelerometer at UE can include:

- Linear acceleration of the UE

Two positioning modes are supported:

- Standalone: The UE performs accelerometer measurements and computes the location.

- UE-Assisted: The UE performs accelerometer measurements without assistance from the network and sends these measurements to the E-SMLC to compute the location of the UE. E-SMLC computes the position in the network using the accelerometer measurements possibly in conjunction with measurements from other sources.

### 8.x.2 Information to be transferred between E-UTRAN elements

This subclause defines the information (e.g., position, measurement data) that may be transferred between E-UTRAN elements.

#### 8.x.2.1	Information that may be transferred from the UE to E-SMLC

The information transferred from the UE to the E-SMLC consists of capability information and location measurements. The information that may be signalled from the UE to the E-SMLC is summarized in Table 8.x.2.1-1.

Table 8.x.2.1-1: Information that may be transferred from UE to the E-SMLC

| Information | UE-assisted | Standalone |
| --- | --- | --- |
| Accelerometer Location Information |  |  |
| Timestamp | Yes | No |
| Accelerometer sensor measurements | Yes | No |
| UE Location Information |  |  |
| UE position estimate with uncertainty shape | No | Yes |
| Position Time Stamp | No | Yes |
| Location Source (method(s) used to compute location) | No | Yes |

##### 8.x.2.1.1	Standalone mode

In Standalone mode, the UE reports the latitude, longitude and possibly altitude, together with an estimate of the location uncertainty, if available.  The UE should also report accelerometer method as a location source and possibly other location methods that have been used to calculate a fix.

##### 8.x.2.1.2	UE-assisted mode

In UE-assisted mode, the UE reports the linear acceleration as measured over time, by the accelerometer sensor. These measurements enable the E-SMLC to calculate the location of the UE, possibly using other measurements and data.

## 8.x.3 Accelerometer Positioning Procedure

### 8.x.3.1 Accelerometer sensor location transfer procedure

The purpose of this procedure is to enable E-SMLC to request for accelerometer sensor measurements or to enable the UE to provide the accelerometer measurements to the E-SMLC for position calculation.

#### 8.x.3.1.1 E-SMLC initiated location information transfer procedure

Figure 8.x.3.1.1-1 shows the Location Information Transfer operations when the procedure is initiated by the E-SMLC

UE

E-SMLC

1

.

LPP Request Location Information

2

.

LPP Provide Location Information

Figure 8.x.3.1.1-1 E-SMLC initiated location transfer procedure

(1) The E-SMLC sends a LPP Request Location Information message to the UE for invocation of accelerometer sensor positioning. This request includes positioning instructions such as the positioning mode (UE-assisted, Standalone), and quality of service parameters (accuracy, response time).

(2) The UE performs the requested measurements and sends an LPP Provide Location Information message to the E-SMLC before the Response Time provided in step (1) is elapsed. If the UE is not able to perform the requested measurements, or if the Response time elapsed before any of the requested measurements have been obtained, the UE returns an LPP message of type Provide Location Information indicating the reason for not provided location information. For the E-SMLC to compute the position of the UE to a higher accuracy, the UE may need to provide accelerometer measurements in batches measured over time.

#### 8.x.3.1.2 UE initiated location information delivery procedure

Figure 8.x.3.1.2-1 shows the Location Information Transfer operations when the procedure is initiated by the UE

UE

E-SMLC

1. LPP Provide Location Information

Figure 8.x.3.1.2-1 UE initiated location transfer procedure

(1) The UE sends an LPP Provide Location Information message to the E-SMLC. The Provide Location Information message may include UE accelerometer sensor measurements already available at the UE.

## 8.y	Gyroscope sensor positioning methods

### 8.y.1 General

In gyroscope positioning method, the angular velocity of the UE is estimated. This is accomplished by the gyroscope sensor. Using the measured angular velocity and measurements from other positioning methods, the position of the UE is computed.

The measurements computed by the gyroscope at UE can include:

- Angular velocity of the UE against a known/fixed axes.

Two positioning modes are supported:

- Standalone: The UE performs gyroscope measurements and computes the location.

- UE-Assisted: The UE performs gyroscope measurements without assistance from the network and sends these measurements to the E-SMLC to compute the location of the UE. E-SMLC computes the position in the network using the gyroscope measurements possibly in conjunction with measurements from other sources.

### 8.y.2 Information to be transferred between E-UTRAN elements

This subclause defines the information (e.g., position, measurement data) that may be transferred between E-UTRAN elements.

#### 8.y.2.1	Information that may be transferred from the UE to E-SMLC

The information transferred from the UE to the E-SMLC consists of capability information and location measurements. The information that may be signalled from the UE to the E-SMLC is summarized in Table 8.y.2.1-1.

Table 8.y.2.1-1: Information that may be transferred from UE to the E-SMLC

| Information | UE-assisted | Standalone |
| --- | --- | --- |
| Gyroscope Location Information |  |  |
| Timestamp | Yes | No |
| Gyroscope sensor measurements | Yes | No |
| UE Location Information |  |  |
| UE position estimate with uncertainty shape | No | Yes |
| Position Time Stamp | No | Yes |
| Location Source (method(s) used to compute location) | No | Yes |

##### 8.y.2.1.1	Standalone mode

In Standalone mode, the UE reports the latitude, longitude and possibly altitude, together with an estimate of the location uncertainty, if available.  The UE should also report gyroscope method as a location source and possibly other location methods that have been used to calculate a fix.

##### 8.y.2.1.2	UE-assisted mode

In UE-assisted mode, the UE reports the angular velocity as measured over time, by the gyroscope sensor. These measurements enable the E-SMLC to calculate the location of the UE, possibly using other measurements and data.

## 8.y.3 Gyroscope Positioning Procedure

### 8.y.3.1 Gyroscope sensor location transfer procedure

The purpose of this procedure is to enable E-SMLC to request for gyroscope sensor measurements or to enable the UE to provide the gyroscope measurements to the E-SMLC for position calculation.

#### 8.y.3.1.1 E-SMLC initiated location information transfer procedure

Figure 8.y.3.1.1-1 shows the Location Information Transfer operations when the procedure is initiated by the E-SMLC

UE

E-SMLC

1

.

LPP Request Location Information

2

.

LPP Provide Location Information

Figure 8.y.3.1.1-1 E-SMLC initiated location transfer procedure

(1) The E-SMLC sends a LPP Request Location Information message to the UE for invocation of gyroscope sensor positioning. This request includes positioning instructions such as the positioning mode (UE-assisted, Standalone), and quality of service parameters (accuracy, response time).

(2) The UE performs the requested measurements and sends an LPP Provide Location Information message to the E-SMLC before the Response Time provided in step (1) is elapsed. If the UE is not able to perform the requested measurements, or if the Response time elapsed before any of the requested measurements have been obtained, the UE returns an LPP message of type Provide Location Information indicating the reason for not providing the location information. For the E-SMLC to compute the position of the UE to a higher accuracy, the UE may need to provide gyroscope measurements in batches measured over time.

#### 8.y.3.1.2 UE initiated location information delivery procedure

Figure 8.y.3.1.2-1 shows the Location Information Transfer operations when the procedure is initiated by the UE

UE

E-SMLC

1. LPP Provide Location Information

Figure 8.y.3.1.2-1 UE initiated location transfer procedure

(1) The UE sends an LPP Provide Location Information message to the E-SMLC. The Provide Location Information message may include UE gyroscope sensor measurements already available at the UE.

## Annex B (informative): Use of LPP with SUPL

The design goal of LPP is to enable it to be used in user plane location solutions such as OMA SUPL ([17], [18]) and this informative annex shows how LPP can be used in SUPL 2.0.

# B.1	SUPL 2.0 Positioning Methods and Positioning Protocols

The following table shows how the 3GPP positioning protocols are supported in SUPL 2.0.

Table B.1-1: SUPL support of positioning methods

| Positioning Protocol: | RRLP (GSM/GPRS/WCDMA/ LTE/WLAN/WiMAX) | RRC (WCDMA) | LPP (LTE) |
| --- | --- | --- | --- |
| Positioning Method: |  |  |  |
| A-GPS (A-GANSS) SET Assisted |  |  |  |
| A-GPS (A-GANSS) SET Based |  |  |  |
| Autonomous GPS/GANSS |  |  |  |
| Enhanced Cell ID |  |  |  |
| Enhanced Observed Time Difference (E-OTD) | (GSM only) | NA | NA |
| Observed Time Difference of Arrival (OTDOA) NOTE 1 | NA |  |  |
| Barometric Pressure Sensors | NA |  |  |
| WLAN | NA |  |  |
| Bluetooth | NA |  |  |
| TBS NOTE 2 | NA |  |  |
| Accelerometer | NA | NA |  |
| Gyroscope | NA | NA |  |
| NOTE 1: This includes TBS positioning based on PRS signals, which is only supported in LPP (LTE). NOTE 2: TBS positioning based on MBS signals. |  |  |  |

# 4.	Summary and conclusion

Based on the discussion above we propose the following:

Proposal 1:  to introduce accelerometer and gyroscope sensor measurements that measure linear acceleration and angular velocity respectively.

# 5.	References

[1] 	RP-151624, Indoor Positioning Enhancements for UTRA and LTE, NextNav, Huawei, AT&T, TeleCommunication Systems, Intel, CMCC, China Telecom, China Unicom

[2]	RP-170813, UE Positioning Accuracy Enhancements for LTE, Nokia, Alcatel-Lucent Shanghai Bell