3GPP TSG-RAN WG2 Meeting #98	R2-1704998

Hangzhou, CHINA, 15th - 19th May 2017

Agenda item:	9.4.3

Source: 	Huawei, HiSilicon

Title: 	Interference mitigation for Drones

Document for:	Discussion

Introduction

In RAN#75 meeting, a new study item “Study on Enhanced Support for Aerial Vehicles” was approved [1].  The following were identified as part of the study item objectives:

…. - In terms of LTE enhancements, the study should consider the following aspects: - Interference mitigation solutions for improving system-level performance in both UL and DL [RAN1] - Solutions to detect whether UL signal from an air-borne UE increases interference in multiple neighbour cells and whether an air-borne UE incurs interference from multiple cells [RAN1, RAN2] …

In this contribution, we discuss the issue and requirement of interference detection and mitigation for drones based on LTE interference mechanisms.

Interference detection

  - Downlink interference detection

As in last RAN1#88bis meeting some agreements of simulation parameters have been made, we perform a preliminary simulation to demonstrate the downlink interference for drones. The UMa scenario is chosen for simulation, and the BS antenna configuration is 2Tx/2Rx. The heights of drones are 0m, 30m, 100m and 150m respectively. As shown in Fig.1 the a drone’s coupling loss is lower than terrestrial UE, but as a drone’s height increases the coupling loss becomes worse when a drone is flying above eNB(the height is 25m). And from Fig.2 it shows that downlink geometry of a drone is obviously worse than terrestrial UE, and the higher a drone is flying the worse geometry is. So it can demonstrate that downlink interference becomes quite strong as a drone is flying above eNB. Some simulation parameters are listed in Table 1, as the other simulation parameters of UMa AV scenario follow the RAN1’s agreements [2][3].

Table 1 Simulation Parameters

| Parameters | UMa AV |
| --- | --- |
| Cell layout | Hexagonal grid, 19 macro sites, 3 sectors per site (ISD = 500m); |
| BS antenna height | 25m |
| BS antenna configuration | 2Tx/2Rx cross polarized; 4Tx/4Rx cross polarized; 8Tx/8Rx cross polarized(only horizontal beamforming) 8Tx/8Rx cross polarized(horizontal and vertical beamforming) |
| BS antenna Down tilt | 12 degree |
| Height hUT (aerial) | Uniformly distributed between 1.5 m and 150m; |
| Channel estimation | Ideal channel estimation |

Fig.1 Coupling Loss                                       Fig.2 Downlink Geometry

In Current specification UE can detect downlink reference signals from serving cell and neighbor cells, and eNB can send a measurement configuration including measurement object and report configurations. Different measurement events are designed to provide a simplified measurement result. For example if the signal quality of neighbour cell is better than an absolute threshold, a measurement report is triggered by event A4, then the information on signal quality of serving cell and neighbor cells will be sent to eNB.

For Drones flying at a relatively high altitude they will “see” more eNBs than that on the ground, and at the same time they can receive more downlink reference signals from more eNBs which means more downlink interference. If the threshold of event A4 is the same as that of ground UE, a large number of neighbour cells will be included in measurement report. So it is necessary to consider optimization of measurement report to reduce frequent reporing. So we have the following observation and corresponding proposal:

Observation 1: drone UE meeds report measurement results frequently due to increased number of neighbour cells.

Proposal 1: consider optimization of measurement report in case of increased number of strong neighbour cells.

Uplink interference detection

In LTE system it is usual to handle intra-frequency interference. For downlink transmission UEs suffer from interference of neighbour cells, and for uplink transmission the signals from UEs in serving cells and UEs in neighbour cells arrive at eNB simultaneously which leads to uplink interference. It will become even worse for UE at the cell edge. In that case the received signal power from target UE and neighbour UEs may be about the same in eNB side. In LTE several interference cancellation methods have been used. However, how to perform uplink interference detection is not clear from the standpoint of specification, it mainly depends on eNB implementation, except some X2 signalling for UL ICIC. If drones as a specific UE type are supported by LTE system, it is necessary to control its interference due to its large sight. In last meeting, RAN1 has discussed if a specific uplink reference signal should be used. However, from RAN2 perspective, if neighbour eNBs can get idea of scheduling information or uplink reference signal configuration, e.g. sounding reference signal, of a given drone UE in its serving eNB, it is feasible of neighbour eNBs to detect uplink signals from drones, as long as the receiving timing of the neighbour eNB can synchronizes with the receiving timing of the serving eNB.

Proposal 2：it is proposed the neighbour eNBs to detect uplink signals from drones, via scheduling information or uplink reference signal configuration, e.g. sounding reference signal, of a given drone UE from its serving eNB.

Interference Mitigation

  - Downlink Interference Mitigation

ICIC (Inter-cell Interference Coordination) is introduced for LTE in R8 to decrease interference between neighbour macro base stations. This is realized by lowering the power of a part of the subcarriers of a given cell in the frequency domain which then can only be used close to the base station. These subcarriers used in other neighbour cells can avoid be suffered from interfere with the same subcarriers from the given cell. ICIC can be classified in several ways. Depending on the period of resource scheduling, ICIC can be static or semi-static. Semi-static soft frequency reuse can maximize spectrum efficiency in a less complex system. As shown in Fig.3, different subcarriers are allocated to the edge of neighbor cells, and the subcarriers are reused in the cell centre. The power is controlled to prevent cell edges and neighbor cells from being affected.

Fig. 3 soft frequency reuse for ICIC

When heterogeneous network (HetNet) is introduced to LTE, where macro cells are deployed with pico cells inside their coverage area and a macro cell has much higher Tx power than a small cell, the small cell's control channel is inevitably interfered with by the macro cell's, making ICIC applied to the data channel ineffective. Hence, enhanced ICIC (eICIC) is introduced to avoid the victim cell be suffered from the high inference from the aggressive cell by allocating  the Almost Blank Subframes (ABSs) in the time domain in the aggressive cell as shown in Fig.4. When several small cells are used in the coverage area of a single macro cell overall system capacity is increased as each small cell can use the ABS subframes without interference from the other small cells.

Fig.4 Almost Blank Subframe for eICIC

For drone UEs they can fly at a relatively high altitude than normal UEs, so the difference for interference mitigation may be more neighbour cells that should involve during this process. After measurement report is received by eNB, the eNB will know which neighbour cells produce high interference, and then interference mitigation mechanism similar to ICIC/eICIC can be considered.

As ICIC and eICIC are only for UEs in cell edge and usually operate in static mode or semi-static mode, they may not apply well for drone UEs. Drone UEs flying at a high altitude can suffer neighbour interference even in centre of the cell. And the number of drone UEs may change rapidly but the ABS method cannot adapt accordingly in time to reduce resource waste. So a more flexible method should be considered.

Proposal 3: flexible interference mitigation mechanism can be considered for drone UEs.

Uplink Interference Mitigation

When neighbor eNBs can be aware of the uplink resource allocation information of drone UEs, firstly they can detect which drones cause the uplink interference. Then the similar solution with regard to downlink interference mitigation can be considered, i.e. if interference cancellation is effective enough neighbor eNB can cancel this interference, but if it is not feasible neighbor eNBs have to leave the corresponding resource unused.

Proposal 4: the method of interference avoidance and interference cancellation can still be used for uplink interference mitigation with the provision of uplink resource allocation information of drones.

Conclusion

By analysing the interference detection and mitigation for drones we have the following observations:

Observation 1: drone UE need report measurement results frequently due to increased number of neighbour cells.

And it is proposed:

Proposal 1: consider optimization of measurement report in case of increased number of strong neighbour cells.

Proposal 2：it is proposed the neighbour eNBs to detect uplink signals from drones, via scheduling information or uplink reference signal configuration, e.g. sounding reference signal, of a given drone UE from its serving eNB.

Proposal 3: flexible interference mitigation mechanism can be considered for drone UEs.

Proposal 4: the method of interference avoidance and interference cancellation can still be used for uplink interference mitigation with the provision of uplink resource allocation information of drones.

Reference

- RP-170779, “New SID on Enhanced Support for Aerial Vehicles”, NTT DOCOMO INC, Ericsson, 3GPP TSG RAN Meeting #75, Dubrovnik, Croatia, March 6-9, 2017.

- R1-1706475 WF on evaluation assumptions for aerials study  Ericsson, NTT DOCOMO, Intel

- R1-1706469 WF on remaining evaluation assumptions for aerials study Ericsson, Qualcomm, NTT DOCOMO, Samsung, Intel, Softbank, ZTE, Nokia

Annex

Table 2 Evaluation assumptions for aerials study

| Parameters |  | UMi AV | UMa AV | RMa AV |
| --- | --- | --- | --- | --- |
| Cell layout |  | Hexagonal grid, 19 micro sites, 3 sectors per site (ISD = 200m); FFS: if 37 micro sites, 3 sectors per site are needed | Hexagonal grid, 19 macro sites, 3 sectors per site (ISD = 500m); FFS: if 37 macro sites, 3 sectors per site are needed | Hexagonal grid, 19 macro sites, 3 sectors per site (ISD = 1732m; optionally ISD = 5000m); FFS: if 37 macro sites, 3 sectors per site are needed |
| BS antenna height |  | 10m | 25m | 35m |
| Carrier frequency |  | 2GHz | 2 GHz | 700 MHz; optionally 800 MHz |
| Total BS Tx power |  | 41/44 dBm for 10/20MHz | 46/49 dBm for 10/20MHz | 46/49 dBm for 10/20MHz |
| BS antenna configuration |  | 2Tx/2Rx cross polarized; Optionally (M, N, P) = (8, 4, 2) according to TR36.873 with 32 Tx ports and 32 Rx; other antenna configurations are not precluded | 2Tx/2Rx cross polarized; Optionally 8Tx/8Rx cross polarized; Optionally (M, N, P) = (8, 4, 2) according to TR36.873 with 32 Tx ports and 32 Rx; other antenna configurations are not precluded | 2Tx/2Rx cross polarized; other antenna configurations are not precluded |
| BS antenna pattern |  | FFS | FFS | FFS |
| BS antenna element pattern |  | For FD-MIMO, according to TR38.901 | For FD-MIMO, according to TR38.901 | For FD-MIMO, according to TR38.901 |
| BS antenna element gain + connector loss |  | 8dBi | 8dBi | 8dBi |
| UT location | Outdoor terrestrial/indoor terrestrial/aerial | Outdoor terrestrial and indoor terrestrial (same as UMi/UMa in TR38.901), and aerial UTs |  | Outdoor terrestrial and indoor terrestrial (same as RMa in TR38.901), and aerial UTs |
|  | LOS/NLOS (terrestrial) | LOS and NLOS |  |  |
|  | LOS/NLOS (aerial) | FFS | FFS | FFS |
|  | Height hUT (terrestrial) | For non FD-MIMO: FFS; For FD-MIMO: Same as UMi in TR 38.901 | For non FD-MIMO: FFS; For FD-MIMO: Same as UMa in TR38.901 | Same as RMa in TR38.901 |
|  | Height hUT (aerial) | Uniformly distributed between 1.5 m and [150]m; Optionally: fixed height values in the range between 1.5m and [150]m | Uniformly distributed between 1.5 m and [150]m; Optionally: fixed height values in the range between 1.5m and [150]m | Uniformly distributed between 1.5 m and [150]m; Optionally: fixed height values in the range between 1.5m and [150]m |

- Note 1: The above scenarios are for evaluation purposes.

- Note 2: The base station height will be revisited in case of evaluations on positioning enhancements.

| Parameters | UMi AV | UMa AV | RMa AV |
| --- | --- | --- | --- |
| Indoor terrestrial UT ratio defined as indoor tesstrial UTsoutdoor tesstrial UTs+indoor terrestrial UTs | 80% | 80% | 50% |
| Outdoor terrestrial UT ratio defined as outdoor terrestrial UTsoutdoor terrestrial UTs+indoor terrestrial UTs | 20% | 20% | 50% |
| Aerial UT ratio defined as aerial UTsoutdoor terrestrial UTs+indoor terrestrial UTs | FFS | FFS | FFS |
| Number of indoor terrestrial UTs + outdoor terrestrial UTs + aerial UTs | [15] per sector | [15] per sector | [15] per sector |
| Traffic model | FFS | FFS | FFS |
| UT mobility (horizontal plane only) | FFS | FFS | FFS |
| Min. BS – Terrestrial UT distance (2D) | 10m | 35m | 35m |
| Min. BS – Aerial UT distance | FFS | FFS | FFS |
| UT distribution (horizontal) – for outdoor terrestrial/indoor terrestrial/aerial | Uniform | Uniform | Uniform |
| Channel models for terrestrial UT | According to TR 38.901 or TR 36.873 | According to TR 38.901 | According to TR 38.901 |
| Channel models for aerial UT | FFS | FFS | FFS |
| Terrestrial UT Tx Power | 23dBm | 23dBm | 23dBm |
| Aerial UT Tx Power | 23dBm | 23dBm | 23dBm |
| Power control | Baseline: open loop power control. FFS open loop power control parameters | Baseline: open loop power control. FFS open loop power control parameters | Baseline: open loop power control. FFS open loop power control parameters |
| Tterrestrial or aerial UT antenna element pattern | Omnidirectional/isotropic | Omnidirectional/isotropic | Omnidirectional/isotropic |
| Terrestrial or aerial UT antenna element gain | 0dBi | 0dBi | 0dBi |
| Number of terrestrial or aerial UT antennas | TX: 1 or 2 cross polarized; RX: 2 cross polarized other antenna configurations are not precluded | TX: 1 or 2 cross polarized; RX: 2 cross polarized other antenna configurations are not precluded | TX: 1 or 2 cross polarized; RX: 2 cross polarized other antenna configurations are not precluded |
| BS receiver noise figure | 5dB | 5dB | 5dB |
| Terrestrial/aerial UT receiver noise figure | 9dB | 9dB | 9dB |
| Baseline receiver for terrestrial/aerial UT | MMSE-IRC; non-ideal interference estimation |  |  |
| Feedback assumption | Non-ideal CSI feedback and non-ideal CSI-RS channel estimation |  |  |
| Channel estimation | Non-ideal. Optionally, ideal channel estimation for demodulation purpose |  |  |