3GPP TSG-RAN2 Meeting #98	R2-1705225

# Hangzhou, China, 15th May 2017 - 19th May 2017

| CR-Form-v11.2.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.355 | CR | 0180 | rev | - | Current version: | 14.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification for prsOccGroupLen IE in OTDOA PRS-Info for FeMTC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_feMTC-Core |  |  |  |  |  | Date: |  |  | 2017-05-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In the field definition of prsOccGroupLen in PRS-Info, it says “the product of the PRS periodicity T_PRS from the prs-ConfigurationIndex and the PRS occasion group length cannot exceed 1280”, we think it’s not clear for the following aspects: - Which parameter is the product? And which two parameters are the multiplying factors? - Which parameter has the restriction that its value cannot exceed 1280? RAN1 has the following related agreements for OTDOA for FeMTC: - Supported legacy PRS periodicities are {160, 320, 640, 1280} subframes that are jointly signaled with starting subframe offsets in PRS configuration indices – No change w.r.t. legacy LTE PRS indices - To support more frequent PRS transmission in time, add the following parameters to support multiple PRS occasions in a legacy PRS period – PRS occasion interval in a PRS period: {10, 20, 40, 80} subframes Based on the first agreement, we think the value of the PRS periodicity T_PRS from the prs-ConfigurationIndex has a limited range of values and should not exceed 1280 subframes. Based on the second agreement, we understand multiple PRS occasions in a PRS period can be supported. These multiple PRS occasions can be seen as a PRS occasion group. The length of PRS occasion group is the number of multiple PRS occasions. Then the PRS periodicity T_PRS can be the product of PRS occasion group length and PRS occasion interval. However, we think the field definition of prsOccGroupLen in PRS-Info doesn’t clearly reflct the RAN1 agreements. |  |  |  |  |  |  |  |  |
| Summary of change: |  | To clarify that when prsOccGroupLen is configured, such RAN1 restrictions should be considered: - The T_PRS which is the product of prsOccGroupLen and PRS occasion interval should be within the value set of {160, 320, 640, 1280} subframes. - The values that can be used for PRS occasion interval are 10, 20, 40 or 80 subframes. Impact analysis Impacted functionality: The changes only impact the corrected functionality. Inter-operability: If the network is implemented according to the CR and the UE is not, there is no inter-operability issue. If the UE is implemented according to the CR but the network is not, the UE may fail to get exact PRS-Info and cannot execute the exact positioning measurement. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The UE will fail to get exact PRS-Info and cannot execute the exact positioning measurement. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.5.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<Start of change>

–	PRS-Info

The IE PRS-Info provides the information related to the configuration of PRS in a cell.

-- ASN1START

# PRS-Info ::= SEQUENCE {

prs-Bandwidth			ENUMERATED { n6, n15, n25, n50, n75, n100, ... },

prs-ConfigurationIndex	INTEGER (0..4095),

numDL-Frames			ENUMERATED {sf-1, sf-2, sf-4, sf-6, ..., sf-add-v14xy},

...,

prs-MutingInfo-r9		CHOICE {

po2-r9					BIT STRING (SIZE(2)),

po4-r9					BIT STRING (SIZE(4)),

po8-r9					BIT STRING (SIZE(8)),

po16-r9					BIT STRING (SIZE(16)),

...,

po32-v14xy				BIT STRING (SIZE(32)),

po64-v14xy				BIT STRING (SIZE(64)),

po128-v14xy				BIT STRING (SIZE(128)),

po256-v14xy				BIT STRING (SIZE(256)),

po512-v14xy				BIT STRING (SIZE(512)),

po1024-v14xy			BIT STRING (SIZE(1024))

}														OPTIONAL,				-- Need OP

[[ prsID-r14			INTEGER (0..4095)				OPTIONAL,				-- Need ON

add-numDL-Frames-r14	INTEGER (1..160)				OPTIONAL,				-- Cond sf-add

prsOccGroupLen-r14	ENUMERATED {g2, g4, g8, g16, g32, g64, g128,... }

OPTIONAL, 			-- Cond Occ-Grp

prsHoppingInfo-r14	CHOICE {

nb2-r14					INTEGER (0.. maxAvailNarrowBands-Minus1-r14),

nb4-r14					SEQUENCE (SIZE (3))

OF INTEGER (1.. maxAvailNarrowBands-Minus1-r14)

}													OPTIONAL				-- Cond PRS-FH

]]

}

maxAvailNarrowBands-Minus1-r14		INTEGER ::= 15	-- Maximum number of narrowbands minus 1

-- ASN1STOP

| Conditional presence | Explanation |
| --- | --- |
| sf-add | The field is mandatory present if the numDL-Frames field has the value 'sf-add'; otherwise it is not present. |
| Occ-Grp | The field is mandatory present if a PRS occasion group is configured; otherwise it is not present. |
| PRS-FH | The field is mandatory present if frequency hopping is used for PRS; otherwise it is not present. |

PRS-Info field descriptions

prs-Bandwidth This field specifies the bandwidth that is used to configure the positioning reference signals on. Enumerated values are specified in number of resource blocks (n6 corresponds to 6 resource blocks, n15 to 15 resource blocks and so on) and define 1.4, 3, 5, 10, 15 and 20 MHz bandwidth.

prs-ConfigurationIndex This field specfies the positioning reference signals configuration index IPRS as defined in [16].

numDL-Frames This field specifies the number of consecutive downlink subframes NPRS with positioning reference signals, as defined in [16]. Enumerated values define 1, 2, 4, or 6 consecutive downlink subframes. The value sf-add indicates that NPRS is provided in the field add-numDL-Frames.

prs-MutingInfo This field specifies the PRS muting configuration of the cell. The PRS muting configuration is defined by a periodic PRS muting sequence with periodicity TREP where TREP, counted in the number of PRS occasion groups [18], can be 2, 4, 8, 16, 32, 64, 128, 256, 512, or 1024 which is also the length of the selected bit string that represents this PRS muting sequence. If a bit in the PRS muting sequence is set to "0", then the PRS is muted in all the PRS occasions in the corresponding PRS occasion group. A PRS occasion group comprises one or more PRS occasions as indicated by prsOccGroupLen, each of NPRS downlink positioning subframes as defined in [16]. The first bit of the PRS muting sequence corresponds to the first PRS occasion group that starts after the beginning of the assistance data reference cell SFN=0. The sequence is valid for all subframes after the target device has received the prs-MutingInfo. If this field is not present the target device may assume that the PRS muting is not in use for the cell. When the SFN of the assistance data reference cell is not known to the UE and prs-MutingInfo is provided for a cell in the OTDOA-NeighbourCellInfoList IE, the UE may assume no PRS is transmitted by that cell. When the UE receives a TREP-bit muting pattern together with a PRS periodicity TPRS for the same cell which exceeds 10240 subframes (i.e., TREP × TPRS > 10240 subframes), the UE shall assume an n-bit muting pattern based on the first n-bits, where n = 10240/TPRS.

prsID This field specifies the PRS-ID as defined in [16].

add-numDL-Frames This field specifies the number of consecutive downlink subframes NPRS with positioning reference signals, as defined in [16]. Integer values define 1, 2, 3, …, 160 consecutive downlink subframes.

prsOccGroupLen This field specifies the length of the PRS occasion group. Enumerated values define 2, 4, 8, 16, 32, 64 or 128. The product of the PRS occasion group length and PRS occasion interval, that is, the PRS periodicity T_PRS from the prs-ConfigurationIndex should be within the value set of {160, 320, 640,1280} subframes. The values that can be used for PRS occasion interval are 10, 20, 40 or 80 subframes. If omitted, the PRS occasion group length is 1.

prsHoppingInfo This field specifies the PRS frequency hopping configuration [16]. The choice nb2 indicates hopping between 2 narrowbands; the choice nb4 indicates hopping between 4 narrowbands. The first PRS positioning occasion of the first PRS occasion group that starts after the beginning of SFN=0 of the assistance data reference cell is located at the centre of the system bandwidth. The frequency band of each subsequent PRS occasion is indicated by nb2 or nb4, respectively, which defines the narrowband index as specified in TS 36.211 [16]. If this field is absent, no PRS frequency hopping is used.

<End of change>