3GPP TSG-RAN WG2 Meeting #98	R2-1705330

Hangzhou, China, 15th – 19th May 2017

Agenda item:	10.2.1

Source:	Samsung

Title:	TP for capturing RRM Model in Stage-2

Document for:	Discussion

# 1	Introduction

At RAN2#97bis following agreements were made w.r.t RRM model [1]:

Agreements

1	The RRC configured beam consolidation/selection of beam quality of gNB detected beams to derive a cell quality shall be performed after the L1 filter.

2	The L1 filter filters signal quality corresponding to gNB beams detected by the UE

3: The measurement model (applicable for both multi beam and single beam case) in NR shall consist of the following:

a-	L1 filtering of beam measurements

FFS Whether there is any additional specified filtering of the beam measurements

b-	Derivation of cell quality from one or more gNB beam quality

c-	L3 filter (RRC configured) of cell quality

d-	Evaluation reporting criteria (RRC configured)

Even though there are some FFS issues it makes sense to capture the RRM model in Stage-2. We provide the following TP to be captured.

# TP to stage-2 for the RRM model

## X.X	Measurement Model

Figure X.X-1: Measurement model

-	A: measurements (beam specific samples) internal to the physical layer.

-	Layer 1 filtering: internal layer 1 filtering of the inputs measured at point A. Exact filtering is implementation dependent. How the measurements are actually executed in the physical layer by an implementation (inputs A and Layer 1 filtering) in not constrained by the standard.

-	A1: Measurements (i.e. beam specific measurements) reported by layer 1 to layer 3 after layer 1 filtering.

-	Beam Consolidation/Selection: Beam specific measurements are consolidated to derive cell quality if N > 1, else when N = 1 the best beam measurement is selected to derive cell quality. The behaviour of the Beam consolidation/selection is standardised and the configuration of this module is provided by RRC signalling. Reporting period at B equals one measurement period at A1.

-	B: A measurement (i.e. cell quality) derived from beam-specific measurements reported to layer 3 after beam consolidation/selection.

-	Layer 3 filtering: Filtering performed on the measurements provided at point B. The behaviour of the Layer 3 filters are standardised and the configuration of the layer 3 filters is provided by RRC signalling. Filtering reporting period at C equals one measurement period at B.

-	C: A measurement after processing in the layer 3 filter. The reporting rate is identical to the reporting rate at point B. This measurement is used as input for one or more evaluation of reporting criteria.

-	Evaluation of reporting criteria: This checks whether actual measurement reporting is necessary at point D. The evaluation can be based on more than one flow of measurements at reference point C e.g. to compare between different measurements. This is illustrated by input C and C1. The UE shall evaluate the reporting criteria at least every time a new measurement result is reported at point C, C1. The reporting criteria are standardised and the configuration is provided by RRC signalling (UE measurements).

-	D: Measurement report information (message) sent on the radio interface.

Layer 1 filtering will introduce a certain level of measurement averaging. How and when the UE exactly performs the required measurements will be implementation specific to the point that the output at B fulfils the performance requirements set in [RAN4 Ref]. Layer 3 filtering and parameters used is specified in [38.331] and does not introduce any delay in the sample availability between B and C. Measurement at point C, C1 is the input used in the event evaluation.

# References

[1] 	R2-17xxxxx, RAN2 Chairman Notes, RAN2#97bis Spokane meeting.