3GPP TSG-RAN2 Meeting #98	R2-1705788

# Hangzhou, China, 15th May 2017 - 19th May 2017

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.331 | CR | 2935 | rev | - | Current version: | 13.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to inter-cell measurement parameter for NB-IoT – option B |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CMCC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R2 |  |  |  |  |  |  |  |  |  |
| Work item code: | NB_IOTenh-Core |  |  |  |  |  | Date: |  |  | 2017-05-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-13 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-14 (Release 15) Rel-14 (Release 16) |  |
| Reason for change: |  | Reference to R2-1702929, a ‘too late reselection’ problem was detected during NB-IoT field trial due to the restriction of existing ReselectionThreshold IE. In order to solve this problem, ReselectionThreshold IE is to be extended from (0..31)*2 to (0..63)*2. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Extend the value range of ReselectionThreshold for NB-IoT from (0…31)*2 to (0...63)*2. Impact Analysis Impacted functionality: The changes only impact the corrected functionality. Inter-operability: If the NW is implemented according to the CR but the UE is not or vice versa, the NB-IoT UE can still leverage the legacy value of ReselectionThreshold to perform cell reselection. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | In some scenarios, it’s hard for the NB-IoT UE to trigger inter-cell measurements. As a consequence, high interference from neighbour cell, low SINR and losing of paging message may occur. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.7.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section>

–	SystemInformationBlockType3-NB

The IE SystemInformationBlockType3-NB contains cell re-selection information common for intra-frequency, and inter-frequency cell re-selection as well as intra-frequency cell re-selection information other than neighbouring cell related.

SystemInformationBlockType3-NB information element

-- ASN1START

SystemInformationBlockType3-NB-r13 ::=	SEQUENCE {

cellReselectionInfoCommon-r13			SEQUENCE {

q-Hyst-r13								ENUMERATED {

dB0, dB1, dB2, dB3, dB4, dB5, dB6, dB8, dB10,

dB12, dB14, dB16, dB18, dB20, dB22, dB24

}

},

cellReselectionServingFreqInfo-r13		SEQUENCE {

s-NonIntraSearch-r13					ReselectionThreshold

},

intraFreqCellReselectionInfo-r13		SEQUENCE {

q-RxLevMin-r13							Q-RxLevMin,

q-QualMin-r13							Q-QualMin-r9			OPTIONAL,	-- Need OP

p-Max-r13								P-Max					OPTIONAL,	-- Need OP

s-IntraSearchP-r13						ReselectionThreshold,

t-Reselection-r13						T-Reselection-NB-r13

},

freqBandInfo-r13						NS-PmaxList-NB-r13				OPTIONAL,	-- Need OR

multiBandInfoList-r13					SEQUENCE (SIZE (1..maxMultiBands)) OF

NS-PmaxList-NB-r13			OPTIONAL,	-- Need OR

lateNonCriticalExtension					OCTET STRING				OPTIONAL,

...,

[[	intraFreqCellReselectionInfo-v1350		IntraFreqCellReselectionInfo-NB-v1350 OPTIONAL 	-- Cond Qrxlevmin

]],

[[ cellReselectionServingFreqInfo-v13xy	cellReselectionServingFreqInfo-NB-v13xy OPTIONAL, –- Need OR

intraFreqCellReselectionInfo-v13xy	intraFreqCellReselectionInfo-NB-v13xy OPTIONAL –- Need OR

]]

}

IntraFreqCellReselectionInfo-NB-v1350 ::=	SEQUENCE {

delta-RxLevMin-v1350						INTEGER (-8..-1)

}

cellReselectionServingFreqInfo-NB-v13xy	::=	SEQUENCE {

s-NonIntraSearchExtend-r13					ReselectionThreshold,	OPTIONAL	-- Need OP

}

intraFreqCellReselectionInfo-NB-v13xy	::= SEQUENCE {

s-IntraSearchPExtend-r13					ReselectionThreshold,	OPTIONAL		-- Need OP

}

-- ASN1STOP

SystemInformationBlockType3-NB field descriptions

multiBandInfoList A list of additionalPmax and additionalSpectrumEmission values as defined in TS 36.101 [42, table 6.2.4-1] applicable for the intra-frequency neighbouring NB-IoT cells if the UE selects the frequency band from freqBandIndicator in SystemInformationBlockType1-NB.

p-Max Value applicable for the intra-frequency neighbouring E-UTRA cells. If absent the UE applies the maximum power according to the UE capability.

q-Hyst Parameter Qhyst in TS 36.304 [4], Value in dB. Value dB1 corresponds to 1 dB, dB2 corresponds to 2 dB and so on.

q-QualMin Parameter “Qqualmin” in TS 36.304 [4], applicable for intra-frequency neighbour cells. If the field is not present, the UE applies the (default) value of negative infinity for Qqualmin.

q-RxLevMin, delta-RxLevMin Parameter “Qrxlevmin” in TS 36.304 [4], applicable for intra-frequency neighbour cells.If delta-RxLevMin is not included, actual value Qrxlevmin = q-RxLevMin * 2 [dBm].If delta-RxLevMin is included, actual value Qrxlevmin = (q-RxLevMin + delta-RxLevMin) * 2 [dBm].

s-IntraSearchP, s-IntraSearchPExtend Parameter “SIntraSearchP” in TS 36.304 [4]. If the field s-IntraSearchPExtend is present, actual value SIntraSearchP = actual value s-IntraSearchPExtend + 64[dB] instead.

s-NonIntraSearch, s-NonIntraSearchExtend Parameter “SnonIntraSearchP” in TS 36.304 [4]. If the field s-NonIntraSearchExtend is present, actual value SnonIntraSearchP = actual value s-NonIntraSearchExtend + 64[dB] instead.

t-Reselection Parameter “TreselectionNB-IoT_Intra” in TS 36.304 [4].

| Conditional presence | Explanation |
| --- | --- |
| Qrxlevmin | This field is optionally present, Need OR, if q-RxLevMin is set to the minimum value. Otherwise the field is not present. |

<End of modified section>