3GPP TSG-RAN WG2 Meeting #98	R2-1704329

Hangzhou, China, 15 - 19 May 2017

Agenda item:	9.1.2.3

Source:	Nokia, Alcatel-Lucent Shanghai Bell

Title:	Data lossless path switch

WID/SID:	FS_feD2D_IoT_relay_wearable - Release 15

Document for:	Discussion and Decision

# 1	Introduction

Service continuity requirement is captured in TR 36.746 [1]:

#### 4.2.1.8	Requirement 8 – Service continuity

The relay solution shall support service continuity when the evolved ProSe Remote UE selects or switches its path between Uu and PC5. Following service continuity scenarios are considered in this study with a lower priority compared to the path switch between Uu and PC5:

-	service continuity among different evolved ProSe UE-to-Network Relay UEs (i.e. reselection of evolved ProSe UE-to-Network Relay UE) for PC5 and non-3GPP access;

-	service continuity for the switching scenario between direct 3GPP connection and indirect non-3GPP connection.

The service continuity of switching from PC5 to non-3GPP access (and vice versa) within the same evolved ProSe UE-to-Network Relay UE or switching from non-3GPP access to another non-3GPP access is not studied as part of this study. The service continuity of the evolved ProSe Remote UE and the evolved ProSe UE-to-Network Relay UE moving together from one cell to another cell is supported in this study.

In this contribution, we focus on the scenario where Remote UE switches its path from PC5 to Uu although the identified issue concerns also service continuity when switching between two different relays.

# 2	Discussion

Path switch from PC5 to Uu may be triggered by several factors, e.g. eNB may decide that QoS is cannot be met properly via relay or radio conditions on PC5 interface deteriorate (e.g. due to UEs moving away from each other or due to congestion). In this situation, a decision about the path switch would be made by the eNB. Path switch may happen without eNB change (Scenario 4 from [1]) or with eNB change (Scenario 5 in [1]). The problem we are describing here is common for both scenarios.

According to the agreed protocol stack for User Plane as captured in the TR [1], the data packet needs to traverse two separate RLC connections when it is being relayed via Relay UE, which causes two issues when path is switched from PC5 to Uu:

- eNB does not know which downlink PDCP packets have not been successfully received by the Remote UE. Using current RLC STATUS report, the eNB only knows whether a PDCP packet is successfully received by the Relay UE.

- eNB may not have the DL PDCP packets that have not been successfully received by the Remote UE as they may have been removed from eNB’s transmission buffer if the eNB receives acknowledgement from the Relay UE, even in case the PDCP packet has not yet been received by the UE.

The problem is presented in Figure 1.

Figure 1: Problem with lost PDCP packets during PC5 to Uu path switch

As mentioned previously same problem may occur in scenario 5, where additionally data forwarding of PDCP PDUs from source eNB to target eNB should be performed, but eNB may not be able to forward all relevant packets if they are already discarded from its buffer. This leads to data loss during path switch, which should be avoided.

Observation 1: If current procedures are kept path switch from PC5 to Uu interface may lead to lost PDCP PDUs.

Several solutions to this problem may be envisaged:

- End to end PDCP packet acknowledgment from remote UE to eNB.

- Delaying the acknowledgment from Relay UE to eNB until all the relevant RLC PDUs are acknowledged by the Remote UE.

- Sending back RLC PDUs unacknowledged by the Remote UE from Relay UE to eNB upon path switch.

Proposal 1: RAN2 should study how to ensure data lossless PC5 to Uu path switch.

Proposal 2: Potential solutions for ensuring data lossless PC5 to Uu path switch listed above should be captured in the TR.

RAN2 has decided to support service continuity (although with lower priority) also for relay reselection case. Therefore, it should be noted that the same issue is about to occur in case relay is reselected during active connection and a common approach for both scnearios should be sought.

Observation 2: The same data loss problem will take place also in case of relay reselection during an active data connection.

# 3	Summary

In this paper, we discuss an issue of potential data loss during path switch from PC5 to Uu interface for Remote UE. The identified issue is explained and it is proposed to study some potential solutions, e.g.

- End to end PDCP packet acknowledgment from remote UE to eNB.

- Delaying the acknowledgment from Relay UE to eNB until all the relevant RLC PDUs are acknowledged by the Remote UE.

- Sending back RLC PDUs unacknowledged by the Remote UE from Relay UE to eNB upon path switch.

It is therefore proposed to agree on the following observation and proposals:

Observation 1: If current procedures are kept path switch from PC5 to Uu interface may lead to lost PDCP PDUs.

Proposal 1: RAN2 should study how to ensure data lossless PC5 to Uu path switch.

Proposal 2: Potential solutions for ensuring data lossless PC5 to Uu path switch listed above should be captured in the TR.

Observation 2: The same data loss problem will take place also in case of relay reselection during an active data connection.

# References

- R2-1703973, 3GPP TR 36.746 V0.5.0 (2017-04), Study on further enhancements to LTE Device to Device (D2D), User Equipment (UE) to network relays for Internet of Things (IoT) and wearables