3GPP TSG-RAN WG2 #98	Tdoc R2-1704368

Hangzhou, P.R. of China, 15th – 19th May 2017

Agenda Item:	10.3

Source:	Ericsson

Title:	On the addition of enhanced ECN support in NR

Document for:	Discussion, Decision

# Introduction

ECN has been specified in TS36.300 since a few years back, the specification text in section 11.6 is suggested to be included also in TS38.300.

This paper suggests to enhance ECN support in NR with the addition of ECN bits in the headers of protocols that encapsulate IP packets or similar and that are accessible on the RLC layer. This makes it possible to ECN mark RLC-PDUs when congestion is detected on the RLC layer. The paper explains why such a modification is beneficial for the performance of NR.

# Discussion

## Motivation

ECN (Explicit Congestion Notification) is a technology that has been available for quite a while, its present semantics is specified in RFC3168 [2]. The main feature with ECN is that IP packets are marked instead of discarded by congested nodes. The obvious benefit is a lower (almost zero) packet loss, which will significantly reduce the amount of retransmission.

ECN is transport protocol agnostic. While the ECN marking is done on the ECN bits in the IP header, the actual feedback of the ECN marking in performed by the transport protocols, thus no dedicated reverse in-band or out-of-band signaling channel on the IP layer or below is needed to manage the feedback for the ECN information to the transport protocol sender. ECN support is specified for TCP, RTP/UDP and SCTP, ECN support for QUIC is also work in progress.

Transport protocol reaction to ECN is to reduce the sending rate, the default action specified in [2] is to reduce the sending rate in a manner like the action if packet loss occurs.

L4S (Low Loss Low Latency Scalable throughput) is a novel technology that can give very low latency even at high bitrates and network load levels, the concept is briefly presented in [4]. Services that can exploit L4S include for instance 360 Video and Virtual Reality. L4S exploits ECN marking in a different way than specified in [2]. This to achieve high link utilization with low queue latency, the details of this is currently standardized in IETF.

Because the ECN bits are defined in the IP header, no deep packet inspection is needed, therefore ECN can also work over IPsec tunnels as copying of the ECN bits between inner and outer IP headers is defined in [1].

ECN has for a long while had a relatively slow pickup, mainly because it has been difficult to claim large benefits with ECN, for instance large file transfers do benefit very little from ECN support. The trend is however pointing at a larger use of internet services that are highly interactive. The user experience of interactive services is highly depending on a timely delivery of objects. In many cases, a lost packet in the transport layer leads to extra delay on the application layer due to retransmission. Today Apple enables ECN as default for their services and devices [5].

Novel technology like L4S requires efficient and prompt ECN marking of packets for its function. The reasons are twofold

- Prompt ECN marking is required to quickly react to growing queues.

- L4S relies on measuring the amount of ECN marked packets, the transport protocol sending rate is finely adjusted based on this information. This property is exploited to make it possible for the transport protocols to quickly increase the sending rate, to avoid problems with sending rate overshoot and increased delay, it is thus instrumental that the measured amount of ECN marked packet is as fresh as possible.

Current 3GPP technology ECN support is limited to ECN marking in the IP header, which in practice limits ECN marking to the PDCP layer, prior to ciphering and possible compression. If congestion due to e.g. queue buildup is detected on the RLC layer, due to queue buildup, then a signal needs to be sent back to the PDCP layer to ECN mark an IP packet before it is encapsulated in a PDCP-PDU. The problem is that this gives a tail marking property to the ECN marking, which makes it hard to fulfil the requirement from L4S of prompt ECN marking.

To summarize, the issues with existing ECN marking capability in 3GPP access are:

- If congestion is detected e.g. on the RLC layer:

  - It becomes necessary to signal up to the PDCP layer to ECN-CE mark the next available PDCP-SDU (IP packet). This gives a tail-marking behavior that can degrade ECN efficiency for services that require low latency.

  - Mark/discard decision difficult. It is not possible to know on the RLC layer if an RLC-SDU is ECN capable, this means that a not-ECN capable must be discarded on the PDCP layer, this leads to inefficient tail-drop AQM implementations.

  - Split architecture deployments may implement PDCP processing in one node and RLC and lower layer processing in another node, with a transport link in between. The additional delay between the PDCP and RLC layer makes it a challenge to achieve a low queue delay on the RLC layer, given the simultaneous requirement to have enough data available on the RLC layer to optimally use the transport blocks.

- Header compression efficiency: Changing ECN bits gives a reduced compression efficiency for instance for ROHC. This situation is further aggravated if PDCP-SDUs are ECN marked on the PDCP layer prior to header compression.

- Processing efficiency: For high speed data it can be beneficial to preprocess a few PDCP-SDUs for reasons of processing efficiency. This can give further tail-mark/drop problems as described in 1) above

An addition of ECN marking capability on layers below the IP layer in 3GPP access gives the following benefits:

- Enhanced ECN marking efficiency. ECN marking will always be in-front of queue, which is beneficial especially for services that require low latency. For instance, URLLC is one example but enhanced ECN marking capabilities is beneficial for a very wide set of use cases, ranging from OTT services to specialized industrial use cases.

- Efficient ECN marking is maintained even though PDCP-SDUs are preprocessed on PDCP layer.

- Increased header compression efficiency in case that the ECN bits are zeroed prior to header compression.

- PDCP-SDU discard is not recommended in NR, deployment of ECN reduces the likelihood of buffer overflow and packet discards on the RLC layer.

In addition, the suggested change addresses the remaining issue #1 in an LS on ECN from IETF [3].

Section 5 gives an example that shows the potential with enhanced ECN marking capabilities in NR.

ECN support only in PDCP layer becomes a limiting factor for services that require low latency and high throughput.

ECN support in a protocol header that is accessible on the RLC layer is required for the enhanced ECN capability in NR that is necessary for the support of low latency and low loss access that is beneficial for a large variety of services.

## Realization

For the optimal implementation of ECN and especially the implementation of L4S support in RAN it is recommended that a lower layer header (from here on denoted LLH) that is accessible from the RLC layer include the ECN bits. This makes it possible to set the ECN bits in the LLH when congestion is detected on the RLC layer. The semantics to copy the ECN bits between the IP header and the LLH should follow the guidelines in RFC6040 [1] in which case the outer header is represented by LLH. The benefit with this is that it is possible to mark packets that are in the head of the queue with the obvious result that congestion signals reach the destinations faster. 
The location of the ECN bits (PDCP header, RLC header or other header) is left for discussion and decision in the RAN2 working group.

The steps to involved in the ECN bits handling are as follows, for simplicity the term lower layer header (LLH) is used here, this depicts the protocol header where the ECN bits modification is effected when congestion is detected on the RLC layer:

- ECN bits in IP header (PDCP-SDU) are copied to lower layer header

  - ECN bits in IP header are optionally zeroed for increased compression efficiency.

- PDCP-PDU is forwarded down to RLC layer.

- When congestion related mark/drop decision is made in RLC layer:

  - LLH has ECT(0) or ECT(1) code point set:  set CE code point in PDCP-PDU

  - LLH has Not-ECT code point set  discard packet, as per previous discussion, this case should be avoided

  - LLH has CE code point set  no action

- At PDCP de-capsulation of IP header: Copy ECN bits from LLH to IP header

  - If IPv4  update checksum in IP header

The suggested steps above are of low complexity. As regards to bullet Cb) above, PDCP discard in NR is regarded as a general problem and should be avoided, this is another motivation for the use of ECN as packet discards due to congestion can be avoided.

Affected nodes:

The suggested change requires a change in both gNB and UE. It is recommended that the ECN support is implemented both for UL and DL user plane processing.

ECN support in the form of ECN bits in a protocol header implies added functionality to copy the ECN bits between the IP header and the lower layer protocol header that is accessible from the RLC layer.

## Text proposal

This is the text proposal to TS 38.300.

## 2	References

[xx]	IETF RFC 3168 (09/2001): "The Addition of Explicit Congestion Notification (ECN) to IP".

## X.Y      Explicit Congestion Notification

The eNB and the UE support of the Explicit Congestion Notification (ECN) is specified in Section 5 of [xx] (i.e., the normative part of [xx] that applies to the end-to-end flow of IP packets), and below.  ECN is beneficial especially for latency sensitive interactived applications such as chat and gaming as well as for real-time voice and video, this because loss as a congestion signal is avoided, losses that would otherwise necessitate retransmission of packets with additional application delay as a result.

ECN is supported by the user-plane protocols through carrying the ECN codepoints.

Editor’s note: It is for further study which user-plane protocol carries the ECN codepoints.

# Conclusion

In section 2 we made the following observations:

Observation 1	ECN support in PDCP layer only becomes a limiting factor for services that require low latency and high throughput.

Observation 2	ECN support in a protocol header that is accessible on the RLC layer is required for the enhanced ECN capability in NR that is necessary for the support of low latency and low loss access that is beneficial for a large variety of services.

Observation 3	ECN support in the form of ECN bits in a protocol header implies added functionality to copy the ECN bits between the IP header and the lower layer protocol header that is accessible from the RLC layer.

Based on the discussion in section 2 we propose the following:

Add support for enhanced ECN for Uplink and Downlink user plane traffic in NR.

Study which layer (PDCP, RLC or other) should carry the ECN bits.

# References

https://tools.ietf.org/html/rfc6040

https://tools.ietf.org/html/rfc3168

R2-1700677, “Response to Reply LS in response to (SP-150579) on 3GPP Work on Explicit Congestion Notification for Lower Layer Protocols”, IETF, RAN2#97, Athens, Greece, 13 - 17 February, 2017

http://www.gsma.com/newsroom/wp-content/uploads/WWG.16-v1.0.pdf

https://www.ietf.org/proceedings/98/slides/slides-98-maprg-tcp-ecn-experience-with-enabling-ecn-on-the-internet-padma-bhooma-00.pdf

# Example simulation

The potential with efficient ECN marking in the context of L4S is shown in this section. The example shows a comparison between three congestion control algorithms when a large file is transmitted over a 20ms RTT bottleneck where the link rate changes between 20 and 100Mbps in 5 second steps. The three congestion control algorithms are run in one iteration each.

The congestion controls are:

- Cubic: The most widely used congestion control algorithm today. Cubic is of the type AIMD (Additive Increase Multiplicative Decrease), which means that rate increase when link throughput is increased can become slow.

- BBR: A novel congestion control algorithm devised by Google that estimates the link throughput and RTT and determines the optimal sending rate based on this information. For an efficient operation BBR must estimate the min RTT at regular intervals (every 10s), in effect this means that the transmission rate is reduced for a short period. The success of such operations depend on the traffic mix over the bottleneck, other traffic over the bottleneck can give faulty min RTT estimates, which will degrade the performance of BBR

- DCTCP-bis: A modified data center TCP (DCTCP) congestion control algorithm that exploits the potential with L4S, the development of congestion control algorithm goes the working name ‘Prague’. The DCTCP-bis algorithm is a very early version that points out the potential with Prague given the enhanced congestion information given by L4S.

Figure 1 shows the throughput, DCTCP-bis manage to fill the bottleneck most efficiently with BBR on second place. Cubic has problems to ramp up the speed when the link bitrate increases due to the AIMD nature of Cubic. The graph also shows that DCTCP-bis is slightly faster than BBR to reach full link utilization.

Figure 2 shows the queue delay in the bottleneck: Cubic has the largest standing queue. This is because the AQM must be tuned with a higher mark/discard threshold to allow for full link utilization. The graph illustrates that both Cubic and BBR gives a slow start delay spike, this can have a negative effect on other already ongoing flows as they will too experience this delay spike. DCTCP-bis does not give any noticeable delay spike in slow start at all, the latter indicates that the L4S concept, with enhanced ECN support in RAN and congestion control adapted for this, has potential to give low queue delay even in complex scenarios with multiple parallel flows over the same bottleneck.

BBR manages to reach roughly the same queue delay as DCTCP-bis. BBR however takes longer to restore a low queue delay when the bottleneck bandwidth drops from 100 to 20Mbps than what is the case with DCTCP-bis.

Figure 1 Throughput

Figure 2 Queue delay