# 3GPP TSG RAN WG2 Meeting #98	R2-1705171

# Hangzhou, China, 15th -19th May 2017

Agenda Item:	10.4.1.2

Source: 	Huawei, HiSilicon

Title:	RRC Establishment Cause

Document for:	Discussion and Decision

- Introduction

The RRC establishment causes are mainly used for the network to differentiate the RRC connection establish request for the subsequent admission control processing. In NR, it has developed many use cases and identified various potential services. In this contribution, we share our view on the possibility to add more establishment cause for the various service requirements.

- Discussion

  - Establishment cause in LTE

In LTE, the establishment cause values were specified [1]:

EstablishmentCause ::=				ENUMERATED {

emergency, highPriorityAccess, mt-Access, mo-Signalling,

mo-Data, delayTolerantAccess-v1020, mo-VoiceCall-v1280, spare1}

In addition, in rel-14 RAN2 agreed to reuse mo-VoiceCall for MO video.

In rel-13, it’s also specified the establishment cause for NB-IOT to provide the establishment cause for the RRC connection request or the RRC connection resume request as provided by the upper layers. In NB-IOT, the cause value of emergency, high priority access, delayTolerantAccess-v1020 and mo-VoiceCall-v1280 will not be applied for NB-IOT and introduced some new cause values: mo-ExceptionData and delayTolerantAccess-v1330.

EstablishmentCause-NB information element

-- ASN1START

EstablishmentCause-NB-r13 ::=			ENUMERATED {

mt-Access, mo-Signalling, mo-Data, mo-ExceptionData,

delayTolerantAccess-v1330, spare3, spare2, spare1}

-- ASN1STOP

The establishment causes are mainly used for the network to differentiate the RRC connection establish request for the subsequent actions. Firstly network needs to understand the establishment cause for admission control. Networks can perform CAC with the deterministic establishment cause. For RRC connection establishment request with spare value, it is difficult for networks whether or not to reject the request or not. Secondly, the establishment cause can also impact on access barring control. Before UE sends RRC connection establishment request to networks, the UE can identify the call type according the mapping relationship between establishment cause and call type to determine whether or not the request should be barred. ACB is the basic functionality in rel-8. In the following version, it’s also specified other enhancement based on it. In access control, eNB sends system information contains the ACB related parameters so that UE can perform access control according to the establishment cause and access class.

The Establishment Cause in LTE can still be used as the baseline for NR discussion.

Proposal 1: Reuse LTE Establishment Cause as the start point for NR discussion, i.e. to have:

- emergency, highPriorityAccess, mt-Access, mo-Signalling, mo-Data, delayTolerantAccess?, mo-VoiceCall, mo-VideoCall;

In addition, in past release there were lots of discussions on whether MT-Voice and MT-Video are needed. For NR, these cause values can be considered once we have clear view on what paging mechanisms will be from both RAN and CN side.

Proposal 2: can consider Establishment Cause mt-voice, mt-video once the paging mechanism in both RAN and CN is clear;

  - RRC connection caused by critical communication

The RRC connection cause is used according to the table which specifies the mapping relationship of NAS procedure to RRC establishment causes [2]. Currently call type of NAS includes originating signalling", "emergency calls", "originating MMTEL voice”, "originating MMTEL video", "originating SMSoIP", "originating SMS", "terminating calls", originating calls".

Currently, In NR, it’s possible to bring more call type to adapt various service requirements. In SA1, it has developed many use cases and identified various potential services to support Critical Communications, massive MTC and enhanced MBB. Especially for the critical communication, since it’s a totally new use case family for 3gpp, more call types and RRC establish causes are possible to be specified.

Proposal 3: More establishment Cause needs to be defined for adapting various service requirements, especially for critical communication (e.g higher reliability, very low latency, higher accuracy positioning, etc.)

- Proposal

In this contribution, we discussed RRC connection establishment causes in NR and proposed that:

Proposal 1: Reuse LTE Establishment Cause as the start point for NR discussion, i.e. to have:

- emergency, highPriorityAccess, mt-Access, mo-Signalling, mo-Data, delayTolerantAccess?, mo-VoiceCall, mo-VideoCall;

Proposal 2: can consider Establishment Cause mt-voice, mt-video once the paging mechanism in both RAN and CN is clear;

Proposal 3: More establishment Cause needs to be defined for adapting various service requirements, especially for critical communication (e.g. higher reliability, very low latency, higher accuracy positioning, etc.)

- References

[1] TS 36.331-e22.doc

[2] TS 24.301-e30.doc