3GPP TSG-RAN WG2 #99	R2-1707790

Berlin, Germany, 21st – 25th August 2017	(update of R2-1702556)

Agenda Item:	9.7.2

Source:	Ericsson

Title:	Security aspects of supporting LTE connected to 5G-CN

Document for:	Information

# Introduction

This contribution is a revision of R2-1702556 submitted to last RAN2#97-Bis meeting. It has been agreed that LTE eNBs should be connected to Next Gen CN (5G-CN) to support connectivity options 5 (LTE standalone) and 7/7a (LTE assisted NR) as well as support fast RAN level mobility to option 2 (standalone NR). This contribution discusses the impacts on LTE specifications from this related to access security.

# Discussion

The work on 5G security is currently ongoing in SA3 making it difficult to assess all impacts on the LTE and NR specifications related to supporting 5G CN connectivity. Nevertheless, it is still important that the RAN WGs work in parallel to the SA3 work to develop RAN level mechanisms that could meet the security requirements determined in SA3. As part of this work it is important to assess if existing LTE security solutions can be applied in NG-RAN and what are the impacts on LTE security if SA3 introduces additional security mechanisms.

RAN WGs should be able to assess if the LTE access security solutions can be applied to (or are suitable for) NG-RAN from a technical point of view, while SA3 should assess the security of these solutions and determine if any enhancements are needed from a security point of view.

# Possibility to reuse LTE access security solutions

A big part of the LTE access security solution is the handling of keys (incl. key derivation, signalling between nodes). The key handling mechanism in LTE has been designed to handle forward and backwards security.

- Backward security: prevents attacks where the attacker has obtained a user key (e.g from a compromised node) and uses this key to decode recorded traffic sent/received earlier by the UE in previous nodes. Backwards security is supported by using “one-way” key derivation at handover and state transitions making it computationally infeasible for the target node to derive any key used in the source node.

- Forward security: prevents attacks where the attacker has obtained a user key (e.g from a compromised node) and uses this key to decode future traffic between the UE and future nodes. Forward security is supported in LTE RAN with help of the CN providing new key material after handover and state transitions down to the RAN.

Although there have been discussions on decoupling the key derivation from the handover for intra-RAN node handover, there has so far not been any proposals challenging the need to support the forward and backward security concepts between different RAN nodes also in NG-RAN (LTE eNBs and NR gNBs connected to 5G-CN).

It is assumed NG-RAN security will adopt both forward and backward security between different RAN nodes.

The solution in LTE for forward and backward security is based on using vertical and horizontal key derivations at handover and state transitions, where the key derivation at handover is controlled by the Next hop Chaining Counter (NCC) signalled to the UE. To our knowledge so far no issues has been raised with this concept from a RAN point of view. Assuming this concept meets the 5G security requirements as well in SA3 it should be possible to adopt also for the NG RAN.

The current mechanism in LTE for derivations of RAN keys  that support forward and backward security should be adopted as a working assumption for NG-RAN assuming it can fullfill the 5G security requirements (to be determined by SA3).

# Possible impacts to LTE for supporting 5G-CN security

- This section discusses potential impacts on LTE if SA3 decides to enhance the 5G security in different ways. The enhancements below are speculative, nevertheless it is useful to analyse potential RAN impacts.

## What if new integrity protection or ciphering algorithms and UE security capabilities are introduced for 5G-CN?

Currently in LTE four different ciphering and integrity protection algorithms are supported. Which algorithm the UE should use is signalled over the radio interface in RRC security setup and at handover. The UE shall support the first three algorithms while the fourth is optional.

The mechanism to handle the security algorithms is future proof since it is possible to add more algorithms in the future (up to 16 algorithms with current signalling). So even if new algorithms are introduced as part of 5G CN and NR it should be possible to support this in LTE using existing methods (with minimum standard impacts) assuming legacy algorithms are mandatory for 5G UEs. No impacts on existing LTE eNB HW is foreseen assuming it is allowed for LTE eNBs to continue to use the legacy algorithms.

It should be possible to add new security algorithms in 5G-CN and NG-RAN with minimum RAN2 specification effort and without impacting legacy LTE eNB HW.

Regarding the UE security capabilities, these capabilities are in LTE common for AS and NAS and managed by the CN and sent down to the RAN during UE context setup. The capabilities are also echoed back to the UE on NAS layer to prevent bid-down attacks. It is assumed similar solution can be adopted for 5G-CN. Which algorithms will be mandatory for the UE / network to support can be discussed later.

Assuming the UE security capabilities are handled in the same way for 5G-CN as in EPS there should be no or minimum impacts on RAN2 specifications.

Ask SA3 to confirm the following:

  - That 5G-CN will allow support of legacy LTE security algorithms to be used in AS layer for LTE eNBs serving UEs attached to 5G-CN.

  - The UE security capabilities related to the security algorithms used in AS layer will be signalling on NAS layer and provided to the AS layer during initial UE context setup (similar to how it works in EPC).

  - The if new algorithms are introduced for 5G-CN / NR these algorithms can also be optional supported by LTE eNBs serving UEs attached to 5G-CN.

## What if longer security keys should be used for 5G?

Currently in LTE, 128 bit keys are used for RRC and UP ciphering and integrity protection (e.g. Kupenc). The KeNB (and KeNB* and NH) are however 256 bits since the 128 bit keys are actually truncated at the last step from 256 bit keys inside the eNB and UE as can be seen in the figure 1 from 33.401.

MME

HSS

CK,IK

256

256

KDF

256

256

SN id, SQN  AK

KeNB

KASME

256

SNCK,IK

ME

SN id, SQN  AK

KDF

256

KDF

KDF

KDF

256

KDF

KSN id, SQN  AK

eNB

KeNB

KNASenc

KeNB

KNASint

KASME

MME

256

ME

KNASenc

KNASint

2KDF

CK,IK

HSS

Trunc

256

Trunc

256

KDF

KDF

256

256

128

KN256

Trunc

Trunc

KRRCint

128

256

256

UP-enc-alg,

Alg-ID

256

KUPenc

KUPenc

256

256

NAS-enc-alg,

Alg-ID

NAS-int-alg,

Alg-ID

Physical cell ID, EARFCN-DL

NAS UPLINK COUNT

KeNB*

256

256

nc

KDF

KDF

KDF

256

KUPenc

256

KUPenc

KUPenc

KRRCint

12128

256

Trunc

256

KUPenc

256

NH

KDF

KRRCint

Trunc

Trunc

KDF

KUPenc

256

Trunc

256

256

128

128

256

128

256

UP-enc-alg, Alg-ID

KDF

RRC-int-alg, Alg-ID

NH

RRC-enc-alg, Alg-ID

256

256

Physical cell ID, EARFCN-DL

256

KeNB

eNB

eNB

KeNB*

KDF

KRRCenc

KRRCenc

256

256

128

Trunc

KDF

NH

SN id, SQN  AK

SN id, SQN  AK

NH

KeNB

256

KeNB

KeNB

256

KDF

Trunc

KASME

KASME

UP-int-alg, Alg-ID

KUPint

256

256

256

256

MME

MME

KUPint

128

SN id, SQN  AK

Figure 1 Current key derivation for EPS on the network side

This means that even if 256 bit keys should be applied for NR radio interface protection, it is possible to inter-work will with legacy LTE eNBs since:

- LTE already support management of 256 bit keys (including inter-node signalling and key derivation)

- LTE eNBs only supporting legacy security algorithms can continue to use 128 bit keys as today (by truncating)

It is assumed that 256 bit keys can be introduced in the standard as new 256 bit security algorithms using the mechanism described in the previous section. In this way it could be optional for LTE eNB to support the 256 bit algorithms.

LTE already supports the handling of security keys up to 256 bits meaning no impacts on LTE for supporting inter-working with 5G-CN and NR for keys up to 256 bits.

Ask SA3 if 256 bit algorithms will be support in NG-RAN and if so confirm the assumption that LTE today already can handle 256 bit keys and that only eNBs support the new algorithms would be impacted.

## What if different AKAs will be used in the 5G-CN, meaning Kasme / NH / KeNB etc. are derived in a different way?

The way the current LTE access security is defined (the right part of Figure 1), it does not care from where or how the KeNB and NH is derived (the left part of Figure 1). So if SA3 would introduce a new way to derive the KeNB and NH sent down to the eNB over the NG-C interface this does not matter for the LTE RAN as long as the resulting key is still 256 bits. The further handling of the KeNB and NH in the RAN (and UE) can be based on legacy LTE principles (NCC, key derivation) i.e. the right part of Figure 1.

Regardless how the KeNB and NH are derived in the 5G-CN it should still be possible to treat them in the same way in the NG-RAN as in LTE assuming the size of the keys are still 256 bits.

Ask SA3 what AKAs will be used in 5G-CN and if this has any impacts om the key handling in RAN or if legacy KeNB, NH and NCC based key handling can be applied.

## Impacts of supporting UP integrity protection in NG-RAN?

It has been agreed in SA3 that NG-RAN should to support UP integrity protection for DRBs. It should be technically possible to also support for UP integrity protection for eNBs and UEs in LTE connected to 5G-CN. The use of UP integrity protection could be a network choice assuming the UE support it. Since user plane DRBs are always explicitly configured by the network using dedicated signalling and always established after security activation there will most likely not be any impacts on other functionality and it is possible to decide when to use UP integrity protection on a per eNB level. There will most likely also be no risk of bid down attacks since:

- Configuration of UP integrity protection would be done using integrity protected RRC or NAS signalling,

- UE security capabilities are assumed to be protected against bid-down attacks.

It should be possible to add support for UP integrity protection in NG-RAN with minimum RAN2 specification effort and without impacting legacy LTE eNB HW.

- In the LS S3-172080 SA3 indicated the following:

- it is acceptable that not all eNBs connected to 5GC support UP integrity protection

- it UP integrity protected could easily be available also for eNBs connected to 5GC this is off course good.

- With analysis above in mind the following proposal is made.

UP IP can be optionally supported in LTE eNBs for UEs attached to 5G-CN

# Conclusion

Even if 5G-CN and NG-RAN introduces support for new:

- New AKA or key generation mechanism in 5G-CN

- New integrity and/or ciphering algorithms

- Larger key size (up to 256 bits)

- UP integrity protection

this can be done in a backwards compatible way for LTE eNBs connected to 5G-CN requiring minimum specification impacts since LTE eNB can continue to (optionally) support only 128 bit keys and existing algorithms, and existing key handling (derivation) in the RAN (key handling in CN is not seen).

Proposal 1	The current mechanism in LTE for derivations of RAN keys  that support forward and backward security should be adopted as a working assumption for NG-RAN assuming it can fullfill the 5G security requirements (to be determined by SA3).

Proposal 2	Ask SA3 to confirm the following:

a.	That 5G-CN will allow support of legacy LTE security algorithms to be used in AS layer for LTE eNBs serving UEs attached to 5G-CN.

b.	The UE security capabilities related to the security algorithms used in AS layer will be signalling on NAS layer and provided to the AS layer during initial UE context setup (similar to how it works in EPC).

c.	The if new algorithms are introduced for 5G-CN / NR these algorithms can also be optional supported by LTE eNBs serving UEs attached to 5G-CN.

Proposal 3	Ask SA3 if 256 bit algorithms will be support in NG-RAN and if so confirm the assumption that LTE today already can handle 256 bit keys and that only eNBs support the new algorithms would be impacted.

Proposal 4	Ask SA3 what AKAs will be used in 5G-CN and if this has any impacts om the key handling in RAN or if legacy KeNB, NH and NCC based key handling can be applied.

Proposal 5	UP IP can be optionally supported in LTE eNBs for UEs attached to 5G-CN

It is proposed to send an LS (draft available in R2-1707791) to SA3 confirming the proposals 1-4 above in this document.