3GPP TSG-RAN WG2 #99                                                            	R2-1707909

Berlin, Germany, 21 - 25 Aug 2017	Revision of R2-1706410

Source:	CATT

Title:	Procedure of paging in inactive

Agenda Item:	10.4.1.6.3

Document for:	Discussion and Decision

Introduction

This paper discusses the procedure of paging for UE in inactive. Paging in NR can be initiated by RAN or by CN for UE in inactive, the procedure for RAN-initiated paging should only involve AS, but the procedure for CN-initiated paging should also involve the NAS which is different from RAN-initiated paging. UE distinguishes whether the paging message is initiated by RAN or CN from the different kind of UE ID included in the paging message, i.e.  a RAN UE ID allocated as UE identity in inactive state is included in RAN-initiated paging , and a CN UE ID is included in CN-initiated paging

Discussion

UE in inactive checks both RAN UE ID and CN paging ID, if the UE finds the RAN UE ID matching with the UE_ID in paging message, the UE would be aware of the paging is initiated by RAN, else if the CN UE ID is matching with the UE_ID in paging message, it is initiated by CN. UE will initiate different procedures according to the paging initiated by CN or RAN.

RAN-initiated paging

If the UE receives a paging message including RAN UE ID, it is initiated by RAN. UE should transit from RRC-inactive state to RRC-connected state by RRC connection resume procedure. The procedure of RAN-initiated paging is as described in Figure1.

Figure1. Procedure of RAN-initiated paging

Step1: gNB receives the data transmitted from CN to UE which is in RRC-inactive state. gNB checks the UE context of the paging UE. If it is successful in retrieving and verifying the UE context, gNB sets the ue-identity to the RAN UE ID including in the paging message, then gNB sends the paging message on the  paging occasion which is calculated according to the CN UE ID stored in UE context to be used in the calculation. Else if the UE context is retrieved or verified unsuccessfully, gNB will indicate to CN UE context release.

Step2: UE monitors the paging message on PO, and receives the paging message, when UE checks the RAN UE ID matched with the ue-identity in paging message, UE initiates the transition from RRC-inactive state to RRC-connected state upon UE finds the paging is initiated by RAN. A RRC connection resume request message which including the RAN UE ID is assembled and sent by the UE to initiate the RRC resume procedure.

Step3: gNB will check the RAN UE ID including in the message and UE context at the reception of the RRC connection resume request message. If the UE context matching with the RAN UE ID exists, gNB constructs the MSG 4 (of RA procedure) and sends the message to UE.

Step4: When UE receives the RRC connection resume message, UE enters RRC-connected state and send MSG5 (of RA procedure) to indicate gNB.

Step5: gNB receives the MSG5, RRC connection resume procedure complete. The data can be transferred between UE and CN.

Proposal 1: If the UE receives a paging message including RAN UE ID, the UE should initiate RRC connection resume procedure to transit from RRC-inactive state to RRC-connected state.

CN-initiated paging

if the paging message including a CN UE ID, it is a CN-initiated paging message, when UE receives a CN-initiated paging, the UE considers losing connection with the gNB, so UE enters RRC-IDLE state, and indicates to NAS about the transition and the paging message, then UE waits for the order from NAS, if NAS notifies UE to setup RRC connection, UE will transit from RRC-IDLE state to RRC-connected state by RRC connection setup procedure.

Figure 2. Procedure of CN-initiated paging for UE in inactive

Step1: CN sends a paging message including the CN UE ID to gNB. gNB receives the paging message from the CN, assembles the paging message which the ue-identity is set to the CN UE ID including in the received message, and sends the paging message to UE on paging occasion calculated by CN UE ID.

Step2: UE in inactive state monitors the paging message on PO, and receives the paging message when UE checks the RAN UE ID not matched with the ue-identity in paging message but the CN UE ID matched. UE is aware of that it is a CN-initiated paging. UE enters into idle state, and indicates to NAS about RRC connection release and the CN paging message.

Step3: The paging information is provided to NAS, which in response may initiate RRC connection establishment, e.g. receives an incoming call or data transfer. If NAS indicates UE to setup RRC connection, the UE will send the RRC connection request message to gNB to initiate the RRC connection setup procedure, when UE receives the RRC connection setup message, UE enters into RRC-connected state, and sends RRC connection complete message to gNB. Then gNB initiates the UE context setup procedure and DRB configuration procedure. The data transfer from UE and CN can be performing .

Proposal 2: If the UE receives a CN-initiated paging message, the UE should enter into idle state, indicate to NAS about RRC connection release and then indicate the reception of CN paging.

Conclusion

In this contribution, we discuss the procedure of paging and propose:

Proposal 1: If the UE receives a paging message including RAN UE ID, the UE should initiate RRC connection resume procedure to transit from RRC-inactive state to RRC-connected state.

Proposal 2: If the UE receives a CN-initiated paging message, the UE should enter into idle state, indicate to NAS about RRC connection release and then indicate the reception of CN paging.

Reference

[1]  RAN2#97bis, Chairman notes.

[2] TS 36.331