3GPP TSG-RAN WG2 Meeting #99                                       	                     R2-1708045

Berlin, Germany, 21st – 25th August 2017

Agenda Item:	10.3.1.13

Source:	MediaTek Inc.

Title:	Fallback mechanism for Bandwidth part operation

Document for:     Discussion and Decision

# 1 Introduction

In RAN1 NR AH#2 meeting [1], RAN1 send RAN2 and RAN4 a LS, which lists all RAN1 agreement about bandwidth part (BWP) operation since RAN1#88 to date. BWP enables UE to achieve power saving by adaptively applying/monitoring different size/range of bandwidth. The following is part of RAN1 agreement about BWP in RAN1 NR AH#2 meeting [1].

Agreements: - Activation/deactivation of DL and UL bandwidth parts can be - by means of dedicated RRC signaling - Possibility to activate in the bandwidth part configuration - by means of DCI (explicitly and/or implicitly) or MAC CE [one to be selected] - by means of DCI could mean - Explicit: Indication in DCI (FFS: scheduling assignment/grant or a separate DCI) triggers activation/deactivation - Separate DCI means DCI not carrying scheduling assignment/grant - Implicit: Presence of DCI (scheduling assignment/grant) in itself triggers activation/deactivation - This does not imply that all these alternatives are to be supported. - FFS: by means of timer - FFS: according to configured time pattern

In this paper, we first provide a brief summary of RAN1 agreement for BWP, and then propose fallback mechanism to assist BWP operation, i.e., UE can automatically fallback/switch to a BWP with a small bandwidth size when there is smaller traffic activity.

# 2 Introduction to Bandwidth Part (BWP)

NR support a high carrier bandwidth. According to [2], from RAN1 specification perspective, maximum channel bandwidth per NR carrier is 400MHz in Rel-15. Therefore, to support the data transmission of those UE not capable of supporting the carrier bandwidth, the resource allocation could be derived based on a two-step frequency-domain assignment process, i.e., indication of a bandwidth part (the 1st step) and indication of the PRBs within the bandwidth part (2nd step). In the two-step indication, each bandwidth part (BWP) consist of a group of continuous PRBs in a component carrier. If UE is configured with BWP, UE is not required to receive any DL signals outside the BWP frequency range, i.e., UE can monitor only part of frequency range of the carrier bandwidth.

The BWP configuration is for a RRC connected UE, and the bandwidth of BWP configured to a UE should be equals to or is smaller than the maximal bandwidth capability supported by a UE. A BWP is at least as large as the SS block bandwidth, and may or may not contain the SS block. BWP configuration may probably include the numerology, the frequency location (e.g. center frequency), and bandwidth (e.g. number of continuous PRBs). Each bandwidth part is associated with a specific numerology (sub-carrier spacing, CP type).

UE expects at least one DL BWP and one UL BWP being active among the set of configured BWPs for a given time instant. The indication of active DL/UL BWP(s) to a UE may be through one or combinations of DCI, MAC CE, or time pattern. In addition, activation/deactivation of DL and UL BWP(s) can be by means of dedicated RRC signaling, and by DCI (explicitly and/or implicitly) or MAC CE [one to be selected]. Further, it is FFS whether activation/deactivation of DL and UL BWPs is by timers.

Observation 1: Bandwidth part supports resource allocation of UE that cannot support the carrier bandwidth.

In addition to support UE with limited bandwidth capability, the concept of BWP enable bandwidth adaptation in a finer granularity, i.e., in terms of PRBs. A high BWP support high data transmission rate, with the cost of higher power consumption at RF for monitoring wider bandwidth. Therefore, if the data rate is low, e.g. traffic from low data rate service, UE could apply a small BWP to reduce power consumption in bandwidth monitoring.

Observation 2: BWP configuration enable UE DL/UL bandwidth adaptation, and thus has potential to save bandwidth monitoring power while still satisfying data rate requirement.

# 3 Discussion

With the support of activation/deactivation/indication mechanism for BWP in RAN1, bandwidth adaptation mechanism could be developed in RAN2 to save bandwidth monitoring power. To be specific, UE can fallback to a BWP with smaller frequency range, i.e., small BWP, when any of the following trigger conditions are met:

- UE goes into DRX mode. If DRX is configured and DRX operation is applied, UE needs not monitor the whole carrier bandwidth during on-Duration, i.e., in on-Duration, UE could monitor the PDCCH in the configured/fallbacked BWP, rather than PDCCH of the overall carrier bandwidth. UE can be switched to a larger BWP until network send PDCCH to inform UE of data arrival.

- Inactivity timer expiration. An inactivity timer could be introduced for BWP fallback. For example, if a UE is idle without DL and UL data transmission until the inactivity timer expires, UE fallback to small BWP.

- Buffer status below a threshold. If UE reports buffer status below a threshold, network could consider to switch the UE to a small UL BWP since there is no requirement for high UL data rate.

Observation 3: UE can save bandwidth monitoring power by fallback to a smaller BWP when low data rate traffic pattern is detected.

There may be several ways to decide the fallback BWP. For example,

- Alternative 1. The fallback BWP may be explicitly specified by the network when network configures BWP for a UE.

- Alternative 2. A default BWP is always used for BWP fallback, i.e. anchor BWP. For example, the smallest BWP containing SS block is used for BWP fallback.

- Alternative 3. Fallback BWP is temporarily decided and signaled by the network from configured BWPs. When BWP fallback is needed, network indicate a BWP to be activated as fallback BWP through DCI or MAC CE command.

In addition, BWP fallback may be triggered explicitly or implicitly.

- Explicit indication: UE perform BWP switch based on network instruction, e.g., through DCI or MAC CE.

- Implicit indication: For example, UE apply the fallback BWP autonomously when UE enters DRX mode or when an inactivity timer for BWP fallback expires. And network expects a UE applies fallback BWP if fallback condition(s) are met.

Proposal 1: RAN2 study trigger condition(s) for BWP fallback.

Proposal 2: RAN2 introduce an inactivity timer for BWP fallback.

# 3 Conclusion

Based on the observations:

Observation 1: Bandwidth part supports resource allocation of UE that cannot support the carrier bandwidth.

Observation 2: BWP configuration enable UE DL/UL bandwidth adaptation, and thus has potential to save bandwidth monitoring power while still satisfying data rate requirement.

Observation 3: UE can save bandwidth monitoring power by fallback to a smaller BWP when low data rate traffic pattern is detected.

We propose:

Proposal 1: RAN2 study trigger condition(s) for BWP fallback.

Proposal 2: RAN2 introduce an inactivity timer for BWP fallback.

# 4 Reference

- 3GPP, RAN1 NR AH#2 Chainman Notes

- 3GPP, TR 38.802 V14.1.0, Study on New Radio Access Technology, Release 14