3GPP TSG-RAN WG2 NR #99 Meeting                                            	R2-1708092

Berlin, Germany, 21st-25th August 2017	(Update of R2-1706430)

Agenda item:	10.4.1.4.7

Source:	Samsung

Title:	RRM measurement for multiple numerologies in NR

Document for:	Discussion & Decision

# 1. Introduction

RAN#75 in March approved a 5G WID [1] on New Radio (NR) access technology, which targets a unified framework for traffic with diverse QoS requirements e.g. enhanced mobile broadband (eMBB), massive machine-type-communications (mMTC), ultra-reliable and low latency communications (URLLC).

In RAN1 [2] using different sets of numerology and frame structures for different service verticals has been discussed. From RAN1 meetings during SI/WI phase, the followings were agreed:

RAN1 Agreements (#87)

- NR strives for efficient support of dynamic resource allocation of different numerologies in FDM/TDM fashion.

- NR should support adaptation and network indication of the valid time and frequency resources which may be used for inter-/intra-frequency RRM measurements and reports for ‘CONNECTED’ mode UEs

RAN1 Agreements (#89)

- Support following features for NR CSI acquisition

  - FFS Frequency domain subset restriction

    - FFS on number of configurable subsets

    - FFS on detailed signaling/configuration

    - FFS measurement restriction of interference measurement

      - FFS on measurement restriction of channel measurement

  - For time domain, measurement restriction of channel and interference measurement

  - CSI reporting via short duration PUCCH

    - FFS on detailed setting in CSI reporting setting

  - CSI reporting via long duration PUCCH

    - FFS on detailed setting in CSI reporting setting

  - PUCCH reporting which is contained in a single slot

    - FFS on PUCCH reporting which is contained in multiple slots

RAN1 Agreements (AH#2)

- Following CSI-RS properties for RRM measurement for L3 mobility are supported in NR

  - Configurable periodicity (as already agreed)

    - {5, 10, 20, 40, [80, 160]} ms are supported

    - This does not mean periodicity will be configured per CSI-RS resource

  - Configurable transmission bandwidth (as already agreed)

    - FFS candidate values

  - Configurable measurement bandwidth (as already agreed) and frequency location

    - At least minimum carrier bandwidth for each frequency band/range and at least one additional wider bandwidth for each SCS (e.g., maximum UE bandwidth) are supported

      - FFS other candidate values for wider bandwidth for each SCS

    - Measurement of CSI-RS in subband/bandwidth part which may or may not contain SS Blocks is supported

  - Configurable parameters for sequence generation

  - Configurable numerology

    - For each frequency range, subcarrier spacing values applicable to data, CSI-RS for beam management and SS block in the frequency range are supported

  - Association between CSI-RS for RRM measurement and SS block

    - It is assumed that property of spatial QCL between SS block and CSI-RS for beam management will be reused

  - Configurable CSI-RS time/frequency resource (as already agreed)

    - CSI-RS design including RE mapping and density for beam management is assumed as baseline

  - Note that above properties are relevant for RAN4 RRM measurement evaluations and not intended to be an exhaustive list of properties

From RAN2 meetings during SI/WI phase, the followings were agreed:

RAN2 Agreements (#97)

Agreements

1	An RRC_CONNECTED UE should be able to perform RRM measurements on always on IDLE RS (e.g. synchronization signal).

2	An RRC_CONNECTED UEs should be able to perform RRM measurements on additional RS (e.g. CSI-RS, MRS, etc.).

3	Network should be able to configure RRM measurements via dedicated signalling to be performed on additional RS and/or IDLE RS

Above agreements 2 and 3 are based on the assumption that RAN1 will define additional RS or connected mobility purposes which is not yet concluded in RAN1.

RAN2 Agreements (#97b)

Agreements

1	In NR, a measurement object is corresponding to one carrier frequency.

2	As part of the Measurement Object it is possible to configure a list of CSI-RS resource specific configurations for RRM measurement. (If this CSI-RS configuration is found to be usable for other purposes then its placement in the measurement object can be reconsidered)

This contribution discusses RRM measurement and handover procedure in NR when the network supports multiple numerologies.

# 2. 	Design Considerations for Connected Mobility in NR

  - Key Design Issues to Support Multiple Numerologies for HO

Resource for RRM measurement

For the convenience of discussion, how the resource for RRM measurement is configured can be categorized as following scenarios:

Scenario 1 – common resource across cells

Scenario 2 – only numerology specific resource per cell

Scenario 3 – common resource part across cells + numerology specific additional resource part per cell

For design simplicity, it is nice to have a common operation as well as common resource/reference signal for RRM measurement (as scenario 1), regardless of the type of service/slice. The common operation/resource, however, may not be efficient from the resource usage perspective, if it must support all kinds of services as well as requirements – even including future services. For example in the case of high mobility or wide BW, more resource elements to map RS are needed to acquire accurate measurement quantity. If we decide common resource for RRM measurement, the network should provide the densest RS to meet the most stringent requirement of a service requiring high mobility or wide BW. This may incur the higher overhead to the network for supporting the multiple services/numerologies.

If we separate resources for each service/slice group which can be categorized in terms of supporting numerology (as scenario 2), resources in carrier could be used more efficiently. For example, URLLC service may require shorter periodicity of RS than that for eMBB service. If URLLC UE is not connected to a cell at all, it is fine for the cell to provide just the nominal periodicity of RS for RRM measurement. Furthermore, the impact from the additional RS could be confined within the part of whole resource which is assigned only for the specific service.

Since scenario 1 and 2 has each benefit and drawback, scenario 3 can be considered as a compromising way. According to scenario 3, common resource for RRM measurement is assigned across cells to meet nominal services/requirements. If services with more stringent requirements are needed, additional resources could be assigned to provide proper accuracy for RRM measurement.

Observation 1: Common resource across cells for RRM measurement regardless of the type of numerology/service is beneficial for design simplicity, but may be resource-wasteful or may not provide sufficient accuracy of measurement quantity for all possible services.

Observation 2: Different resource for RRM measurement per numerology could be considered for further optimization from the resource efficiency and measurement accuracy perspectives.

Type of RS for RRM measurement

RAN1/2 agreed that both NR-SS and CSI-RS are used for RRM measurement and L3 mobility. Configuration details for SS and CSI-RS has been discussing so far. From the system point of view, the overhead from NR-SS should be minimized since NR-SS is the “Always-on” signal in NR. The aspects of the overhead comprise number of resource elements, BW used for RS mapping, and resource usage in time. However there is a trade-off between minimizing overhead of RS and accuracy of RS measurement. Due to the longer period (20 ms in default and configurable up to 160 ms) of NR-SS compared to that (1ms) of CRS in LTE, the network may not provide the sufficient number of samples to UE only via NR-SS, subject to detection error and efforts to track multiple cells. Thus CSI-RS may be required in specific scenarios to achieve more accurate RS measurement quality. Note that NR-SSS in common resource is contributed to RRM measurement of CONNECTED UE as well.

Observation 3: RAN1/2 agreed that both SS and CSI-RS is used for RRM measurement and L3 mobility.

Observation 4: RAN1 is discussing about CSI-RS configuration which may comprise timing, bandwidth, numerology, or resource elements.

Indication of measurement resource

During SI phase, RAN1 agreed on the indication of valid time and frequency resource for inter-/intra-frequency RRM measurements and reports for CONNECTED mode UE. Especially for intra-frequency case, the agreement implies that a time and frequency portion of the carrier is indicated to CONNECTED UE. RAN2 also agreed on CSI-RS configuration as a part of measurement object which implies dedicated signaling as RRC for the indication. Despite the measurement resource may be shared for L2 mobility (e.g. CSI-RS for beam management), the separate configuration for L3 mobility via RRC should be supported.

Proposal 1: To CONNECTED mode UE, measurement resource including time/frequency portion of the carrier may be indicated via RRC by the network for RRM measurement.

Potential impact of numerology to measurement/HO accuracy

UE may not assess well the interference from neighbor cells due to numerology mismatch and different hypothesis per numerology.

Observation 5: UE may not exactly measure interference of RSs from neighbor cells due to numerology mismatch.

The single cell may separate the resource into multiple regions corresponding to common and dedicated numerologies for multiple services/slices and configure those multiple regions dynamically to avoid resource wastage. Under these situations, UE which operates with a dedicated numerology will monitor neighboring cells via common resource and common numerology, if the UE does not know the resource/numerology configuration at those neighboring cells. In case that the UE measures neighboring cells only by common numerology within common resource, there is no impact from numerology mismatch between serving and neighboring cells.

Observation 6: There is no issue of numerology mismatch if UE measures neighboring cells only by common numerology/RS within common RRM resource across cells.

There however may be some constraints for RRM measurement based on common resource. First, as identified in observations, it may not meet requirements for all kind of services, but meet those for typical services. Second, it may suppress the freedom of cell planning and HO strategy for different service/slices.

If further enhancements are considered to handle those diverse scenarios, the alignment of common resource/numerology across cells is not assured or UE may experience numerology mismatch between serving cell and neighboring cells. This mismatch of numerology between serving and neighboring cell is harmful to obtain the consistent cell quality and goes severe when resource is shared within multiple numerologies dynamically. Although the numerology where a UE operates for measurement is static, the numerology and other physical parameters which are associated to the resource of neighboring cell could be transient (e.g. time sharing). In this case, the measured cell quality of the neighboring cell may be varied across different resource regions with different numerologies. Without any clue, the UE just obtains filtered result based on all those measured quantity regardless of numerology type which is used for the RRM resource.

Observation 7: There may be some potential issues due to numerology mismatch when CSI-RS in different numerology from NR-SSS is used or different numerologies are used for CSI-RSs within neighboring cells.

Proposal 2: RAN2 strives for RRM measurement of UE based on the same type of RS as well as the same numerology across cells based on the two options, SS-based (option 1) and CSI-RS-based (option 2).

Option1: Network does not configure UE with CSI-RS so that UE monitors only SS with the default numerology.

Option 1 does not introduce in NR Rel-15 any additional mechanisms/complexity for supporting different numerology configurations in different cells on the same frequency, but may cause inaccurate measurement result or higher resource wastage.

Option2: Network configures UE with CSI-RS and numerology so that UE monitors only CSI-RS with the configured numerology.

Option 2 introduces UE monitoring behavior to configured measurement resource pattern for supporting different numerology configurations in different cells on the same frequency, for the sake of higher measurement accuracy and efficiency.

Since UE can assume the common SS periodicity across cells, the UE does not need additional information for common RRM resource. Meanwhile, different cells may be configured in different CSI-RS with different numerology. To avoid numerology mismatch of CSI-RS, the UE needs information for RRM resource which could be regarded as common part with identical numerology. Since UE can measure the RRM resource with the configured numerology, we assume that only single numerology is associated to a measurement object. If network wants to change the numerology for RRM, the network can configure different measurement object. It means that only single numerology is associated to SS or a list of CSI-RS configuration in a measurement object.

Proposal 3: To avoid numerology mismatch when CSI-RS is configured for RRM, UE is configured for measurement report to a CSI-RS with measurement resource pattern for monitoring serving cells and/or neighbor cells, with the cost of more specification efforts.

Proposal 4: Single numerology is associated to single measurement object which may include SS or a list of CSI-RS configuration.

Having said that, to measure accurate cell quality in certain limited scenarios, differentiation of measured RRM resource in terms of numerology is required. In this respect, the time domain measurement resource restrictions for the UE to support LTE eICIC could be reused. Owing to the restriction, the UE can measure just a reliable resource for RRM measurement.

Observation 8: To handle potential problems due to numerology mismatch, measurement resource restriction for LTE eICIC could be reused.

In LTE, there two types of time domain measurement resource restriction which may be related to the problem of numerology mismatch what raised here. One is the time domain measurement resource restriction pattern for the PCell measurements (RSRP, RSRQ and the radio link monitoring) (referring to 36.331) (i.e. indication of ABS subframe). Another is the measurement subframe pattern of neighboring cells which is introduced to indicate non-MBSFN subframes of neighboring cell.

For assessment, we setup the simple case to apply the indication of ABS subframe into this problem of numerology mismatch. If the ABS subframes of only serving cell are indicated similar as LTE, a UE in the serving cell measures subframes which is indexed by 0 in Figure 1. Since the UE cannot recognize the source of interference due to numerology mismatch, cell quality measured by the UE may not be consistent. As an extension, if the ABS subframes for both serving and neighboring cells are indicated, the UE can restrict for serving cell measurement to the subframes which is non-ABS in the serving cell and ABS in the neighboring cell. In spite of the extension, the UE only distinguish not-impaired signal power from the impaired. That is, accurate channel quality based on SINR is not acquired in this case. So, it is doubtful to acquire reliable measurement quantity for connected mobility.

Figure 1: Indication of ABS subframe of serving/neighbour cells

Proposal 5: RAN2 considers further enhancement to LTE measurement restriction to handle numerology mismatch among cells.

# 3. Summary

Based on the discussion and identified observations in this contribution, we propose followings:

Observation 1: Common resource across cells for RRM measurement regardless of the type of numerology/service is beneficial for design simplicity, but may be resource-wasteful or may not provide sufficient accuracy of measurement quantity for all possible services.

Observation 2: Different resource for RRM measurement per numerology could be considered for further optimization from the resource efficiency and measurement accuracy perspectives.

Observation 3: RAN1/2 agreed that both SS and CSI-RS is used for RRM measurement and L3 mobility.

Observation 4: RAN1 is discussing about CSI-RS configuration which may comprise timing, bandwidth, numerology, or resource elements.

Observation 5: UE may not exactly measure interference of RSs from neighbor cells due to numerology mismatch.

Observation 6: There is no issue of numerology mismatch if UE measures neighboring cells only by common numerology/RS within common RRM resource across cells.

Observation 7: There may be some potential issues due to numerology mismatch when CSI-RS in different numerology from NR-SSS is used or different numerologies are used for CSI-RSs within neighboring cells.

Observation 8: To handle potential problems due to numerology mismatch, measurement resource restriction for LTE eICIC could be reused.

Proposal 1: To CONNECTED mode UE, measurement resource including time/frequency portion of the carrier may be indicated via RRC by the network for RRM measurement.

Proposal 2: RAN2 strives for RRM measurement of UE based on the same type of RS as well as the same numerology across cells based on the two options, SS-based (option 1) and CSI-RS-based (option 2).

Option1: Network does not configure UE with CSI-RS so that UE monitors only SS with the default numerology.

Option2: Network configures UE with CSI-RS and numerology so that UE monitors only CSI-RS with the configured numerology.

Proposal 3: To avoid numerology mismatch when CSI-RS is configured for RRM, UE is configured for measurement report to a CSI-RS with measurement resource pattern for monitoring serving cells and/or neighbor cells, with the cost of more specification efforts.

Proposal 4: Single numerology is associated to single measurement object which may have SS or a list of CSI-RS configuration.

Proposal 5: RAN2 considers further enhancement to LTE measurement restriction to handle numerology mismatch among cells.

References

[1] RP-170823, New WID Proposal: Work Item on New Radio (NR) Access Technology

[2] 3GPP TR 36.889 (2016-03)