3GPP TSG-RAN WG2 #99	     		R2-1708204

Berlin, Germany, 21st – 25th August 2017

Source: 	AT&T

Title:	Intra-frequency DC to enable mobility with close to zero ms interruption

Agenda Item:	10.2.8

Document for:	Discussion

# Introduction

In RAN2#96, it was discussed how to realize zero ms UP interruption in NR and the following agreements were made:

Agreements

1	The mobility enhancement similar to that discussed for LTE (“Maintaining Source eNB connection during handover”) should be considered also for NR.

2	For DC (NR-NR), study how to reconfigure the UE from an MeNB to an SeNB to target the 0 ms UP interruption. FFS whether also applicable to LTE-NR

Additionally, it was also agreed in RAN2#97 to allow possibility of intra-frequency dual connectivity/multi-connectivity:

=>	RAN2 protocols for NR should be flexible to allow the possibility of intra-freq DC/multiconnectivity.

Furthermore, based on LSs from RAN1 and RAN4, it is shown in [1] that NR-NR dual connectivity is feasible to accomplish intra-frequency mobility with close to zero ms interruption. This contribution further elaborates on the motivation for enabling intra-frequency dual connectivity for NR by providing some use cases and potential benefits of this feature.

# Intra-frequency DC for Mobility

For NR, the gNB may be functionally split between a centralised PDCP/RRC (gNB-CU) and decentralised RLC/MAC/PHY (gNB-DU) [2]. A new interface between gNB-CU and gNB-DU, called F1, is being specified by RAN3. Figure 1 below shows an example architecture where a UE could be supported by beams from two different DUs.

At mmWave frequencies, the channel experienced by a UE may suffer from blockage events that could result in sudden sharp drops in signal strength (of the order of 30 dB) due to physical objects blocking the UE-TRP link. It was shown in [3] based on Blockage Model A (UMi) described in the NR channel model TR 38.900, that for a 30 km/hr blocker velocity, the mean blockage duration of a single blockage event is 600 ms. It was also shown in [3] that when a UE could be served by beams from 3 TRPs, the mean duration between inter-TRP beam switches could be about 2.56 seconds. These estimates indicate that a UE could potentially experience frequent beam failure events due to blockage, potentially resulting in frequent beam switches.

.

Figure 1: NR deployment based on gNB UP functional split

For stationary or slow-moving UEs in a location served by beams from two TRPs belonging to two different DUs, such blockage events may result in frequent inter-DU mobility events. Mobility with close to zero ms interruption would be desirable to manage UP connectivity during such frequent inter-DU mobility events. Intra-frequency DC could serve well to accomplish mobility with close to zero ms interruption in such use cases as described below:

- Initially upon identification of the target cell, an SeNB addition procedure may be used to enable DC with the target cell (DU2 in the above example).

- Once DC is established between the MeNB and SeNB, UP connectivity may be managed as follows:

  - Dynamically determining the amount of data flow via each DC leg by the flow control algorithm residing at the CU depending upon the quality for beams between MeNB and SeNB.

  - When there is failure of all beam pair links from the MeNB or SeNB, data flow to the MeNB or SeNB may be suspended until there is restoration of beam quality on the failed leg (also done via PDCP flow control mechanism).

  - Upon such beam failure as described above, fast centralized retransmissions of lost RLC PDUs [6] may be performed via other DC leg to quickly recover from failed transmissions.

  - If no beams from the MeNB leg are capable of being restored, a role change between MeNB and SeNB as proposed in [4] may be required, followed by SeNB release.

As highlighted above, intra-frequency DC shows significant potential in many situations to reduce UP interruption time at mobility events. Thus, intra-frequency DC can be an effective way of enabling mobility with close to zero ms interruption.

Observation 1: Intra-frequency DC can be an effective means of enabling mobility with close to zero ms interruption.

# Future Considerations

RAN working groups are studying a feature called Integrated Access and Backhaul (IAB) [5] where NR nodes can be self-backhauled wirelessly to other NR nodes to facilitate easy deployment. For densely deployed mmWave NR systems, the area covered by an NR node can be quite small, so a dense deployment of NR nodes in a given area could require a significantly larger number of deployment sites. There is considerable interest in being able to deploy NR nodes without fiber access to every deployment site. Due to large available bandwidths in mmWave spectrum bands, and the ability to deploy narrow beams using multi-antenna element arrays, it may be more feasible to support self-backhauled links for NR systems operating in mmWave spectrum.

Figure 2 below shows an example scenario where IAB link (self-backhaul links) could be used to connect DU3 and DU4 to DU1, which has fiber access to the CU. Note that here the CU-DU split is assumed to be based on [2] and as described above in Section 2. In such a scenario, when the yellow highlighted UE moves from DU2 to DU3 to DU4, if intra-frequency dual connectivity or multi-connectivity is enabled for the UE, it could significantly help in reduction of UP interruption time to achieve mobility with close to zero ms UP interruption.

Figure 2: Integrated Access and Backhaul Scenario

Observation 2: Intra-frequency DC also seems to have the potential to offer reduction in UP interruption time for future deployment scenarios such as integrated access and backhaul.

In conclusion, based on significant potential benefits offered by intra-frequency DC and considering its feasibility as shown in [1], we propose that NR should support intra-frequency DC.

Proposal 1: NR should support intra-frequency dual connectivity.

# Conclusion

In this contribution, we discuss the potential for intra-frequency DC to enable mobility with zero ms UP interruption time. The following observations and proposal are offered for consideration:

Observation 1: Intra-frequency DC can be an effective means of enabling mobility with close to zero ms interruption.

Observation 2: Intra-frequency DC also seems to have the potential to offer reduction in UP interruption time for future deployment scenarios such as integrated access and backhaul.

Proposal 1: NR should support intra-frequency dual connectivity.

# Reference

- R2-1708880, DC for intra-frequency mobility in NR, Huawei, HiSilicon, AT&T

- TR 38.801, Study on New Radio Access Technology, Radio Access Architecture and Interfaces.

- R2-1704487, Beam Failure (BF) and Radio Link Interruption (RLI), AT&T

- R2-1706709, SgNB to MgNB reconfiguration for 0ms interruption handover, Huawei, HiSilicon

- RP-170831, Study on Integrated Access and Backhaul for NR, AT&T, Qualcomm, Samsung.

- R3-173235, Mechanism for fast retransmission of lost PDUs, Ericsson, AT&T