3GPP TSG-RAN WG2 Meeting NR #99		R2-1708254

Berlin, Germany, 21st – 25th August 2017                         	            Revision of R2-1706758

Agenda Item:	10.4.1.6.1

Source: 	MediaTek Inc.

Title:  	RAN-based Notification Area in NR

Document for:	Discussion and decision

# Introduction

In RAN2#98, we had some discussion on RAN area configurations, noted as follows.

=> Offline discussion to try to converge to a single solution (Nokia, offline discussion 46) - Update from offline: No consensus. Companies considered the area could be identified by either a cell list or a RAN area id list. Additionally a TA list was discussed but companies wanted more time => Can be discussed again at the next meeting

Although there was no consensus during previous meeting, it is clear that the RAN-based notification area could be identified by either a cell list or a RAN area id list. In this contribution, we discuss further details.

# Discussion

## Background

As background information, the following text about RAN-based notification area management is captured in the 3GPP TR 38.804 [1].

A UE in the RRC_INACTIVE state can be configured with the RAN-based notification area, whereupon: - a notification area can cover a single or multiple cells, and can be smaller than CN area; - a UE does not send any "location update" indication when it stays within the boundaries of the notification area; - leaving the area, a UE updates its location to the network. There are several different options on how the RAN-based notification area can be configured: - List of cells; - A UE is provided an explicit list of cells (one or more) that constitute the RAN-based notification area. - RAN area. - A UE is provided (at least one) RAN area ID; - A cell broadcasts (at least one) RAN area ID in the system information so that a UE knows which area the cell belongs to. NOTE 1: It will be decided in the work item phase whether to support both options, list of cells and RAN area ID, or only one of them. NOTE 2: A list with cells may contain only one entry implementing RAN-based notification area comprising one cell.

## Identification of RAN-based notification area

The RAN-based notification area could be identified by either a cell list or a RAN area ID (RAI) list. A cell list allows configuration at finer granularity (e.g., RAN-based notification area comprising one cell) but brings high overhead if the UE-specific list contains many cells. In contrast, RAI list is more efficient when the UE is to be paged in a larger RAN-based notification area. Considering different gNB deployment, UE distribution and mobility in NR, the size of RAN-based notification area varies a lot. Cell list can be used if the UE is considered quasi-stationary and there are only several cells in a RAN area, otherwise we use RAN area ID list.

Observation 1:	Cell list can be used if there are only several cells in a RAN area, otherwise RAN area ID list should be used.

Therefore, we suggest supporting both options for the identification of RAN-based notification area.

Proposal 1:	The RAN-based notification area could be identified by either a cell list or a RAN area id list. Both options are supported in NR.

## RAN area configurations

For CN-initiated paging, tracking areas are non-overlapping, and a UE-specific TA list consisting of multiple TAs may be configured. For RAN-initiated paging, since the operation is limited to RAN level, it is possible to make a cell belong to multiple RAN areas. This allows flexible RAN area configuration considering UE distributions in the network. Moreover, since a RAN area may contain only one cell, so the ‘cell list’ can be implemented as RAN area list, and we have a unified solution for the identification of RAN-based notification area, i.e. using RAN area ID list.

Proposal 2:	A cell may broadcast multiple RAN area IDs and thus belong to multiple RAN areas. In this way, cell list can be implemented as RAN area ID list.

# Conclusion

We have the following observation:

Observation 1:	Cell list can be used if there are only several cells in a RAN area, otherwise RAN area ID list should be used.

It is proposed to discuss and decide on the following proposals:

Proposal 1:	The RAN-based notification area could be identified by either a cell list or a RAN area id list. Both options are supported in NR.

Proposal 2:	A cell may broadcast multiple RAN area IDs and thus belong to multiple RAN areas. In this way, cell list can be implemented as RAN area ID list.

# Reference

- TR 38.804, Study on New Radio Access Technology; Radio Interface Protocol Aspects, Release 14