3GPP TSG-RAN WG2 #99	R2-1708342

Berlin, Germany, 21st – 25th August 2017

Agenda Item:	10.3.2.5

Source:	Ericsson

Title:	RLC AM Polling mechanism

Document for:	Discussion, Decision

# Introduction

RAN2-98 agreed:

Agreements:

- RLC polling mechanisms are the same as in the LTE.   No additional enhancements are needed.

This contribution discusses about PDU_WITHOUT_POLL and BYTE_WITHOUT_POLL counters related to polling.

# Discussion

RLC Polling is used by RLC transmitter to request status report from receiving RLC entity. In LTE, there are two counters associated with the polling which are both counting transmitted data after last poll bit was transmitted. One of the counters is PDU_WITHOUT_POLL which keeps track of transmitted PDUs and another is BYTE_WITHOUT_POLL, which keeps track of transmitted bytes.

RAN2 agreed to have one SN per SDU in RLC AM. Impact of the change to transmission/receive window is that the receiver window moves slower as no individual SNs are assigned for SDU segments like in LTE. The question is whether this has impact on PDU_WITHOUT_POLL and BYTE_WITHOT_POLL counters.

PDU_WITHOUT_POLL counter is incremented for every received PDU, whether it is segment or not. There is no direct association between SDU SN and PDU_WITHOUT_POLL on transmitter side. On receiver side, when SDU segment with P-bit is received, the receiver waits until all PDUs associated with the PDU are received before initiating status report. Alternatively, status report is triggered by T-reassembly/reordering timer.

BYTE_WITHOUT_POLL is incremented based on received bytes. The purpose of this counter is to prevent window stalling when large PDUs are transmitted. Like PDU_WITHOUT_POLL, there is no problem to keep the LTE behaviour on transmitter side. From receiver perspective, the behaviour is same as PDU_WITHOUT_POLL as the receiver only observes that poll-bit was set to one.

LTE behaviour is adopted for counters: PDU_WITHOUT_POLL and BYTE_WITHOUT_POLL.

Text proposal [1] is adopted

# Conclusion

Based on the discussion in section 2 we propose the following:

Proposal 1	LTE behaviour is adopted for counters: PDU_WITHOUT_POLL and BYTE_WITHOUT_POLL.

Proposal 2	Text proposal [1] is adopted

# References

R2-1708341. RLC Polling TP. Ericsson