3GPP TSG-RAN WG2 #99	Tdoc R2-1708350

Berlin, Germany, 21-25 August 2017

Updated from R2-1707174

Agenda Item:	10.3.1.8

Source:	Ericsson

Title:	Grant Free and Semi-Persistent Scheduling in NR

Document for:	Discussion, Decision

# Introduction

In RAN2#97, the following was agreed for semi-persistent scheduling (SPS):

- NR supports an SPS scheme similar to LTE - NR supports skipping UL grant scheme similar to LTE

In RAN2#97bis, the following agreements were reached:

Agreements on grant-free

=>	From RAN2 point of view it would be beneficial to be able to share “SPS/grant free” UL resources amongst different UE.  Mechanism to identify the UE for collision resolution purpose may be needed.   The details can be discussed in RAN1.

Agreements on SPS:

-	Like in legacy LTE, at least SPS period is configured by RRC.  FFS how frequency resources, MCS, etc., for SPS are provided to the UE depends on RAN1 discussion.

-	UL skipping for dynamic grant should be configurable.  FFS if UL skipping for SPS is configurable

-	Working assumption:  Like in LTE, DRX behaviour with SPS UL should be to restart inactivity timer when UL data is transmitted, and not to restart when SPS UL grant is not used.  This behaviour depends on outcome of DRX design.

In RAN2#98, the following agreements were reached:

Agreements

1.	In NR, when the UE is configured with SPS, the UE should always skip SPS grant if there is no data to transmit, i.e., Skipping SPS grant is mandated in NR regardless of SPS periodicity.

2.	LCP is performed the same regardless whether the grant is dynamic or SPS.  SPS is a “configured grant”.

3.	FFS is multiple SPS is supported for duplication or to support different numerologies

4.	Implicit release of UL SPS resources is not supported

Additionally, in RAN2#98, RAN2 received an LS from RAN1 informing on agreements w.r.t Grant Free [1]:

Agreements:

- If network configures, UL data transmission without UL grant can be performed after semi-static resource configuration in RRC without L1 signalling

- If network configures, L1 signaling for activation/deactivation and/or modification on parameters for UL data transmission without UL grant can be applied

- RAN1 is discussing whether the mechanism to distinguish UL SPS and UL data transmission without UL grant is necessary.

In RAN2-98AH,  there are the following agreement regarding modelling of SPS and grant free transmission:

Agreements

=>	Modelling in the MAC for grant-free will be discussed after the difference between the two schemes is better understood pending RAN1 progress.  RAN2 will aim to have a unified MAC operation for common functionalities between grant-free and UL SPS with understanding that there can be differences after input from RAN1.

=>	RAN2 understands that to support UL SPS similar to LTE a mode of operation in which RRC configuration (with no initial PHY resources) with L1 activation/deactivation needs to be supported.  RAN2 will continue discussion on UL SPS, with LTE functionality.

=>	A common RRC signalling can be design to allow the configuration of different UL transmissions schemes.

In RAN1-89AH, there are the following agreements

Agreements:

- Type of UL data transmission without grant

  - Type 1: UL data transmission without grant is only based on RRC (re)configuration without any L1 signalling

  - Type 2: UL data transmission without grant is based on both RRC configuration and L1 signalling to activation/deactivation for UL data transmission without grant

    - Note: functionality of modification is achieved the L1 signalling by activation

  - Type 3: UL data transmission without grant is based on RRC configuration, and allows L1 signalling to modify some parameters configured by RRC but no L1 signalling for activation

- For UL data transmission without grant, type 1 and type 2 have already been agreed, FFS type 3.

- FFS the reliability issues for L1 signalling.

- For Type 1 UL transmission without UL grant, the RRC (re-)configuration includes at least the following

  - Periodicity and offset of a resource with respect to SFN=0

  - Time domain resource allocation

  - Frequency domain resource allocation

  - UE-specific DMRS configuration

  - Note:

    - one TB is mapped to a resource at least consisting of time/frequency-domain resource

    - RAN1 will not introduce specific resource allocation and DMRS configuration for UL data transmission without grant separate from UL data transmission with UL grant within the Rel.15 WI

  - An MCS/TBS value

  - Number of repetitions K

  - Power control related parameters

  - FFS HARQ related parameters

  - FFS if multiple resources can be configured

- For Type 2 UL transmission without UL grant

  - The RRC (re-) configuration for resource and parameters includes at least the following

    - Periodicity of a resource

    - Power control related parameters

  - At least the following additional parameters for the resource are given by L1 signalling

    - Offset associated with the periodicity with respect to a timing reference indicated by L1 signalling for activation

      - FFS: the timing reference

    - Time domain resource allocation

    - Frequency domain resource allocation

    - UE-specific DMRS configuration

    - An MCS/TBS value

    - Note:

      - one TB is mapped to one resource

      - RAN1 will not introduce specific resource allocation and DMRS configuration for UL data transmission without grant separate from UL data transmission with UL grant within the Rel.15 WI

  - FFS multiple resources can be configured

  - FFS HARQ related parameters

  - FFS whether number of repetitions K is configured by RRC signalling and/or indicated by L1 signalling

This paper investigates the modelling of SPS mechanism and grant free mechanism in NR. It is concluded that RAN2 should maximize the commonality of two mechanisms to avoid unnecessary complexity for both standardization and implementation without clear benefit.

# Discussion

SPS in LTE and NR is mainly used to configure the UE semi-statically with a periodic resource allocation which ideally matches a periodic traffic pattern, where periods of approximately 20ms are typical, or for URLLC type of services where the resource request procedure(s) incur an unwanted latency in receiving an UL grant.

In LTE, and in NR, “UL skipping” is used as for uplink scheduling as well as SPS-based scheduling. Thereby, the mandate for the UE to always utilize an UL grant – even if no UL data is available by a padding transmission – is removed. This way, the UE skips UL padding transmissions. The motivation for this feature on SPS was that SPS intervals were reduced to very short intervals, and that too frequent redundant padding transmissions are thereby avoided.

It becomes evident that in NR, the SPS framework will be used for these two distinct use cases: low-latency uplink access on the one side with a minimum periodicity, and VoIP-like periodic traffic with relaxed periodicities of about 20ms.

As in LTE, some of the parameters that need to be configured for semi-persistent scheduling in LTE are periodicity, address/ID (e.g., SPS-C-RNTI), resource allocation and MCS to be used in the UL grant. For SPS, part of these parameters (periodicity, address) are configured by RRC (SPS Configuration), and the remaining configured by PDCCH (SPS Activation/Deactivation). This allows for flexibility of the resource allocation and link adaptation, since a new configuration message can be sent in any PDCCH. It can be assumed that in general a similar configuration is used for NR SPS and Grant Free.

Like in legacy LTE, SPS period and RNTI are configured by RRC.

Like in legacy LTE, frequency resources, MCS, etc., for SPS are semi-statically configured

The RAN1 agreements [1] for “UL data transmission without UL grant” entail that additionally, a configured grant, optionally “without L1 signalling”, can be given to the UE; for where a “semi-static resource configuration” can be given. This is suggested to be set by RRC signalling.

In addition to the above, is also agreed that “If network configures, L1 signaling for activation/deactivation and/or modification on parameters for UL data transmission without UL grant can be applied”.

From a layer 2 p.o.v the above fit directly with the existing concept of “configured grants” in MAC, and especially, as an extension of legacy SPS with the additional add-on of providing also the L1 related information on MCS, resource allocation etc., in the SPS RRC configuration.

“Grant free transmission” is defined as an extension of the current SPS framework and is modelled as a part of configurable options of the NR SPS framework.

In SPS, “UL skipping” comes with the need for a procedure to confirm the activation/deactivation.

Also for NR SPS, including the extension of a “Grant Free” mode, optional activation/deactivation and/or modification and to avoid error cases, a confirmation message acknowledging the UL resource L1 activation/deactivation or modification signalling should be sent to the gNB, following legacy LTE and agreed NR baseline.

Note that, for an RRC configuration activated without L1 control, the need for a confirmation message on the SPS resource should be discussed. This could be made with a MAC CE similar to LTE or rely on the completion of the RRC procedure or a combination thereof. This should however studied further, as the details are still unclear.

In NR, a RRC_connected UE can be either in active or inactive state. For a UE in inactive state, the RRC context is kept in both UE and network; and the network/UE can reactivate the context (such as C-RNTI and DRB etc) when there comes a service for the UE. However, there is no benefit to keep the allocated radio resources for an UE when the UE is in active state be still effective after the UE enters inactive state. There are the following disadvantages to allow an inactive UE to continue access the allocated resources during the UE in active state:

- The traffic status of an inactive UE is not known at the network, which means possible waste of radio resources.

- The network loses control of the allocated resources:

  - For congestion control when there is high traffic load from inactive UEs sharing the resources

  - For resource reallocation: e.g. adjustment of granted resource size, or reallocation of the allocated resource for other purposes.

Considering the infeasibility mentioned above, we propose:

The allocated resources for SPS/grant free transmission are released when a UE enters to RRC_inactive state from RRC_active state.

# Conclusion

Based on the discussion in section 2 we make the following observations and propose the following:

- Like in legacy LTE, SPS period and RNTI are configured by RRC.

Like in legacy LTE, frequency resources, MCS, etc., for SPS are semi-statically configured.

Based the discussion in Section 2, we have the following proposals:

Proposal 1	“Grant free transmission” is defined as an extension of the current SPS framework and is modelled as a part of configurable options of the NR SPS framework.

Proposal 2	The allocated resources for SPS/grant free transmission are released when a UE enters to RRC_inactive state from RRC_active state.

# References

R2-1706103	 LS on UL data transmission without UL grant for NR, RAN1.