3GPP TSG-RAN WG2 #99	R2-1709243

Berlin, Germany, 21 - 25 August 2017

Agenda item:	10.2.4

Source:	Nokia, Nokia Shanghai Bell

Title:	Format of measurement results between MN and SN

WID/SID:	NR_newRAT-Core - Release 15

Document for:	Discussion and Decision

# 1	Introduction

In the 3GPP RAN2 NR Adhoc meeting in Qingdao, the following agreements were made during a discussion on the email discussion summary report in [1] related to the SCG cell related parameters (at least for EN-DC):

Agreements related to SCG cell related parameters (at least for EN-DC)

1	RAN2 confirm that MN only initiates SN addition/ release i.e. that MN initiated SCG addition/ release is not supported

2.1	(At SN addition) MN provides measurement results rather than explicitly indicating the SCG cell to be added

2.2	No further SCG cell related parameters (beyond the measurements) need to be exchanged (i.e. no need for inter-node signalling regarding SCG cell addition assuming UE capability related info is indicated differently)

3	Measurement results provided by MN to SN at SN addition are specified by RRC (inter node message). FFS whether encoding of measurements is defined in NR or LTE RRC.

It was discussed and agreed that the MN and SN will exchange inter-node RRC messages to share measurement results provided by MN to SN for the SN addition scenario. However, the protocol formatting aspects was not concluded. In this contribution, we provide a short analysis for the same.

# 2	Background

In Figure 1 the sequence of steps for EN-DC are described. The Secondary Node Addition procedure is initiated by the MeNB and is used to establish a UE context at the SgNB to provide radio resources from the SgNB to the UE. This procedure is used to add at least the first cell (PSCell) of the SCG. Figure 10.2.1-1 shows the Secondary Node Addition procedure. Based on the description in [2], the sequence of steps is as follows:

The MeNB decides to request the SgNB to allocate radio resources for a specific E-RAB, indicating E-RAB characteristics (E-RAB parameters, TNL address information corresponding to bearer type). In addition, MeNB indicates the requested SCG configuration information, including the entire UE capabilities and the UE capability coordination result. The MeNB provides the latest measurement results for SgNB to choose and configure the SCG cell(s). The MeNB may request the SgNB to allocate radio resources for MCG split SRB operation. Addition of SRB3 is decided by the SN. In case of SCG Split bearer, the MeNB provides X2 DL TNL address information for the respective E-RAB and also the maximum QoS level that it can support. The SgNB may reject the request.

Figure 1: Secondary Node Addition procedure (section 10.2.1-1 in [2])

SCG-ConfigInfo-r12-IEs ::=			SEQUENCE {

radioResourceConfigDedMCG-r12	RadioResourceConfigDedicated		OPTIONAL,

sCellToAddModListMCG-r12		SCellToAddModList-r10				OPTIONAL,

measGapConfig-r12				MeasGapConfig						OPTIONAL,

powerCoordinationInfo-r12		PowerCoordinationInfo-r12			OPTIONAL,

scg-RadioConfig-r12				SCG-ConfigPartSCG-r12				OPTIONAL,

eutra-CapabilityInfo-r12		OCTET STRING (CONTAINING UECapabilityInformation)	OPTIONAL,

scg-ConfigRestrictInfo-r12		SCG-ConfigRestrictInfo-r12			OPTIONAL,

mbmsInterestIndication-r12		OCTET STRING (CONTAINING

MBMSInterestIndication-r11)		OPTIONAL,

measResultServCellListSCG-r12	MeasResultServCellListSCG-r12		OPTIONAL,

drb-ToAddModListSCG-r12			DRB-InfoListSCG-r12					OPTIONAL,

drb-ToReleaseListSCG-r12		DRB-ToReleaseList					OPTIONAL,

sCellToAddModListSCG-r12		SCellToAddModListSCG-r12			OPTIONAL,

sCellToReleaseListSCG-r12		SCellToReleaseList-r10				OPTIONAL,

p-Max-r12						P-Max								OPTIONAL,

nonCriticalExtension			SCG-ConfigInfo-v1310-IEs			OPTIONAL

}

It was agreed in the same meeting that the election of the PSCell is the SN’s responsibility and that is the reason why the MN needs to provide measurement results rather than explicitly indicating the SCG cell to be added. This means that the SCG-ConfigInfo does not need to carry neither the “sCellToAddModListSCG” nor the “sCellToReleaseListSCG” IE below. Instead, following the “measResultListEUTRA” IE seems to be the right way to proceed in this matter.

MeasResultListEUTRA ::=				SEQUENCE (SIZE (1..maxCellReport)) OF MeasResultEUTRA

MeasResultEUTRA ::=	SEQUENCE {

physCellId							PhysCellId,

cgi-Info							SEQUENCE {

cellGlobalId						CellGlobalIdEUTRA,

trackingAreaCode					TrackingAreaCode,

plmn-IdentityList					PLMN-IdentityList2 				OPTIONAL

}			 												OPTIONAL,

measResult							SEQUENCE {

rsrpResult							RSRP-Range						OPTIONAL,

rsrqResult							RSRQ-Range						OPTIONAL,

...,

[[	additionalSI-Info-r9				AdditionalSI-Info-r9		OPTIONAL

]],

[[	primaryPLMN-Suitable-r12			ENUMERATED {true}			OPTIONAL,

measResult-v1250					RSRQ-Range-v1250			OPTIONAL

]],

[[	rs-sinr-Result-r13					RS-SINR-Range-r13			OPTIONAL,

cgi-Info-v1310						SEQUENCE {

freqBandIndicator-r13				FreqBandIndicator-r11		OPTIONAL,

multiBandInfoList-r13				MultiBandInfoList-r11		OPTIONAL,

freqBandIndicatorPriority-r13		ENUMERATED {true}			OPTIONAL

}																OPTIONAL

]]

}

}

In addition, the ARFCN of the SN frequency will also be needed.

-- ASN1START

ARFCN-ValueEUTRA ::=				INTEGER (0..maxEARFCN)

ARFCN-ValueEUTRA-v9e0 ::=			INTEGER (maxEARFCN-Plus1..maxEARFCN2)

ARFCN-ValueEUTRA-r9 ::=				INTEGER (0..maxEARFCN2)

-- ASN1STOP

Proposal 1: Measurement results provided by MN to SN at SN addition carry at least the ARFCN value and PCI’s of cells in the SN with a quality indicator (RSRP, RSRQ or equivalent).

The only question that remains is the format that needs to be followed i.e. whether the encoding follows MN or SN RRC principles. There are 3 possible options;

- Option 1:  encoding follows a “common” format known to both the RRC specifications

- Option 2: the source encodes the measurement results based on the target RRC specification (assuming that the measurement was setup by the source MN, so this is a different case compared to the scenario where the MN forwards measurement report/results to the SN when the SN did setup the measurement towards the UE)

- Option 3: a part of the measurement results follows option 1 and the rest follows option 2

Let’s take a simple example to see which option may be favourable. For SN addition, we already know that it is the MN which will setup an inter-RAT event (B1 for EN-DC). Even though we haven’t discussed the details of the measurement report corresponding to the B1 measurement event, it is foreseen that it will contain the PCI of the measured cells with some quality indicator e.g. RSRP, RSRQ. For NR, we foresee that some beam quality information will be also needed in case the NR HF cells are involved. While a common format makes sense for cell and cell quality information that has a common basis to both the RATs, information like beam quality might just be very specific to only NR (unless someone plans to introduce beam sweeping for LTE PHY). To support the latter, the format needs to be in Option 2. Hence it seems Option 3 kind of measurement result formatting is a good way forward.

Proposal 2: Formatting of measurement results provided by MN to SN follows encoding a part of the results in a common format understandable by both the MN and SN RRC and another (optional) part which is encoded in the SN RRC format.

# 3	Conclusion

In this contribution, we have discussed several options of formatting the measurement results to be delivered to the SN. We also provided the protocol formatting aspects for the measurement results by presenting different options concluding that sharing of the results must follow a common part and an optional part which is based on SN RRC. Based on the discussions we propose:

Proposal 1: Measurement results provided by MN to SN at SN addition carry at least the ARFCN value and PCI’s of cells in the SN with a quality indicator (RSRP, RSRQ or equivalent).

Proposal 2: Formatting of measurement results provided by MN to SN follows encoding a part of the results in a common format understandable by both the MN and SN RRC and another (optional) part which is encoded in the SN RRC format.

# References

- R2-1707209: Summary of email discussion [98#26][NR] SN/MN information exchange, Samsung Telecommunications

- R2-1706858: Allowing mapping of default bearer on SCG during Initial Context Setup, Nokia, Alcatel-Lucent Shanghai Bell