3GPP TSG-RAN WG2 #99	R2-1709870

Berlin, Germany, 21st – 25th August 2017

Agenda Item:	10.2.15

Source:	Ericsson

Title:	TP on Radio Link Failure for 38.300

Document for:	Discussion, Decision

# Text Proposal

### 9.2.7		Radio Link Failure

In RRC_CONNECTED, the UE declares Radio Link Failure (RLF) when one of the following criteria are met:

-	Expiry of a timer started after indication of radio problems from the physical layer. If radio problems are recovered before the timer is expired, the UE stops the timer.

-	Random access procedure failure;

-	RLC failure

FFS Whether indications related to beam failure recovery may affect the declaration of RLF.

After RLF is declared, the UE:

-	stays in RRC_CONNECTED;

-	selects a suitable cell and then initiates RRC re-establishment;

-	enters RRC_IDLE if a suitable cell wasn't found within a certain time after RLF was declared.

In DC, RLF is declared separately for the MCG and for the SCG. The actions following RLF described above only apply for RLF on the MCG. After RLF on the SCG, the UE stops normal operation on the SCG and reports the failure to the network.