3GPP TSG-RAN WG2 #99	Tdoc R2-1708323

Berlin, Germany, 21st – 25th August 2017

Agenda Item:	10.3.4.3

Source:	Ericsson

Title:	SDAP entity establishment

Document for:	Discussion, Decision

# Introduction

RAN2-97bis Spokane agreed:

Agreements 1 NR/NR DC should support that different QoS flows of the same PDU session can be mapped to MgNB and SgNB. 2 In the case of NR/NR DC where different QoS flows of the same PDU session are mapped to MgNB and SgNB then there is one SDAP entity in the MgNB and one in SgNB for that PDU session. RAN2 understand that support of this behaviour is still under discussion on SA2

This contribution discusses about SDAP entity establishment for one gNB and for DC scenarios.

# Discussion

RAN2-AH in January agreed that there is one SDAP entity per PDU session. RAN2-97bis made clarification to the agreement that two SDAP entities may be established in DC scenarios. This contribution discusses about different scenarios in SDAP entity establishment.

When PDU session is not split between MN and SN, there is no problem to identify SDAP entity. The SDAP entity may be established in PDU session establishment and the SDAP can be identified with PDU session ID.

Having PDU session split between MN and SN results that there are two SDAP entities, one in SN and another in MN. Even when the PDU session is split, same PDU session ID is still used. This results that both SDAP entities could have conflict in UE if the PDU session Id is used.

When PDU session is split between MgNB and SgNB, same PDU session ID is used

Having same PDU session between SDAP entities should not be a problem as it is expected that the corresponding cell group configures its SDAP entity, resulting that SDAP entity configurations are received separately. From the configuration source, UE may identify and assign ID for SDAP entity if needed.

In LTE, when PDN connection is split between MN and SN, the DRB IDs are unique between MN and SN. Having both, DRB and Flow unique inside same PDU session allows UE to have one mapping table, from which it can perform flow to DRB mapping. Alternatively, having DRB ID unique inside SDAP entity would require two level mapping between flow SDAP and DRB. The solution adds complexity and it is preferable to keep LTE behaviour with unique DRB inside same PDU session.

DRB IDs inside same PDU session shall be unique.

UE may establish SDAP entity ID when SDAP entity is configured. Method to allocate SDAP entity ID is left for UE implementation.

# Conclusion

In section 2 we made the following observations:

Observation 1	When PDU session is split between MgNB and SgNB, same PDU session ID is used

Based on the discussion in section 2 we propose the following:

Proposal 1	DRB IDs inside same PDU session are unique

Proposal 2	UE may establish SDAP entity ID when SDAP entity is configured. Method to allocate SDAP entity ID is left for UE implementation.

# References

R2-1708320. QoS Flow Relocation in NR-DC between MN and SN. Ericsson.  Berlin, Germany, 21st – 25th August 2017.