3GPP TSG-RAN WG2 #99	Tdoc R2-1708325

Berlin, Germany, 21st – 25th August 2017

Agenda Item:	10.3.4.3

Source:	Ericsson

Title:	SDAP configurations with NAS-Reflective QoS

Document for:	Discussion, Decision

# Introduction

Table 1	SA2 3GPP TS 23.501 V1.2.0

5.7.1	General Overview

Following characteristics apply for processing of uplink traffic:

- (R)AN transmits the PDUs over N3 tunnel towards UPF. When passing an UL packet from (R)AN to CN, the (R)AN includes the QFI value, in the encapsulation header of the UL PDU, and selects the N3 tunnel.

- (R)AN performs transport level packet marking in the uplink, transport level packet marking may be based on the 5QI and ARP of the associated QoS Flow.

5.7.5.3	UPF Procedures for Supporting Reflective QoS

UPF may include the Reflective QoS Indication (RQI) in the encapsulation header on N3 reference point together with the QFI.

5.7.5.4	Reflective QoS Control

When an RQI is received by (R)AN in a DL packet on N3 reference point, the (R)AN shall indicate to the UE the QFI and that this DL packet is subject to Reflective QoS.

This contribution discusses about open issues related to NAS-level reflective QoS and SDAP configurations.

# Discussion

## SDAP configuration and N3 marking with multiple Flows per PDU session

When multiple flows are configured per one PDU session, there is a need to differentiate traffic between flows. For this purpose, SA2 agreed all UL flows are marked with QFI in N3 header. For a gNB, this results that all packets destined to CN need to be marked. The marking from the gNB perspective is trivial when there is only one flow in PDU session or all flows in PDU session are mapped into separate DRBs. However, when multiple flows exist on one PDU session and more than one flow is mapped to a same DRB, gNB needs to be able to distinguish traffic between flows. This is only possible if UL packets are marked from UE with the QFI information. This case may occur even when NAS-reflective QoS is not active. For this reason, the SDAP header configuration should be separate for UL and DL.

Even when there are multiple QFIs per PDU session, it is not necessary for gNB to mark QFI for DL packets if no AS reflective QoS is configured. gNB may use the QFI information provided in N3 header to make decisions on scheduling. This way gNB is able to guarantee per-flow treatment, which is enough to full fill QoS requirements over the air. Detailed RRC configuration is discussed in [2].

Existence SDAP header is configured separately for UL and for DL

## SDAP configuration with NAS-Reflective QoS configurations

According to SA2, a PDU session may have flows where NAS-reflective QoS is disabled and flows where the mechanism is active. When NAS-reflective QoS is active, the N3 header may have RQI indication in addition to QFI, which needs to be carried for UE, if the UE has support for reflective QoS. In the case where UE lacks support for NAS-reflective QoS it may ignore the reflective QoS configurations, therefore, the SDAP header is not needed for purpose of NAS reflective QoS if UE does not support NAS reflective QoS.

When NAS reflective QoS is configured for QFI, the RQA parameter is signalled via N2. If the RQA does not exist for QFI, the QFI is not reflective. This results that no SDAP header is needed in this case.

SDAP header is always included for DL and UL Flows when NAS reflective QoS is activated by 5CN. Activation of NAS-Reflective QoS is done by signalling RQA via N2.

RQA indicates whether QFI is reflective

gNB attaches the QFI for the DL packets marked with RQI.

# Conclusion

In section 2 we made the following observations:

Observation 1	SDAP header is always included for DL and UL Flows when NAS reflective QoS is activated by 5CN. Activation of NAS-Reflective QoS is done by signalling RQA via N2.

Observation 2	RQA indicates whether QFI is reflective

Based on the discussion in section 2 we propose the following:

Proposal 1	Existence SDAP header is configured separately for UL and for DL

Proposal 2	gNB attaches the QFI for the DL packets marked with RQI.

# References

System Architecture for the 5G System. 3GPP TS 23.501. V1.2.0 (2017-07)

R2-1709377, SDAP configuration aspects. Ericsson.  Berlin, Germany, 21st – 25th August 2017