3GPP TSG-RAN WG2 #99	R2-1708343

Berlin, Germany, 21st – 25th August 2017

Agenda Item:	10.3.2.2

Source:	Ericsson

Title:	RLC UM header format

Document for:	Discussion, Decision

# Introduction

In contribution, we propose PDU format for RLC UM mode.

Previous agreements related to RLC:

RAN2 June ad-hoc (Qingdao):

Agreeements

- 12 and 18 bit RLC SN is used for RLC AM NR.  SN for RLC UM is FFS

- Only 16 bit SO is used in NR for both AM and UM.

- NACK_RANGE size is FFS – depends on byte alignment and full RLC STATUS format

Agreements

1.	RLC UM without SN for the complete SDU is selected.   FFS which options is selected

2.	SI field is included in RLC UM header to differentiate complete RLC SDU, the first SDU segment, the middle SDU segment, and the last SDU segment.

-	The header of unsegmented SDU contains only SI field.

-	The header of first segment contains only SI field and SN.

-	The header of middle and last segment contains SI field, SN, and SO.

RAN2#98 (Hangzhou)

Agreements:

- The SO field should be included in the last SDU segment as well as the middle SDU segment.

- One SO size is supported for NR.  SO-field should be able to indicate the largest PDCP PDU size we intend to support.

RAN2#97bis (Spokane)

Agreements RLC segmentation: - As a baseline, segmentation is always enabled for RLC-AM and RLC-UM. FFS if there are cases in which it is beneficial to disable segmentation - An RLC SDU for UM and AM can be associated with only one RLC SN, i.e., the byte segments from an RLC SDU can be associated with the same RLC SN. - Segmentation and re-segmentation is based on RLC SDU, i.e., SO field indicates byte position of the RLC SDU - RLC header is to be designed in following principles: - RLC header indicates if RLC PDU carries a complete RLC SDU or RLC SDU segments. - RLC header does not include SO field if RLC PDU carries a complete RLC SDU. - RLC header does not include SO field when the beginning of the RLC SDU is segmented. - RLC header includes SO field when the middle or end of the RLC SDU is segmented. - RLC header indicates whether the RLC PDU contains the end part of RLC SDU segment or not when the middle or end of the RLC SDU is segmented.

Agreements on RLC PDU format

-	NR RLC PDU and NR RLC PDU header should be byte-aligned.

-	NR RLC PDU will not include Length Indicator (LI) field.

-	NR RLC PDU will use 2-bits “FI-like” field to distinguish the complete RLC SDU, the first SDU segment, the middle SDU segment and the last SDU segment and SO field is needed just in the case of the middle SDU segment or the last SDU segment.

-	NR RLC TMD PDU only consists of a Data field and does not consist of any RLC headers.

-  FFS if NR RLC UMD SDU should not include SN field and only NR RLC UMD SDU segment should carry SN field

-	NR RLC AMD PDU includes a 1-bit D/C field, 1-bit P field

-	RLC status report format is byte-aligned

-	Introduce a new field(s) “NACK SN range” in the status report format. FFS the details of this field and how the status report is coded

# Discussion

As discussed in [1] there are at least two different options on how the SNs could be assigned: 1) Every RLC SDU is associated with SN but only PDUs containing SDU segments have SN in the header or 2) Only SDU segments have SN associated in the first place. When it comes to the header design, the selection strategy does not matter. What it affects, however, is the length of the SN space, i.e., how many bits are allocated for the SN field in the RLC UM headers.

However, regardless of how the SN is assigned, it seems questionable if a 6-bit SN will be enough. Even if SN is only used in case 2) above, there will not be enough SN to support for high data rates with short TTI, carrier aggregation, and so on. If λRLC is the rate of arrival of RLC SDUs to be segmented and N is the number of bits used for RLC UM SN and we assume all offered load can be handled by the radio link, then the wrap-around time for SN can be calculated as:

twrap=2NλRLC

In the worst-case scenario, we need to consider all SDUs are segmented into two SDU segments. As an example let us assume a HD video stream of 5000 kbps where the RLC SDU size is 10000 bits and scheduler provides TBS of 5000 bits ( ≅ RLC PDU size). Table 1 provides wrap-around times for this example. We see that, in this case, short SN lengths of under 10 bits would result in SN wrap-around of less than one second. In particular, 6 bits would wrap in 128 ms, which could easily be within the QoS requirement of such streaming application.

Table 1. t_wrap for the example case with different SN lengths

| SN length | 5 | 6 | 7 | 8 | 10 | 12 | 14 | 16 | 18 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| twrap [ms] | 64 | 128 | 256 | 512 | 2048 | 8192 | 32768 | 131072 | 524288 |

6-bit SN for RLC UM SDU segments is too short

When the SN > 6 bits, then the header size will be a least 2 bytes when accounting for the FI-like field which takes 2 bits.

When SN is >6 bits the header size is at least 2 bytes for the first SDU segment

Up to 14 bits SN could be supported with 2 byte header for the first SDU segment

As up to 14 bits can be supported with 2 byte header, we propose to use 12-bit SN as is used in PDCP. We don’t think anything less than 12 bits should be used, as that may lead to too quick depletion of the SN numbers and a possible wrap-around. As for all SN > 6 there would be two bytes header, there should be no issues in having 12-bit SNs for RLC UM.

As an option, by increasing the header size to 3 bytes, also 18-bit SN chould be supported for use cases requiring larger SN space. This option would have the benefit of similarity to PDCP SN spaces.

Use 12-bit SN for RLC UM PDUs.

Additionally 18-bit SN is supported for RLC UM PDUs.

Capture the header formats presented below in 38.322.

Figure 1. RLC UM PDU format for segmented SDU with 18-bit SN field.

Figure 2. RLC UM PDU format for complete SDU or first SDU segment with 18-bit SN field.

Figure 3. RLC UM PDU format for segmented SDU with 12-bit SN field.

Figure 4. RLC UM PDU format for first SDU segment with 12-bit SN field.

Figure 4. RLC UM PDU format for complete SDU.

# Conclusion

In section 2 we made the following observations:

Observation 1	6-bit SN for RLC UM SDU segments is too short

Observation 2	When SN is >6 bits the header size is at least 2 bytes for the first SDU segment

Observation 3	Up to 14 bits SN could be supported with 2 byte header for the first SDU segment

Based on the discussion in section 2 we propose the following:

Proposal 1	Use 12-bit SN for RLC UM PDUs.

Proposal 2	Additionally 18-bit SN is supported for RLC UM PDUs.

Proposal 3	Capture the header formats presented below in 38.322.

# References

R2-1708338, “Further details of RLC UM operation”, Ericsson, RAN2#99, Berlin, Germany