3GPP TSG-RAN WG2 Meeting#99		R2-1708490

Berlin, Germany, 21st – 25th August 2017

Source:	vivo

Title:	BSR and SR and BSR cancel due to deactivation of LCH

Agenda Item:	10.3.1.6

Document for:	Discussion and Decision

- Introduction

In RAN2 NR AH meeting hold Jan 2017, LCP aspects in NR was discussed and the following agreement was made [1]

1: a single logical channel can be mapped to one or more numerology/TTI duration. 8: LCP takes into account the mapping of logical channel to one or more numerology/TTI duration. Details of LCP will be discussed in the WI phase

In this contribution, we further discuss LCH can be deactivated due to the LCP restriction and the BSR and SR cannel due to deactivation of LCH.

- Discussion

  - Deactivation of LCH

In NR the LCP restriction is extended due to diversity numerology and the introduction of PDCP duplication, there are two cases the LCH can be deactivated due to LCP restriction. Like shown as below figure 1, there are three cells, LCH1 can be mapped on BWP1 of cell 1 with numerology1. LCH2 and LCH3 can be mapped on BWP2 and BWP3 of cell 2/3 with numerology2. LCH4 can be mapped on BWP4 of cell 4 with numerology3. The BWP3 and BWP4 cannot be activated at same time in R15.

Case 1: all the cells of a LCH that it can be mapped to are deactivated

If the cell 3 is deactivated, the LCH 4 cannot be used to sending data due to LCP restriction in case of no LCP restriction reconfiguration.

Case 2: all the BWPs of a LCH that it can be mapped to are deactivated

If the both BWP 2 and BWP 3 are deactivated, the LCH 2 and LCH3 cannot be used to sending data due to LCP restriction in case of no LCP restriction reconfiguration.

Figure 1: LCH, BWP and cell mapping

When the Scell or BWP are deactivated without LCP restriction reconfiguration, the LCH which is mapped on this Scell or BWP cannot be used during the deactivation period, it means that the network does not want the UE to send the data on this logical channel during this period. The triggered BSR and SR due to the logical channel should be cancelled. At least the triggered SR (include D-SR and RACH) due to the deactivation of logical channel should be cancelled.

Proposal 1: When the Scell or BWP are deactivated without LCP restriction reconfiguration, the LCH which is mapped on this Scell or BWP cannot be used during the deactivation period, the triggered BSR and SR due to the logical channel should be cancelled.

- Conclusion

In this contribution, we further discuss LCH can be deactivated due to the LCP restriction and the BSR and SR cannel due to deactivation of LCH and provide the below proposal.

Proposal 1: When the Scell or BWP are deactivated without LCP restriction reconfiguration, the LCH which is mapped on this Scell or BWP cannot be used during the deactivation period, the triggered BSR and SR due to the logical channel should be cancelled.

- References

- RAN2 NR AH Jan 2017 chairman minutes