3GPP TSG-RAN WG2 #99	R2-1708883

Berlin, Germany, 21 – 25 August, 2017

Source: 	Huawei, HiSilicon

Title: 	Allocation of appropriate RACH resources for handover

Agenda Item:	10.2.8

Document for:	Discussion and decision

# Introduction

The following agreements were made for handover in RAN2 #97 meeting:

Agreements: Access information (e.g. RACH configuration) for the target cell is provided in the HO command to enable the UE to access the cell without reading system information. Access information may include beam specific information (if any).

And in RAN2 #97bis following agreements were reached [1]:

Agreements: 1 Handover command can contain at least cell identity of the target cell and RACH configuration(s) associated to the beams of the target cell. RACH configuration(s) can include configuration for contention-free random access. 1b UE selects a suitable beam from all beams of the target cell. 1c UE performs CBRA on the UE's selected beam if CFRA resources are not provided for the UE's selected beam.

In RAN1 88bis meeting, following agreements which related to this issue were achieved:

Agreements: - NR supports indication of PRACH resource allocation for non-contention based random access for a UE Agreements: - Association between one or multiple occasions for SS block and a subset of RACH resources and/or subset of preamble indices is informed to UE by broadcast system information or known to UE or FFS dedicated signaling - FFS gNB can configure an association between CSI-RS for L3 mobility and a subset of RACH resources and/or a subset of preamble indices, for determining Msg2 DL Tx beam

This contribution further discusses the dedicated resource reservation for non-contention beam based random access in handover.

# Beam specific RACH resources in handover

Handover interruption time reduction is one of the requirements for NR. Assigning dedicated resources for non-contention random access may aid to reduce the latency of random access procedure for HO [2]. Based on the agreements of RAN2 #97bis meeting, the target cell can configure and send the beam specific RACH configuration information (RACH resources) to the UE via RRCconnectionReconfiguration message (HO command). UE may choose a RACH resource associated with the suitable beam for getting access to the target cell.

For the beam based system, the agreed possible HO mechanism is shown in Figure 1, in which RACH configuration(s) associated to the beams of the target cell is included in HO command. It may require target cell knows UE’s relevant beams, and the beam information of the target cell can be acquired from measurement report like Figure 1 shows (take DL based measurement as example).

Figure 1: Possible HO procedures with beam information included

Observation 1: The target cell can configure RACH resource(s) associated to the beams and notify it to a UE, and this allows the UE to report detected beams (by something like Beam ID) to target cell.

In general, RACH resources may include time/frequency/preamble. In RAN1 discussion, indexes of time/frequency RA resources associate to corresponding beams may be carried by system information or by dedicated signalling. For handover, it may be included in HO command.

In a system with multi-beams, a UE and its serving gNB need to determine UL and DL Tx beams (beam management). This may happen during/after random access process. In order to reduce the handover interruption, it is beneficial to support the fast beam management with dedicated access resources associated with CSI-RS. More specifically, during the random access process, gNB can configure an association between each CSI-RS of a corresponding beam among the multiple beams and access resources including a different preamble index for determining Msg2 DL Tx beam [3].

Observation 2: For reducing handover interruption time, a gNB can configure the dedicated RACH resources associated with CSI-RS (beams), after receiving Msg1, the gNB can determine suitable Msg2 DL Tx beam.

The proposal  is that all the beams associated with dedicate RACH resources of the target cell should be configured in HO command. At least, beams included in UE’s measurement report may be configured with dedicate RACH resources by target gNB.

Figure 2: UE in handover consider beams

However, it has following issue: There is a delay between last measurement report and handover execution. If UE moves fast, the RACH resources associated with beams configured by target cell may outdate when UE access to the target cell after receiving handover command.

For example, in figure 2 a UE is moving towards to target cell. Beam1 and beam2 are measured and reported in its last measurement report. It is possible that due to the fast moving, when UE starts the random access process, the most suitable beam is beam3 or even beam4. However, if only UE previous detected beams are configured with dedicate RACH resource, the contention based random access is needed because the configured dedicate RACH resource is not available on the accessing beam(s).

Observation 3: For UEs in high mobility, the configured dedicate RACH resources may not be valid when a fast moving UE chooses a suitable beam to perform random access.

According to the RAN2#97bis agreement, the UE may perform contention based random access on the UE's selected beam if contention free random access resources are not provided for the UE's selected beam.

Considering the mobility of the UE, the configured dedicate resource in handover command may not enough if the UE is in high mobility (e.g. high speed). But excessive reservation of RACH resource on many beams would lead to waste of RACH resource. Because the handover procedure could be last for long time, e.g. tens to hundreds of millisecond, so reasonable dedicate resource configuration should be supported for beam based handover.

Observation 4: For mobility of UE if excessive dedicate RACH resources are configured, it will lead to waste of resources. But reasonable dedicate resource configuration should be supported for beam based handover.

Some possible solutions can be applied to overcome this issue. Considering the timelines of beam associated access resource, [4] proposed a solution in which an indicator is sent from source cell to target cell to specify immediate or delay handover execution. This solution was also trying to solve the issue that beam-related info contained in the access information may be outdated. It may work when a delay handover indicator is sent for the slow moving UE. However, for ultra-high speed UE, even target cell is indicated to perform immediate handover, the pre-allocated dedicate beam related RACH resources may still outdate. The key point here is for UE in different mobility, the dedicate RACH resources configuration may be different. To ensure that the network can allocate appropriate RACH resources for handover, upon the request from the source gNB the UE can include mobility information, e.g. serving beam switching state, speed level, direction and etc, in the measurement report, then the source gNB can inform the UE mobility related information to the target gNB, therefore the target gNB can allocate RACH resources appropriately based on information included in the Handover Request message.

If the additional UE mobility information is provided to target cell, the target cell can allocate reasonable RACH resources based on those information, and contention based random access could be avoided.   Ideally, if the dedicate RACH resources are always available whenever a UE needs for beam based handover and regard less the speed the UE , the fast DL beam management can always be performed during the random access.

Observation 5: Some additional UE mobility information could help to avoid the CBRA in handover, and the system can always perform the fast DL beam management during random access.

Proposal 1: UE mobility information (e.g. serving beam switching state) can be included in the measurement report upon the request from the source gNB.

Proposal 2: UE mobility information (e.g. serving beam switching state or speed level, direction) should be sent to target cell via handover request for beam based contention free random access for handover.

# Conclusion

Based on the above, RAN2 is requested to discuss and if possible note on the followings:

Observation 1: The target cell can configure RACH resource(s) associated to the beams and notify it to a UE, and this allows the UE to report detected beams to target cell.

Observation 2: For reducing handover interruption time, a gNB can configure the dedicated RACH resources associated with CSI-RS (beams), after receiving Msg1, the gNB can determine suitable Msg2 DL Tx beam.

Observation 3: For UEs in high mobility, the configured dedicate RACH resources may not be valid when a fast moving UE chooses a suitable beam to perform random access.

Observation 4: For mobility of UE if excessive dedicate RACH resources are configured, it will lead to waste of resources. But reasonable dedicate resource configuration should be supported for beam based handover.

Observation 5: Some additional UE mobility information could help to avoid the CBRA in handover, and the system can always perform the fast DL beam management during random access.

Proposal 1: UE mobility information (e.g. serving beam switching state) can be included in the measurement report upon the request from the source gNB.

Proposal 2: UE mobility information (e.g. serving beam switching state or speed level, direction) should be sent to target cell via handover request for beam based contention free random access for handover.

# References

[1] RAN2-97bis-Spokane-Chairman-Notes

[2] R1-1706524	WF on dedicated RACH allocation	Intel, AT&T, LG Electronics

[3] Chairman's Notes RAN1_88bis_v014

[4] R2-1703565 Beam-aware Intra-NR mobility with RRC involvement, Qualcomm Incorporated