3GPP TSG-RAN WG2 Meeting #99	R2-1710301

Prague, Czech Republic, 9th – 13th October 2017  	Revision of R2-1707930

Source:	CATT

Title:	Grant-free transmission

Agenda Item:	10.3.1.8

Document for:	Discussion and Decision

# Introduction

In last RAN2 meeting, type-1 grant free transmission (i.e. UL data transmission without grant is only based on RRC (re)configuration without any L1 signalling) was discussed and following agreement was achieved.

Agreements => For UEs in RRC_Connected mode, resources for “Type 1” resources are configured by dedicated RRC signalling

This contribution gives our understanding on the activation model of type-1 grant-free transmission.

# Discussion

- Characteristics of type-1 grant free transmission

According to RAN1 progress, Table-1 gives our understanding on the characteristics of grant-free transmission.

Table-1	Similar functionality in NR/LTE

| Characteristics of type-1 grant-free transmission | Similar functionality in NR |
| --- | --- |
| 1/ Async HARQ operation | Dynamic scheduling or SPS operation |
| 2/ Semi-static resource configuration | SPS resource allocation |
| 3/ K repetitions for one transmission | NB-IOT/eMTC SPS transmission |
| 4/ RRC configuration model | Pre-allocated grant in RRC dedicated signaling, which is used for RACH-less HO in LTE |
| 5/ shared resource across UEs | New functionality |
| 6/ target: reduce/skip UL scheduling latency | One motivation for NR SPS |

- Activation model of type-1 transmission (i.e. RRC control)

The model is similar to the pre-allocated grant and transmission operation which was introduced by LTE RACH-less HO, which carries the L1 grant in RRC signaling. In general, the operation is similar to SPS operation, but there are two issues especially for the RRC configuration model compared to the SPS operation as below, and the Figure-1 gives one example.

- Issue 1: grant free resource situation during the RRC reconfiguration ambiguity period

Since the grant-free resource is allocated by network via RRC signaling, the grant-free resource is reserved in network side when network transmits the RRC signaling to UE; and UE can use the resource for transmission immediately upon receiving it successfully. Some ambiguity of start time between the network side and UE side is allowed.

- Issue 2: grant free resource occasion pattern

Although the starting point of the network and UE side can be misaligned, the grant-free transmission occasion should be calculated for alignment. For the LTE pre-allocated grant, the occasion should be kept as fix pattern for each 10ms. So for the grant-free transmission, the grant-free transmission occasion should be also kept as fix pattern within a certain period, e.g. 10ms.

Figure-1.	Example of NR Grant-free transmission operation (RRC configuration model)

Hence, for the RRC configuration model, most characteristics are similar to SPS except for the active time point and the occasion pattern restriction.

Proposal 1: For type-1 RRC controlled grant-free transmission, the activation/deactivation model can be based on LTE pre-grant allocation scheme:

    - UE activates/deactivates the grant-free configuration immediately upon receiving the RRC configuration, i.e. no activation time;

    - The SPS pattern should be fixed and repeated within a certain period (e.g.10ms).

# Conclusion

According to the analysis in section 2, it is proposed that

Proposal 1: For type-1 RRC controlled grant-free transmission, the activation/deactivation model can be based on LTE pre-grant allocation scheme:

    - UE activates/deactivates the grant-free configuration immediately upon receiving the RRC configuration, i.e. no activation time;

    - The SPS pattern should be fixed and repeated within a certain period (e.g.10ms).