3GPP TSG-RAN WG2#99bis	Tdoc R2-1710446

Prague, Czech Republic, 9th – 13th October, 2017	(Revision of R2-1708537)

Agenda Item:	10.4.2.6

Source:	Ericsson

Title:	Response-driven paging to reduce beam sweeping overhead in NR

Document for:	Discussion, Decision

# Introduction

In high frequencies, paging, like other control signals supporting RRC_IDLE and RRC_INACTIVE state, may be provided using beam sweeping in NR. To reduce the overhead associated with such signal delivery, it has been proposed to use group paging, where the paging identifier is associated with multiple UEs and a UE receiving a page message containing its group paging identifier has to contact the network to find out whether it is actually being paged. This concept is also referred to as response-driven paging.

This contribution discusses and proposes improvements of this concept.

# Discussion

As pointed out in the previous contributions R2-168124 [1], R2-1702780 [2] and R2-1702779 [3] and in a companion RAN1 contribution R1-1708724 [4], the overhead of a beam sweep with many beams may be substantial and should preferably be kept minimal. When it comes to paging delivery, R2-168124 [1], R2-1702780 [2] and R2-1702779 [3] have proposed a group paging scheme referred to as response-driven paging. With this concept, shorter paging identifiers (referred to as paging indicators) are used, where multiple UEs can be assigned the same paging indicator. To resolve the ambiguity of paging with such a paging indicator, a UE receiving a page message containing its paging indicator has to contact the network to find out whether it is actually being paged or not.

The response-driven paging reduces the amount of data transmitted in the paging beam sweep, which is good, but it does so at the expense of a higher load on the UEs and the uplink. A UE addressed by its paging indicator will have to perform random access (transmit a preamble and receive a Random Access Response (RAR)) and provide its UE ID to the gNB in message 3, irrespective of whether it is actually being paged. For each responding UE, this turns out to be two UL transmissions (preamble + Msg3) and two DL transmissions (RAR + page ACK/NACK (assuming the NACK option is used)). One of the consequences of this is that the energy consumption increases for UEs in RRC_IDLE or RRC_INACTIVE state.

This response procedure could be streamlined by slightly changing the approach. Instead of providing its UE ID to the gNB, a UE finding its paging indicator in a paging message could instead just indicate this to the network, i.e. indicate that it has received a paging message containing the UE’s paging indicator, but without providing its UE ID. To indicate this to the gNB the UE would only transmit a PRACH preamble, which is dedicated for this purpose and common for all UEs. When receiving this preamble, the gNB (instead of sending a regular RAR) responds with a message containing the UE ID(s) of the UE(s) actually being paged. This could be seen as replacing the RAR with a paging message. When transmitting this response message, the gNB should preferably make use of directional reciprocity and transmit the power in the same directions as it received the preamble from.

The approach that a UE receiving a paging message containing its paging indicator only transmits a PRACH preamble and receives the UE ID(s) of the paged UE(s) in response is more resource efficient and saves energy in the UE compared with the previously proposed response-driven paging approach where the UE provides its UE ID to the gNB in an additional message to be informed of whether it is being paged.

With the response-driven paging option, a UE receiving a paging message containing its paging indicator should transmit a for this purpose allocated PRACH preamble (common for all UEs) and receive the UE ID(s) of the paged UE(s) in response.

As proposed in R2-1702779 [3], response-driven paging and regular paging can be used in parallel and it can be the choice of the network which one to use on a case by case basis.

Use of the response-driven paging should be optional, at the discretion of the network, and the network may choose to use it or not on a case by case basis.

A paging indicator should be shorter than the regular UE ID used with regular paging (e.g. the S-TMSI in LTE). This enables a straightforward way of allocating paging indicators to UEs. If a paging indicator of N bits is used, the paging indicator allocated to a UE could be the N most significant bits of the UE’s UE ID.

The paging indicator (with a length of N bits) allocated to a UE could be equal to the N most significant bits of the UE’s UE ID used for regular paging.

A UE is inherently aware of its UE ID. Hence, using the most significant bits of a UE’s UE ID as the paging indicator eliminates the need for explicit allocation of the paging indicator.

Using the most significant bits of a UE’s UE ID as the paging indicator eliminates the need for explicit allocation of the paging indicator.

The paging indicator associated with a UE is equal to a number of most significant bits of the UE’s UE ID (and hence implicitly allocated).

# Conclusion

In section 2 we made the following observations:

Observation 1	The approach that a UE receiving a paging message containing its paging indicator only transmits a PRACH preamble and receives the UE ID(s) of the paged UE(s) in response is more resource efficient and saves energy in the UE compared with the previously proposed response-driven paging approach where the UE provides its UE ID to the gNB in an additional message to be informed of whether it is being paged.

Observation 2	The paging indicator (with a length of N bits) allocated to a UE could be equal to the N most significant bits of the UE’s UE ID used for regular paging.

Observation 3	Using the most significant bits of a UE’s UE ID as the paging indicator eliminates the need for explicit allocation of the paging indicator.

Based on the discussion in section 2 we propose the following:

Proposal 1	With the response-driven paging option, a UE receiving a paging message containing its paging indicator should transmit a for this purpose allocated PRACH preamble (common for all UEs) and receive the UE ID(s) of the paged UE(s) in response.

Proposal 2	Use of the response-driven paging should be optional, at the discretion of the network, and the network may choose to use it or not on a case by case basis.

Proposal 3	The paging indicator associated with a UE is equal to a number of most significant bits of the UE’s UE ID (and hence implicitly allocated).

# References

R2-168124, Paging in NR at HF operation, Nokia, Alcatel-Lucent Shanghai Bell, RAN#96, 14th – 18th November 2016

R2-1702780, Evaluation of Uplink Access using Paging Indicators, Nokia, Alcatel-Lucent Shanghai Bell, RAN#97bis, 3rd – 7th April 2017

R2-1702779, Configurable Paging Procedure for NR, Nokia, Alcatel-Lucent Shanghai Bell, RAN#97bis, 3rd – 7th April 2017

R1-1708724, On NR paging design, Ericsson, RAN1#89, 15th – 19th May 2017