3GPP TSG-RAN WG2#99bis	R2-1710560

Prague, Czech Republic, 9th-13th August 2017	(revision of R2-1708692)

Source: 	Huawei, HiSilicon

Title: 	RLF for NR

Agenda Item:	10.2.11

Document for:	Discussion and decision

# Introduction

In this document, we discuss:

-	L1 notifications on (un)successful beam failure recovery

-	Avoiding the declaration of RLF using DC

# Discussion

## L1 notifications on (un)successful beam failure recovery

RAN1 has agreed to support monitoring of RS with short enough periodicity that represents the quality of control channels, similarly to what exists for LTE, so that indications similar to out-of-sync and in-sync could be provided for NR. There could be some difference if RAN1 decides to support monitoring of multiple types of RS by the same UE, so that there could be different kinds of out-of-sync or in-sync.

Observation 1: Monitoring of RS as agreed by RAN1 can provide similar indications like the out-of-sync and in-sync for LTE, but RAN1 did not decide whether the same UE can monitor only one type of RS or both simultaneously.

Unlike for LTE, there may be multiple serving beams maintained by the UE for one NR cell. If one serving beam is blocked temporarily, e.g. by the obstruction, other serving beams in the same cell can be used instead. As long as there is at least one available serving beam, there is no radio problem. According to RAN1 agreements, the UE should monitor the quality of current serving beams for data/control channel, of candidate beams that are not selected as serving beams, and of new beams within the monitored serving cell. Beam failure is declared when all serving beams are degraded and the mechanism to recover from beam failure is triggered.

In RAN1 NR#2 meeting (Qingdao, 27-30 June 2017), RAN1 further discussed RLF aspects and agreed [2] the following:

Agreements:

- In case of unsuccessful recovery from beam failure, UE sends an indication to higher layers, and refrains from further beam failure recovery

  - Relationship between RLF and unsuccessful beam failure recovery indication (if any) e.g. whether beam failure recovery procedure influences or is influenced by the RLF event

  - Send LS to inform RAN2 – to be done next meeting

Agreements:

- NR should strive to provide aperiodic indication(s) based on beam failure recovery procedure to assist radio link failure (RLF) procedure, if same RS is used for beam failure recovery and RLM procedures.

  - Example 1: aperiodic indication(s) based on beam failure recovery procedure can reset/stop T310

    - RAN2 can decide specific procedure

  - Example 2: aperiodic indication(s) based on failure of beam recovery procedure

    - How to use aperiodic indication can be decided in RAN2

  - FFS: aperiodic indication(s) based on beam failure recovery procedure to assist RLF procedure if different RS is used

Observation 2: L1 may indicate successful or unsuccessful beam failure recovery.

Possible correlations between beam failure and radio problems are as follows

1)	Each type of event below may occur

a)	Radio problems may occur while beam failure hasn't occurred (e.g. because there could be other candidates beams than the ones used for radio problem detection)

b)	Beam failure may occur while radio problems haven't occurred (e.g. in case of sudden degradation of all serving beams and candidate beams while radio problem can only be reliably detected if the degradation lasts for some time).

2)	After a certain time of beam failure not being recovered, radio problems will occur and cannot be recovered until beam failure is recovered.

3)	If both radio problems and beam failure occurred, recovery of beam failure may not be sufficient for radio problems to be recovered.

Observation 3: There are correlations between detection/recovery of radio problems and detection/recovery of beam failure.

Below we discuss how to handle the indication of successful beam failure recovery and the indication of unsuccessful beam failure recovery.

- successful beam failure recovery indication

If the indication is received while radio problems were not indicated, since the UE is already in normal operation and successful beam failure recover does not affect normal operation, no specific UE action is needed.

Proposal 1: No UE action is needed when successful beam failure recovery indication is received from lower layers while no radio problems were detected.

If the indication is received after radio problems were detected, it may be so that a new beam uncorrelated with the previous serving beam was selected so that the radio link is good again. From this perspective, if successful beam failure recovery occurs shortly before the first phase is due to expire, it would be preferable to not trigger RLF immediately as there is an increased chance of successfully restoring normal operation.

Proposal 2: Upon indication of successful beam failure recovery after radio problems were detected but before expiry of the timer which triggers the declaration of RLF, RLF is not declared before there is enough time to assess the new link quality (see proposal 5a for exact mechanism).

In LTE, the first phase after RLF is governed by a timer T310 which is stopped after detection of N311 consecutive in-sync indications. We discuss below the case how a similar mechanism would be used for NR.

In this case, if the remaining time before T310 expires is too short in order to have N311 consecutive in-sync indications, RLF would be declared anyway.

One possible action could be to stop T310 and assume that the link is good. However, if the new link is also not good enough, this would mean waiting again to receive N310 out-of-sync indications and then again that T310 expirers.

Another possible action could be to restart T310, in which case there is also additional delay in case the new link is not good, but that delay is somehow shorter than in the previous case. Yet another option would be to run T310 but for a shorter duration, specific for such a case. This could be also modeled as a different timer but with the same functionality (also, N311 could be changed if needed).

We prefer that a timer is started with the same functions like T310, which could be T310 or a timer with a different value.

Proposal 2a: When successful beam failure recovery indication is sent by lower layers, if running, the timer whose expiry triggers RLF declaration is restarted (FFS to be decided in stage 3 with the same or a different duration).

- unsuccessful beam failure recovery indication

In LTE, the UE will declare RLF when the preamble transmission reaches the max number of transmission regardless whether T310 is running or not. For NR, when beam failure recovery cannot recover from beam failure, RAN1 agreed that an unsuccessful recovery from beam failure indication for reaching the max transmission for beam failure recovery request will be provided from lower layers. In that case, RAN1 agreed that there will be no further action to recover from beam failure, so there will be continuous out-of-sync indications which would eventually trigger RLF and RRC signalling. In such a case, RLF can be declared in order to timely initiate the RRC signalling procedure regardless whether the timer started upon radio link problems is running or not, like LTE RLF following random access failure. Therefore we propose

Proposal 3: RLF is declared immediately when unsuccessful beam failure recovery indication is received from lower layers.

Proposal 4: Agree the TP below capturing proposal 5, 6, 6a and 6 on the impact of (un)successful beam failure recovery indications to radio link failure.

### 9.2.7	Radio Link Failure

In RRC_CONNECTED, the UE declares Radio Link Failure (RLF) when one of the following criteria are met:

-	Expiry of a timer started after indication of radio problems from the physical layer (if radio problems are recovered before the timer is expired, the UE stops the timer) or after successful beam failure recovery indication from the physical layer;

-	Indications of unsuccessful beam failure recovery from the physical layer;

-	Random access procedure failure;

-	RLC failure.

## Avoiding the declaration of RLF thanks to DC

In LTE, the UE will initiate the SCG failure information procedure when the UE detects RLF for SCG. Upon initiating the SCG failure information procedure, the UE shall:

1>	suspend all SCG DRBs and suspend SCG transmission for split DRBs;

1>	reset SCG-MAC;

1>	stop T307;

1>	initiate transmission of the SCGFailureInformation message in accordance with 5.6.13.3;

Also, the mechanisms of duplicated SRB are supported in all DC cases including all MR-DC and NR-NR DC cases while split SRB are supported in EN-DC and NR-NR DC cases. Meanwhile both duplicated DRB and split DRB are supported in NR-NR DC case. If the UE detects RLF from all monitored cells within one Node, the UE can send the corresponding RLF report via the other Node to avoid reestablishment. Specifically the UE can send the MN RLF report via SN and send the SN RLF report via MN. When RLF occurs for a Node only, e.g. MN or SN, the UE shall at least stop any transmission on this Node and:

- suspend all DRBs (if any) for this Node;

- suspend the transmission for split DRBs (if any) for this Node;

- suspend the transmission for duplicated DRBs (if any) for this Node;

- suspend the transmission for split SRB (if any) for this Node;

- suspend the transmission for duplicated SRB (if any) for this Node;

- reset the MAC for this Node;

- Initiate the transmission of RLF report for this Node via the other Node.

However if both MN RLF and SN RLF occurs, the UE shall initiate the reestablishment procedure.

Proposal 5: For all DC cases (all MR-DC and NR-NR DC cases), if SRBs are configured with a path via both nodes (duplication or split SRB) and RLF occurs in all monitored cells within a Node, the UE shall at least

- suspend all DRBs (if any) for this Node;

- suspend the transmission for split DRBs (if any) for this Node;

- suspend the transmission for duplicated DRBs (if any) for this Node;

- suspend the transmission for split SRB (if any) for this Node;

- suspend the transmission for duplicated SRB (if any) for this Node;

- reset the MAC for this Node;

- Initiate the transmission of RLF report for this Node via the other Node.

When both MN RLF and SN occur, the reestablishment procedure should be initiated in all DC cases.

Proposal 6: For all DC cases (all MR-DC and NR-NR DC cases), if SRBs are configured with a path via both nodes (duplication or split SRB) and both MN RLF and SN RLF occur the reestablishment procedure is initiated.

# Conclusion

This contribution discusses the RLF for NR from the perspective of RAN2 and makes the following proposals:

Proposal 1: No UE action is needed when successful beam failure recovery indication is received from lower layers while no radio problems were detected.

Proposal 2: Upon indication of successful beam failure recovery after radio problems were detected but before expiry of the timer which triggers the declaration of RLF, RLF is not declared before there is enough time to assess the new link quality (see proposal 5a for exact mechanism).

Proposal 2a: When successful beam failure recovery indication is sent by lower layers, if running, the timer whose expiry triggers RLF declaration is restarted (FFS to be decided in stage 3 with the same or a different duration).

Proposal 3: RLF is declared immediately when unsuccessful beam failure recovery indication is received from lower layers.

Proposal 4: Agree the TP below capturing proposal 5, 6, 6a and 6 on the impact of (un)successful beam failure recovery indications to radio link failure.

### 9.2.7	Radio Link Failure

In RRC_CONNECTED, the UE declares Radio Link Failure (RLF) when one of the following criteria are met:

-	Expiry of a timer started after indication of radio problems from the physical layer (if radio problems are recovered before the timer is expired, the UE stops the timer) or after successful beam failure recovery indication from the physical layer;

-	Indications of unsuccessful beam failure recovery from the physical layer;

-	Random access procedure failure;

-	RLC failure.

Proposal 5: For all DC cases (all MR-DC and NR-NR DC cases), if SRBs are configured with a path via both nodes (duplication or split SRB) and RLF occurs in all monitored cells within a Node, the UE shall at least

- suspend all DRBs (if any) for this Node;

- suspend the transmission for split DRBs (if any) for this Node;

- suspend the transmission for duplicated DRBs (if any) for this Node;

- suspend the transmission for split SRB (if any) for this Node;

- suspend the transmission for duplicated SRB (if any) for this Node;

- reset the MAC for this Node;

- Initiate the transmission of RLF report for this Node via the other Node.

Proposal 6: For all DC cases (all MR-DC and NR-NR DC cases), if SRBs are configured with a path via both nodes (duplication or split SRB) and both MN RLF and SN RLF occur the reestablishment procedure is initiated.

# References

- R2-1706057, Reply LS on in-sync/out-of-sync indication for RLF, RAN1

- RAN1#NR2 chairmen note

- 3GPP TS 36.300

- 3GPP TR 36.331: "Evolved Universal Terrestrial Radio Access (E-UTRA); Radio Resource Control (RRC) Protocol specification".