3GPP TSG-RAN WG2 #99bis	Tdoc R2-1710843

# Prague, Czech Republic, 9th – 13th October 2017

Agenda Item:	10.4.1.4.6

Source:	Ericsson

Title:	TP on inter-RAT ANR to 36.300 for EN-DC

Document for:	Discussion, Decision

# Introduction

In RAN WG2 #98-Ad Hoc meeting, it was agreed that the LTE inter-RAT ANR framework will be reused to cover NR.

Agreements

1	LTE inter-RAT ANR framework is extended to cover NR (target to complete this at end of Rel-15, not December 2017)

In this contribution, a Text Proposal (TP) to 36.300 is given on how this agreement is reflected in the specification.

# Overall TP content and open issues

For this TP, we have used the 36.300 text on inter-RAT ANR and added details for the detection of NR cells.

Identified open issues have been marked as FFS.

# Text Proposal

### 22.3.4	Inter-RAT/Inter-frequency Automatic Neighbour Relation Function

Figure 22.3.4-1: Automatic Neighbour Relation Function in case of UTRAN detected cell

For Inter-RAT and Inter-Frequency ANR, each cell contains an Inter Frequency Search list. This list contains all frequencies that shall be searched. For Inter-RAT cells, the NoX2 attribute in the NRT is absent, as X2 is only defined for E-UTRAN. Whether or not X2, and hence also the NoX2 attribute, will be used for relations to non-standalone NR cells is FFS.

The function works as follows:

The eNB serving cell A has an ANR function. During connected mode, the eNB can instruct a UE to perform measurements and detect cells on other RATs/frequencies. The eNB may use different policies for instructing the UE to do measurements, and when to report them to the eNB.

1	The eNB instructs a UE to look for neighbour cells in the target RATs/frequencies. To do so the eNB may need to schedule appropriate idle periods to allow the UE to scan all cells in the target RATs/frequencies.

2	The UE reports the PCI of the detected cells in the target RATs/frequencies. The PCI is defined by the carrier frequency and the Primary Scrambling Code (PSC) in case of UTRAN FDD cell, by the carrier frequency and the cell parameter ID in case of UTRAN TDD cell, by the Band Indicator + BSIC + BCCH ARFCN in case of GERAN cell, by the PN Offset in case of CDMA2000 cell, and by the NR-PSS/SSS in case of NR cell.

When the eNB receives UE reports containing PCIs of cell(s) the following sequence may be used.

3	The eNB instructs the UE, using the newly discovered PCI as parameter, to read the CGI and the RAC of the detected neighbour cell in case of GERAN detected cells, CGI, LAC, RAC and all broadcasted PLMN-ID(s) in case of UTRAN detected cells, CGI in case of CDMA2000 detected cells, and NRCGI(s), TAC(s) and all available PLMN ID(s) in case of NR detected cells. For the Interfrequency case, the eNB instructs the UE, using the newly discovered PCI as parameter, to read the ECGI, TAC and all available PLMN ID(s) of the inter-frequency detected cell. The UE ignores transmissions from the serving cell while finding the requested information transmitted in the broadcast channel of the detected inter-system/inter-frequency neighbour cell. To do so, the eNB may need to schedule appropriate idle periods to allow the UE to read the requested information from the broadcast channel of the detected inter-RAT/inter-frequency neighbour cell.

4	After the UE has read the requested information in the new cell, it reports the detected CGI and RAC (in case of GERAN detected cells) or CGI, LAC, RAC and all broadcasted PLMN-ID(s) (in case of UTRAN detected cells) or CGI (in case of CDMA2000 detected cells) or NRCGI(s), TAC(s) and all PLMN-ID(s) that have been detected (in case of NR detected cells) to the serving cell eNB. In the inter-frequency case, the UE reports the ECGI, the, tracking area code and all PLMN-ID(s) that have been detected. If the detected cell is a CSG or hybrid cell, the UE also reports the CSG ID to the serving cell eNB.

5	The eNB updates its inter-RAT/inter-frequency Neighbour Relation Table.

In the inter-frequency case and if needed, the eNB can use the PCI and ECGI for a new X2 interface setup towards this eNB. The setup of the X2 interface is described in section 22.3.2.

NOTE:	The eNB may differentiate the open access HeNB from the other types of (H)eNB by the PCI configuration or ECGI configuration.