3GPP TSG-RAN WG2 #99-bis	R2-1710866

# Prague, Czech, 9th– 13th October 2017

Agenda Item:	10.2.3

Source:	Mediatek Inc.

Title:	Text Proposal for BWP Operation in 38.300

Document for:	Discussion, Decision

# 1 Introduction

The concept of bandwidth part (BWP) was introduced in RAN1#88bis meeting. After several RAN1 meetings one LS from RAN1 [1] was sent to RAN2 with a series of agreements. In this contribution, one TP for BWP operation in 38.300 is provided according to the discussion paper [2], targeting to:

- Capture the usage scenarios for BWP operation;

- Capture basic principle for BWP configuration;

- Capture basic principle for BWP activation/deactivation.

# 2 Text Proposal for 38.300

X.X.X.X Bandwidth Part Operation

In BWP operation, UE performs RF adaptation through either center frequency tuning or bandwidth adaptation or both of them on a wideband carrier.  It enables reduced UE bandwidth capability within a wideband carrier, reduced UE power energy consumption by bandwidth adaptation and UE using different numerologies in FDM within a wideband carrier.

BWP operation is for UE in RRC CONNECTED mode. It is an operation performed for a UE within each UE-specific serving cell, just illustrated in Figure x.x.

Figure x.x BWP operation within each serving cell

When UE in CONNECTED mode, one or multiple UE-specific DL/UL BWPs can be configured for the serving cell. Each DL/UL BWP is characterized by numerology, center frequency, bandwidth and CORESET. For a serving cell, UE expects at least one DL BWP and one UL BWP being active among the set of configured BWPs for a time instant and only receives/transmits within the active DL/UL BWPs. BWP activation/deactivation can be performed by means of RRC signaling and explicit indication in scheduling DCI. UE switching from the active DL bandwidth part to a default DL bandwidth part can be controlled by a timer.

There is an initial active DL/UL BWP pair for UE in IDLE/Inactive mode. It keeps valid until the UE is explicitly (re)configured with BWPs during or after RRC connection is established.

# 3 Reference

[1] R1-1711998, LS on Bandwidth Part Operation in NR

[2] R2-1710864, Basic Framework for Bandwidth Part Operation, Mediatek Inc.