3GPP TSG-RAN WG2 #99bis	Tdoc R2-1711180

Prague, Czech Republic, 9th – 13th October 2017	Revision of R2-1708348

Agenda Item:	10.3.1.6

Source:	Ericsson

Title:	Further aspects on BSR transmission and cancellation

Document for:	Discussion, Decision

# Introduction

Uplink scheduling is a key functionality to meet a broad range of use cases including enhanced mobile broadband, massive MTC, critical MTC, and additional requirements defined during the RAN requirements study. Details to how this functionality can be designed, compared to the baseline framework of LTE is discussed in contributions.

In RAN2#98 some agreements related to BSR were reached.

Agreements

- The number of LCGs will be increased up to 8.

- The concept of periodicBSR-timer and retx-BSRtimers are reused and are configured per MAC entity

- As a baseline, the concept of logicalChannelSR-ProhibitTimer is reused in NR. It is allowed to configured infinite value for this timer.

- The logicalChannelSR-Mask is supported

In RAN2#AdHoc held on 27th – 29th June 2017, some agreements related to the BSR triggers were made

Agreements

=>	No new BSR triggers in addition to LTE are needed

Being based on the LTE triggers as a baseline for NR, this contribution discusses the further potential enhancements for BSR triggers to better support multiple numerologies. BSR formats are discussed in [1].

# Discussion

## BSR transmission and cancellation in non-CA scenarios

In LTE, if a regular BSR is triggered due to data becoming available for transmission for a logical channel, and there is no uplink grant received, the UE shall trigger a SR. In NR, different LCHs may be mapped to different SR configurations/resources. Upon reception of a SR, the gNB assigns a grant that corresponds with resources used by the LCHs that are associated with the used SR configuration. Therefore, this grant may be only applicable to a subset of LCHs. It means some LCHs may have to wait for future grants after the current transmission. It is reasonable for the UE to take same actions as that in LTE, to include a BSR in the current transmission for those rest LCHs regardless the grant is suitable or not. We think it is simple and easy to control BSR transmissions.

As in LTE, a BSR comprises BSs for all LCGs regardless of which transmission duration is associated with the received grant

In LTE, when an SR is triggered, it shall be considered as pending until it is cancelled. All pending SR(s) shall be cancelled when a MAC PDU is assembled and this PDU includes a BSR which contains buffer status up to (and including) the last event that triggered a BSR, or when the UL grant(s) can accommodate all pending data available for transmission. As suggested in P1 and if we use the same BSR transmission rules for NR as in LTE, it is also reasonable to reuse the LTE SR cancellation rules for NR.

As in LTE, a pending SR is cancelled if a MAC PDU includes a BSR which contains buffer status up to (and including) the last event that triggered a BSR, or if the UL grant can accommodate all pending data available for transmission

Similarly as in LTE, all triggered BSRs are cancelled in either of two cases

- the UL grant(s) in this TTI can accommodate all pending data available for transmission but is not sufficient to additionally accommodate the BSR MAC control element plus its subheader.

- a BSR is included in a MAC PDU for transmission.

With the same cancellation rules in NR:

As in LTE, all triggered BSRs are cancelled if either

- A UL grant accommodate all pending data available for transmission but is not sufficient to additionally accommodate the BSR MAC control element plus its subheader; or

- a BSR is included in a MAC PDU for transmission.

At the same time, a Padding BSR is not allowed to cancel a triggered Regular/Periodic BSR. A Padding BSR is triggered for a specific MAC PDU only and that trigger is cancelled when the MAC PDU with the Padding BSR has been built.

As in LTE, a Padding BSR does not cancel a triggered Regular/Periodic BSR.

## BSR transmission in CA

In LTE, the MAC entity shall transmit at most one Regular/Periodic BSR in a TTI. If the MAC entity is requested to transmit multiple MAC PDUs in a TTI, the UE can select any one of the MAC PDU to carry the Regular/Periodic BSR, further it may include a padding BSR in any of the MAC PDUs which does not contain a Regular/Periodic BSR.

In NR, the UE MAC entity may receive multiple grants at the same time, and each grant may be associated with a separate numerology/TTI length. The gain by transmitting more than one Regular/Periodic BSRs is not clear. It may however lead to additional complexity for BSR transmission and cancellation. Therefore, we suggest also here to reuse the LTE rules for NR.

As in LTE, the MAC entity shall transmit at most one Regular/Periodic BSR when transmitting more than one MAC PDU at the same time. It is up to UE implementation on which serving cell to transmit Regular/Periodic BSR.

## BSR timers

There are two timers controlling BSR in LTE. The periodicBSR-Timer which triggers BSR periodically, and the retxBSR-Timer which triggers BSR if the UE has data and has not been scheduled. These timers are configured in subframes. As one subframe has the duration of one millisecond in both LTE and NR, we think they can be configured in either subframes or milliseconds. Buffer status reporting is a procedure in which the UE informs the gNB about the amount of data in the UE such that the gNB can make a scheduling decision. There seems to be no impact from varying numerologies or TTI durations in this case. However, as the general timing of everything (e.g. processing times) in NR is shorter than in LTE, shorter values may be considered .

The periodicBSR-Timer and retxBSR-Timer are defined in either subframes or milliseconds. Adding shorter values than in LTE can be considered.

# Conclusion

A text proposal covering the proposals in this paper can be found in [2].

Based on the discussion in Section 2 we propose the following:

Proposal 1	As in LTE, a BSR comprises BSs for all LCGs regardless of which transmission duration is associated with the received grant

Proposal 2	As in LTE, a pending SR is cancelled if a MAC PDU includes a BSR which contains buffer status up to (and including) the last event that triggered a BSR, or if the UL grant can accommodate all pending data available for transmission

Proposal 3	As in LTE, all triggered BSRs are cancelled if either

1)	A UL grant accommodate all pending data available for transmission but is not sufficient to additionally accommodate the BSR MAC control element plus its subheader; or

2)	a BSR is included in a MAC PDU for transmission.

Proposal 4	As in LTE, a Padding BSR does not cancel a triggered Regular/Periodic BSR.

Proposal 5	As in LTE, the MAC entity shall transmit at most one Regular/Periodic BSR when transmitting more than one MAC PDU at the same time. It is up to UE implementation on which serving cell to transmit Regular/Periodic BSR.

Proposal 6	The periodicBSR-Timer and retxBSR-Timer are defined in either subframes or milliseconds. Adding shorter values than in LTE can be considered.

# References

- R2-1711181, "Aspects of BSR format and tables", Ericsson, RAN2#99bis, Prague, Czech Republic, 9th – 13th October 2017

- R2-1711185, "BSR Text proposal", Ericsson, RAN2#99bis, Prague, Czech Republic, 9th – 13th October 2017