3GPP TSG-RAN WG2 #99bis	Tdoc R2-1711234

Prague, Czech Republic, 9th – 13th October 2017

Agenda Item:	10.2.16

Source:	Ericsson

Title:	Default DRB system impact and signalling aspects

Document for:	Discussion, Decision

# Introduction

RAN2 June Ad Hoc agreed:

Agreements:

- There is a need to tell the UE that it has to update the mapping rule.   For the AS reflective QoS it is up to the RAN to decide when to update the mapping rules.  FFS on the details of the header format.

- It is up to the RAN to decide when and which mechanism, explicit RRC re-configuration and/or AS reflective QoS, should be used to provide mapping information to the UE.

- A UE follows the latest QoS flow to DRB mapping information regardless of the fact whether it was explicit RRC or AS reflective QoS.

- Whether a SDAP header is present or not is configured by RRC per DRB

- The gNB indicates to UE using RRC signaling the default DRB for a PDU session.

- RAN decides and configures the default DRB for a PDU session.

This contribution discusses about the need for defining default DRB terminology. Our understanding is that the concept of default DRB is redundant and adds unnecessary complexity to the system. Instead of having default DRB defined, explicitly defined QoS can always be used to make QFI to DRB mapping.

# Discussion

Figure 1: gNB and UE reference point representation.

SA2 provided following input on LS on default DRB establishment for PDU session.

Table 1: SA2 Response LS on default DRB establishment for PDU session

Table 1: SA2 Response LS on default DRB establishment for PDU session

Furthermore TS23.501 specifies that the SMF in the core network provides for each PDU session one or more QoS profiles (QFI(s)) to the gNB via the N2-SM interface. The SMF provides the default QoS rule to the UE. SA2 assumes that the gNB may use the QoS profiles to determine the configuration of DRBs i.e. the mapping of QFIs to DRBs. SA2 further assumes that it is up to the gNB to determine how to map one or more QFIs to a particular DRB.

Furthermore TS23.501 specifies that the SMF in the core network provides for each PDU session one or more QoS profiles (QFI(s)) to the gNB via the N2-SM interface. The SMF provides the default QoS rule to the UE. SA2 assumes that the gNB may use the QoS profiles to determine the configuration of DRBs i.e. the mapping of QFIs to DRBs. SA2 further assumes that it is up to the gNB to determine how to map one or more QFIs to a particular DRB.

Default QoS rule mentioned in the LS (Table 1) is related to UE IP filters and it is not visible for RAN. In default QoS rule, the UE is allowed to map unknown IP Flows via default QoS rule. This “no-match”-filter is not visible for RAN as the traffic destined to default QoS rule will be carried in UL over DRB, which is known to RAN (QFI to DRB mapping exits). This results that the RAN does not need to be aware of default QoS rule of the UE.

There is no default QoS profile and RAN does not need to be aware of default QoS rule.

The 5CN is responsible of defining QoS characteristics for each QoS Flow that it introduces on PDU session. The QoS characteristics may be defined explicitly by having QoS profile, or by relying on standardized 5QI values defined QoS parameters. Table 2 is showing relationship between 5QI types and LTE QCI.

Table 1	5QI types in 5G system

| Standardized 5QI | Same as standardized QCI |
| --- | --- |
| Pre-configured 5QI | Same as operator defined QCI |
| Dynamic assigned 5QI | All characteristics parameters are signaled. |

PDU session will always have at least one QFI and the CN assigns QoS profile for the QFI either explicitly by sending QoS parameters, or implicitly, by using standardized 5QI value. This guarantees that the gNB is always able to make QFI to DRB mapping for the QFI, which is again send to the UE.

The concept of default DRB was defined based on the assumption that the standardized QFIs may be used by 5CN or UE without N1 or N2 signaling. This, however, is not true anymore as the SA2 has agreed to have dedicated signaling only for non-3GPP access [1]. Instead of using default DRB, the gNB is always able to determine QFI to DRB mapping based on N2 signaling from the CN. The mapping can be then propagated to the UE with RRC signaling or AS-reflective QoS mechanism.

The gNB is always able to make QFI to DRB mapping for the QFI

Pre-definition of QoS is only needed for non-3GPP Access Networks

### UE initiated new QFI to DRB mapping

In RAN2#99 concerns were raised about UE having QFI, which is not previously known for RAN. According to SA2, this should not be valid case. When UE has QFI, that it is allowed to mark to the UL packets, 5CN should always be aware of it. As the 5CN is aware of the UL initiated QFI, the 5CN would also make RAN aware of the upcoming QFI. This would allow RAN to configure explicit QFI to DRB mapping for UE. This results, that there is no need for default DRB, even when the first packets are UL packets from UE.

5CN is always aware of QFIs initiated from the UE to UL direction. Therefore, no default DRB is needed.

There is always N1 signalling prior of using QFI.

### First DL traffic and QFI to DRB mapping delivery via RRC

The 5CN uses NAS configured QoS rules, i.e. it uses N1 (between UE and AMF) signalling to convey QoS information, such as TFT, a QFI and possibly other QoS parameters to the UE. As the 5CN informs UE via N1, it appears straightforward to inform RAN via N2 (between gNB and AMF) about new flow being established. Based on the N2 signalling, the gNB may map the QFI to new DRB and inform UE via RRC about new QFI to DRB mapping. Alternatively, the gNB may inform UE via RRC to map the QFI to existing DRB for which it may or may not adjust its internal QoS handling parameters. This mechanism guarantees that UEs SDAP layer does not observe any UL packets without QFI-to-DRB mapping.

When QFI-to-DRB mapping is derived from N1 & N2 signaling, UE does not observe UL packets without QFI to DRB mapping.

### Using NAS level reflective Mapping

When 5CN uses NAS level reflective QoS, this implies that the first packets occurring on a new QFI are downlink packets. NAS level reflective QoS with previously unknown QFI may be only used with standardized QFI/5QI. In order to use non-standardized QFIs the 5CN must provide N2 signaling before first packets are marked with non-standard QFI. In both cases, gNB is able to decide to which DRB to map new QFI and inform UE either via reflective AS QoS or via RRC. This results that the SDAP layer of the UE does not observe any UL packets without QFI to DRB mapping.

gNB configures QFI to DRB mapping for every QFI.

Default DRB is not defined for PDU session.

# Conclusion

In section 2 we made the following observations:

Observation 1	There is no default QoS profile and RAN does not need to be aware of default QoS rule.

Observation 2	The gNB is always able to make QFI to DRB mapping for the QFI

Observation 3	Pre-definition of QoS is only needed for non-3GPP Access Networks

Observation 4	5CN is always aware of QFIs initiated from the UE to UL direction. Therefore, no default DRB is needed.

Observation 5	There is always N1 signalling prior of using QFI.

Observation 6	When QFI-to-DRB mapping is derived from N1 & N2 signaling, UE does not observe UL packets without QFI to DRB mapping.

Based on the discussion in section 2 we propose the following:

Proposal 1	gNB configures QFI to DRB mapping for every QFI.

Proposal 2	Default DRB is not defined for PDU session.

# References

System Architecture for the 5G System, 3GPP TS 23.501, V1.3.0 (2017-09).