3GPP TSG-RAN WG2 #99bis	Tdoc R2-1711252

Prague, Czech Republic, 9th – 13th October 2017

Agenda Item:	10.3.1.8

Source:	Ericsson

Title:	SPS for SCell

Document for:	Discussion, Decision

# Introduction

In RAN2-AH#2 2017, it was agreed that multiple SPS for the same cell will not be supported and that SPS on PSCell will be supported, which means that single SPS configuration can be configured for PCell and PSCell respectively for a UE. In RAN2-99, it was further agreed that UL/DL SPS configuration can be configured and activated simultaneously on both PCell and PSCell and that SPS can be configured for a SCell. However, there is FFS on whether it is restricted to a single configuration or can be allowed on multiple SCells.

This paper discuss the open issues for SPS configuration of SCell.

# Discussion

As it was already agreed that multiple SPS for the same cell will not be supported. This rule shall be applied for SCell as well.

At most one SPS for one SCell will be supported

In NR, the activated SCells for a UE may be configured with different TTI lengths/numerologies. Service like VoIP of a UE can be served in PCell/PSCell of numerology/TTI configuration 1 with SPS configuration 1 while the URLLC (or V2X) service of the same user is served in a SCell of numerology/TTI configuration 2 with SPS configuration 2, wherein SPS configuration 1 and 2 are optimal for the respective service patterns and QoS requirements. In another example, for uplink packet duplication in case of carrier aggregation, SPS for SCell can enable UL duplicated transmission using SPS grants. In both examples, the SPS configurations are simulataneously active for both PCell/PSCell and SCell.

SPS operation can be active simultaneously for PCell/PSCell and SCell

In RAN1, two types of semi-persistent scheme schemes were agree for NR.

- Type 1: the SPS grant is purely conveyed via dedicated RRC signalling without L1 control signal.

The parameter set in RRC signalling comprises scheduling interval, offset, time frequency radio resources, HARQ parameters and MCS. Whether to include other parameters such as power control parameter, slot length, multiplexing need further study.

- Type 2: SPS grant is conveyed jointed via dedicated RRC signalling and L1 control signal

The parameter usch as  scheduling interval, offset and HARQ parameters are conveyed via dedicated RRC signalling and the other parameters such as MCS, time-frequency resources, DMRS and power control paramters are conveyed via PDCCH. PDCCH acts as an activation/deactivation signal for SPS operation.

Considering that different carrier may be configured with different numerology/slot length, the physical layer parameters such as scheduling interval, offset, DMRS, MCS etc could be different for different carriers.  One common SPS configuration for all SCells may be not always feasible. Hence, we propose

SPS is configured per SCell

For type 1 SPS scheme above, the SPS grant is issued via RRC signalling and the granted radio resources are reserved when the SPS grant is issued. Considering the delay of RRC signalling and that a SCell can be frequently activated and deactivated via MAC CE, it seems not so beneficial to support type 1 SPS in a SCell when there is already type 2 SPS.

RAN2 selects one of the following options:

  - Support only Type 2 SPS scheme for SCell

  - Support both Type 1 and Type 2 SPS schemes for SCell

# Conclusion

In section 2 we made the following observations:

Observation 1	At most one SPS for one SCell will be supported

Based on the discussion in section 2 we propose the following:

Proposal 1	SPS operation can be active simultaneously for PCell/PSCell and SCell

Proposal 2	SPS is configured per SCell

Proposal 3	RAN2 selects one of the following options:

a.	Support only Type 2 SPS scheme for SCell

b.	Support both Type 1 and Type 2 SPS schemes for SCell