3GPP TSG-RAN WG2 Meeting #99bis	R2-1711633

Prague, Czech Republic, 9th – 13th October 2017

Agenda Item:	9.13.1

Source: 	MediaTek Inc.

Title:  	NPRACH resource partition for early data transmission

Document for:	Discussion and decision

Introduction

In RAN2 #99, the early data transmission (EDT) had been discussed and the agreements were made as follows [1].

Agreements: - We intend to support early UL data transmission in Msg3 for control plane and user plane CIoT EPS optimisation. - We intend to support early DL data transmission in Msg4 for control plane and user plane CIoT EPS optimisation. - Early data transmission feature is considered when AS security was not established for only transmitting data using CP. - Early data transmission feature is considered when AS security was established for transmitting data using CP and/or UP.

In addition, an email discussion was set to address the detailed aspects of early data transmission.

[99#45][NB-IoT/MTC] Early data transmission (Qualcomm)

Detailed aspects of early data transmission

To address the following but not limited to:

RRC signalling, e.g. whether there is any RRC signalling, at which point control information is sent and how. Security aspects, indication of UE’s intention of early data, contents of Msg2, Msg3, Msg4, Msg5, how to identify a UE etc.

Intended outcome: Report to next meeting

Deadline: 2017-09-21

In this paper, we discuss the design of UE’s indication for the intention of EDT, specifically we consider indicating the desired TB size of Msg3 through NPRACH resource partitioning.

Discussion

To perform UL EDT in Msg3, a device support this feature should be able to show its intention on EDT to the network and the network should be able to allocate a larger UL grant to the UE for data transmission in Msg3. A benefit of EDT and TB size indication can avoid the excessive transmission of padding with MSG3, which is particularly important in bad coverage. The most straight forward way for a UE to indicate the need of larger size Msg3 is done through Msg1, the NPRACH transmission.

Observation: A benefit of EDT and TB size indication can avoid the excessive transmission of padding with MSG3, which is particularly important in bad coverage.

Indicating TB size of Msg3 through NPRACH resource partition

In LTE, preambles for contention-based RACH divide into two groups, group A and group B, to indicate the need of different size of Msg3. In NB-IoT, the size of Msg3 is fixed to 88 bits so that it does not need to indicate the Msg3 size through the NPRACH transmission. However, the NPRACH resource might be partitioned into two portions to indicate the single-tone or multi-tone transmission of Msg3 in the frequency domain. In addition, the network can configures up to 3 NPRACH repetitions to support different CE levels for a cell; thus, the NPRACH resource might be further partitioned in the time domain.

Similar to the design of the legacy LTE, the indication for EDT can be done through NPRACH resource partition. The NPRACH resource can be partitioned into different groups to indicate the different TB size of Msg3. As indicated in [2], the optimal payload size for UL transmission by water meters is 100 Bytes for regular transmission. We propose the possible TB sizes of Msg3 should include 100 Bytes.

Proposal 1: NPRACH resource partition can be used to indicate the TB size of Msg3.

Proposal 2: The TB sizes of Msg3 should include 100 Bytes.

We can consider the NPRACH resource to indicate EDT by introducing new NPRACH sequence or by introducing new NPRACH hopping pattern. Or simply, the NPRACH resource can be partitioned in the subcarrier domain, in the time domain, and/or in the carrier domain to indicate the TB size of Msg3. To take into account the most use cases for NB-IOT transmission are delay insensitive, we slightly prefer to perform NPRACH resource partition in the time domain. Different NPRACH resource pools can be TDMed in the time domain. FFS on whether the NPRACH resource pools for EDT can be partially overlapped and the collision handing on the overlapped part, if allowed. Note that in NB-IOT, UEs in lower coverage treat NPRACH opportunities that collide with NPRACH opportunities of higher coverage level(s) as invalid. UEs only use valid NPRACH opportunities for Msg1 transmission.

Proposal 3: NPRACH resource to indicate TB size of Msg3 can be partitioned in the time domain.

NPRACH resource configuration

NPRACH resource configuration is broadcasted in SIB2-NB for the anchor carrier and in SIB22-NB for the non-anchor carries. Up to 3 NPRACH repetitions can be configured to support different CE levels in a cell. Table 1 captures the NPRACH parameters in the system information.

Table 1: NPRACH parameters

| RRC parameters | Description |
| --- | --- |
| nprach-Periodicity | Periodicity of a NPRACH resource |
| nprach-StartTime | Start time of the NPRACH resource in one period |
| nprach-SubcarrierOffset | Frequency location of the NPRACH resource |
| nprach-NumSubcarriers | Number of sub-carriers in a NPRACH resource |
| nprach-SubcarrierMSG3-RangeStart | Fraction for calculating the starting subcarrier index of the range reserved for indication of UE support for multi-tone Msg3 transmission, within the NPRACH resource |
| maxNumPreambleAttemptCE | Maximum number of preamble transmission attempts per NPRACH resource |
| numRepetitionsPerPreambleAttempt | Number of NPRACH repetitions per attempt for each NPRACH resource |
| nprach-NumCBRA-StartSubcarriers | The number of start subcarriers from which a UE can randomly select a start subcarrier |

To not collide with the configuration for legacy UE, the configuration on NPRACH resource for EDT should be independent to the legacy NPRACH resource configuration. An EDT NPRACH resource pool will associate to a certain TB size of Msg3, assuming the configuration is specific to a certain CE level with a certain NPRACH repetition level. A new TB size of Msg3 association field should be added in the NPRACH parameters, e.g., nprach-TBSMsg3. eNB broadcasts NPRACH resource for EDT configurations in the system information, SIB2-NB for the anchor carrier and SIB22-NB for the non-anchor carriers similar to the legacy NB-IOT.

Then, an EDT capable UE takes into account RRC message size, NAS message size and data in the buffers. If all can be transmitted in one TB, the UE determines a NPRACH resource pool indicating this TB size (or the next higher), and randomly selects a NPRACH resource from the corresponding PRACH pool. UE transmits the PRACH preamble (Msg1) in the selected NPRACH resource.

Proposal 4: The configuration on NPRACH resource for EDT can be independent to the legacy NPRACH resource configuration.

Proposal 5: A new TB size of Msg3 association field should be added in the NPRACH parameters, e.g., nprach-TBSMsg3.

Proposal 6: eNB broadcasts NPRACH resource for EDT configurations in the system information, SIB2-NB for the anchor carrier and SIB22-NB for the non-anchor carriers.

Proposal 7: A UE determines a NPRACH resource pool indicating the TB size which can nearly accommodate RRC message size, NAS message size and data in the buffers, randomly selects a NPRACH resource from the corresponding PRACH pool, and transmits the PRACH preamble (Msg1) in the selected NPRACH resource.

UL grant in random access response

After eNB successfully receives the ‘larger Msg3’ indication from a UE, it allocates the corresponding UL grant in RAR to the UE. For NB-IoT, the UL grant field (15 bits) [3] and is used to indicate uplink subcarrier spacing, subcarrier indication, scheduling delay, Msg3 repetition number, and MCS index indicating TBS, modulation, and number of RUs for Msg3. There are some reserved values in the MCS index for Msg3 NPUSCH (see Table 16.3.3-1 in [4]) and it can be used to indicate the TBS, modulation, and number of RUs for larger-sized Msg3.

The UE then multiplexes the RRC message, NAS message and UL data to transmit in the UL grant.

Proposal 8: The UL grant field in RAR should indicate the TBS, modulation, and number of RUs for larger-sized Msg3.

Falling back to the legacy RRC procedure

In some situations, UE needs to consider falling back to the legacy RRC connection establishment/resumption procedure. For example, when no PRACH resource is broadcasted for data in Msg3 procedure, or when UE intends to perform NAS signaling procedure.

Furthermore, if not all UL data (RRC message, NAS message and data) can be transmitted by the highest indicated TB size, either a) the UE reverts to the legacy procedure, or b) the UE indicates a TB size to transmit the NAS message without data.

For the cases that a UE indicates the need of a certain size of Msg3, but for any reason eNB allocates smaller size in UL grant, the UE may a) consider to transmit part of the UL data with RRC and NAS message and request more UL grant or b) revert to the legacy procedure.

Proposal 9: If no PRACH resource is broadcasted for EDT or if UE intends to perform NAS signaling procedure, a UE uses the legacy RRC connection establishment/resumption procedure for data transmission.

Proposal 10: If not all UL data (RRC message, NAS message and data) can be transmitted by the highest indicated TB size, FFS either a) the UE reverts to the legacy procedure, or b) the UE indicates a TB size to transmit the NAS message without data.

Proposal 11: If UL grant size is smaller than the request by a UE, FFS either a) UE transmits part of the UL data with RRC and NAS message and request more UL grant, or b) UE reverts to the legacy procedure.

Conclusion

It is proposed to discuss and decide on the following observation and proposals:

Observation: A benefit of EDT and TB size indication can avoid the excessive transmission of padding with MSG3, which is particularly important in bad coverage.

Proposal 1: NPRACH resource partition can be used to indicate the TB size of Msg3.

Proposal 2: The TB sizes of Msg3 should include 100 Bytes.

Proposal 3: NPRACH resource to indicate TB size of Msg3 can be partitioned in the time domain.

Proposal 4: The configuration on NPRACH resource for EDT can be independent to the legacy NPRACH resource configuration.

Proposal 5: A new TB size of Msg3 association field should be added in the NPRACH parameters, e.g., nprach-TBSMsg3.

Proposal 6: eNB broadcasts NPRACH resource for EDT configurations in the system information, SIB2-NB for the anchor carrier and SIB22-NB for the non-anchor carriers.

Proposal 7: A UE determines a NPRACH resource pool indicating the TB size which can nearly accommodate RRC message size, NAS message size and data in the buffers, randomly selects a NPRACH resource from the corresponding PRACH pool, and transmits the PRACH preamble (Msg1) in the selected NPRACH resource.

Proposal 8: The UL grant field in RAR should indicate the TBS, modulation, and number of RUs for larger-sized Msg3.

Proposal 9: If no PRACH resource is broadcasted for EDT or if UE intends to perform NAS signaling procedure, a UE uses the legacy RRC connection establishment/resumption procedure for data transmission.

Proposal 10: If not all UL data (RRC message, NAS message and data) can be transmitted by the highest indicated TB size, FFS either a) the UE reverts to the legacy procedure, or b) the UE indicates a TB size to transmit the NAS message without data.

Proposal 11: If UL grant size is smaller than the request by a UE, FFS either a) UE transmits part of the UL data with RRC and NAS message and request more UL grant, or b) UE reverts to the legacy procedure.

Reference

- R2-1710001, RAN2#99 Meeting Report

- R2-1708447, UL small data transmission use cases and requirements, Veolia

- 3GPP TS 36.321 V14.4.0

- 3GPP TS 36.213 V14.3.0