3GPP TSG-RAN WG2 #99bis	R2-1711814

Prague, Czech Republic, 9 - 13 October 2017

Agenda item:	9.8.2

Source:	ESA

Title:	GNSS positioning enhancements: detailed description of SSR messages for multi GNSS PPP

WID/SID:	LCS_LTE_acc_enh-Core - Release 15

Document for:	Discussion

# 1	Introduction

In this paper, we present all standardized RTCM 10403.3 SSR messages [1] and draft SSR messages proposed in RTCM to cover SSR Phase I (basic RT – PPP) published in MADOCA – SEAD Interface Specifications [2]. Some of these messages have been mentioned in [3] but without presenting them in details.

# 2	Background

An updated work item “UE Positioning Accuracy Enhancements for LTE” [4] was approved in RAN#76 with the objective to specify solutions for positioning enhancements in LTE, including support to RTK GNSS:

- GNSS positioning enhancements: - Specify the signalling and procedure to support RTK GNSS positioning over LPP and LPPa, taking into account both UE and network complexity. [RAN2, RAN3, RAN1]

From the last RAN2 meeting, there is general agreement for:

A1. Use the RTCM Standard 10403.3 OSR messages types and data fields to support network RTK

A2. VRS should be considered as a viable network RTK technique implemented using SIB broadcast

A3. Support at least the following RTCM message types:

- Observation message types using MSM definitions (1071 – 1127)

- Station Coordinates messages (1006)

- Receiver and Antenna Descriptions (1033)

- GLONASS code – bias information (1230)

- MAC Network RTK messages (1014, 1017, 1030, 1039, 1031)

- FKP Network RTK messages (1034, 1035)

These agreements comprise a baseline and additional support can be discussed

# 3	Discussion

## 3.1	RTCM SC10403.3

There are different ways to model GNSS errors given the correction information from multiple reference stations. One ideal approach is a complete state space modelling, lately named “State Space Representation (SSR)”. All PPP – labelled technologies are using the same principle as SSR to consistently derive all individual GNSS errors in real time, bringing the accuracy to higher level.

The SSR Messages are developed by RTCM in three major steps:

- The development of messages for precise orbits, satellite clocks and satellite code biases.

- The development of vertical Total Electron Content (TEC) (VTEC) messages and Satellite Phase Biases (initially Phase 3).

- The development of slant TEC (STEC) messages, and tropospheric messages.

Table 1: SSR I messages already standardised by RTCM

|  | Message Type | Message Description |
| --- | --- | --- |
| GPS SSR | 1057 | GPS Orbit Correction |
|  | 1058 | GPS Clock Correction |
|  | 1059 | GPS Code Bias |
|  | 1060 | GPS Combined Orbit and Clock Corrections |
|  | 1061 | GPS URA |
|  | 1062 | GPS High Rate Clock Correction |
| GLONASS SSR | 1063 | GLONASS Orbit Correction |
|  | 1064 | GLONASS Clock Correction |
|  | 1065 | GLONASS Code Bias |
|  | 1066 | GLONASS Combined Orbit and Clock Correction |
|  | 1067 | GLONASS URA |
|  | 1068 | GLONASS High Rate Clock Correction |

Table 2: RTCM SSR message: green – already standardized, blue – proposed but not standardized (source: BKG/Federal Agency of Cartography and Geodesy [5]).

| System/RTCM SSR Message (* proposed) | GPS | GLONASS | Galileo | SBAS | QZSS | BDS |
| --- | --- | --- | --- | --- | --- | --- |
| Orbits | 1057 | 1063 | 1240* | 1246* | 1252* | 1258* |
| Clocks | 1058 | 1064 | 1241* | 1247* | 1253* | 1259* |
| Code Biases | 1059 | 1065 | 1242* | 1248* | 1254* | 1260* |
| Combined Orbits and Clocks | 1060 | 1066 | 1243* | 1249* | 1255* | 1261* |
| VTEC | 1264* |  |  |  |  |  |
| Phase Biases | 1265* | 1266* | 1267* | 1268* | 1269* | 1270* |
| Slant Iono (STEC) | N.A | N.A | N.A | N.A | N.A | N.A |
| Troposphere | N.A | N.A | N.A | N.A | N.A | N.A |

- SSR messages are the latest RTCM additions and expected to replace OSR techniques in the future. This papers presents a selection of Phase I messages standardized in official realeses and a selection of draft messages proposed in RTCM and published in [2].

- SSR I Orbit Correction Messages for GPS and GLONASS systems.

- Table 3: SSR GPS Orbit Correction Message 1057

| # | Name | Data Type | No. of Bits | Possible range | Contents |
| --- | --- | --- | --- | --- | --- |
| Header Part of the SSR GPS Orbit Correction Message 1057 |  |  |  |  |  |
| 1 | Message Number | Uint12 | 12 | 0 – 4095 DF002 | SSR Message Type Number (Value: 1057) |
| 2 | GPS Epoch Time 1s | Uint20 | 20 | 0 – 604799 [s] DF385 | GPS Reference Time (Full seconds since the beginning of the GPS week) |
| 3 | SSR Update Interval | Bit(4) | 4 | 0 – 15 DF391 | The SSR update intervals for this message |
| 4 | Multiple Message Indicator | Bit(1) | 1 | 0 or 1 DF388 | Indicator for transmitting message with the same Message Number and Epoch Time (1: multiple message transmitted) |
| 5 | Satellite Reference Datum | Bit(1) | 1 | DF375 |  |
| 6 | IOD SSR | Uint4 | 4 | 0 – 15 DF413 | Issue of Data number for SSR in this message |
| 7 | SSR Provider ID | Uint16 | 16 | 0 – 65535 DF414 | SSR Provider ID number |
| 8 | SSR Solution ID | Uint4 | 4 | 0 – 15 DF415 | SSR Solution ID number |
| 9 | No. of Satellites | Uint6 | 6 | 0 – 63 DF387 | Number of Satellites included in the message |
|  | Subtotal (#1 ~ #9) | 68 bits or 8.5 Bytes |  |  |  |
| Satellite Specific Part of the SSR GPS Orbit Correction Message 1057 |  |  |  |  |  |
| #10 - #17 are repeated for each satellites (1 ~ NS (No. of Satellites: see #9)) |  |  |  |  |  |
| 10 | GPS Satellite ID | Uint6 | 6 | 0 – 63 DF068 | GPS Satellite ID |
| 11 | GPS IODE | Uint8 | 8 | 0 – 31 DF071 | IODE value of broadcast ephemeris used for calculation of Correction. |
| 12 | Delta Radial | Int22 | 22 | ±209.7151 [m] DF365 | Resolution: 0.1 [mm] |
| 13 | Delta Along - Track | Int20 | 20 | ±209.7148 DF366 | Resolution: 0.4 [mm] |
| 14 | Delta Cross - Track | Int20 | 20 | ±209.7148 DF367 | Resolution: 0.4 [mm] |
| 15 | Dot Delta Radial | Int21 | 21 | ±1.048575 [m/s] DF368 | Resolution: 0.001 [mm/s] |
| 16 | Dot Delta Along - Track | Int19 | 19 | ±1.048575 [m/s] DF369 | Resolution: 0.004 [mm/s] |
| 17 | Dot Delta Cross - Track | Int19 | 19 | ±1.048575 [m/s] DF370 | Resolution: 0.004 [mm/s] |
|  | Subtotal (#10 + #17) | 135 bits or 16.875 Bytes |  |  |  |
|  | Total | 68 + 135 x NS |  |  |  |

- Table 4: SSR GLONASS Orbit Correction Message 1057

| # | Name | Data Type | No. of Bits | Possible range | Contents |
| --- | --- | --- | --- | --- | --- |
| Header Part of the SSR GLONASS Orbit Correction Message 1057 |  |  |  |  |  |
| 1 | Message Number | Uint12 | 12 | 0 – 4095 DF002 | SSR Message Type Number (Value: 1063) |
| 2 | GLONASS Epoch Time 1s | Uint20 | 17 | 0 – 86399 [s] DF386 | GLONASS Reference Time (Full seconds since the beginning of the GLO week) |
| 3 | SSR Update Interval | Bit(4) | 4 | 0 – 15 DF391 | The SSR update intervals for this message |
| 4 | Multiple Message Indicator | Bit(1) | 1 | 0 or 1 DF388 | Indicator for transmitting message with the same Message Number and Epoch Time (1: multiple message transmitted) |
| 5 | Satellite Reference Datum | Bit(1) | 1 | 0 or 1 DF375 | Orbit corrections refer to Satellite Reference Datum (0: ITRF, 1: Regional) |
| 6 | IOD SSR | Uint4 | 4 | 0 – 15 DF413 | Issue of Data number for SSR in this message |
| 7 | SSR Provider ID | Uint16 | 16 | 0 – 65535 DF414 | SSR Provider ID number |
| 8 | SSR Solution ID | Uint4 | 4 | 0 – 15 DF415 | SSR Solution ID number |
| 9 | No. of Satellites | Uint6 | 6 | 0 – 63 DF387 | Number of Satellites included in the message |
|  | Subtotal (#1 ~ #9) | 65 bits or 8.125 Bytes |  |  |  |
| Satellite Specific Part of the SSR GLONASS Orbit Correction Message 1057 |  |  |  |  |  |
| #10 - #17 are repeated for each satellites (1 ~ NS (No. of Satellites: see #9)) |  |  |  |  |  |
| 10 | GLONASS Satellite ID | Uint5 | 5 | 1 - 24 DF384 | GLONASS Satellite ID |
| 11 | GLONASS IOD | Uint(8) | 8 | 0 - 255 DF392 | IODE value of broadcast ephemeris used for calculation of Correction Differences. |
| 12 | Delta Radial | Int22 | 22 | ±209.7151 DF365 | Real orbit correction for broadcast ephemeris. Resolution: 0.1 [mm] |
| 13 | Delta Along - Track | Int20 | 20 | ±209.7148 DF366 | Along – Track orbit correction for broadcast ephemeris. Resolution: 0.4 [mm] |
| 14 | Delta Cross - Track | Int20 | 20 | ±209.7148 DF367 | Cross – Track orbit correction for broadcast ephemeris. Resolution: 0.4 [mm] |
| 15 | Dot Delta Radial | Int21 | 21 | ±1.048575 [m/s] DF368 | Velocity of Radial orbit correction for broadcast ephemeris. Resolution: 0.001 [mm/s] |
| 16 | Dot Delta Along - Track | Int19 | 19 | ±1.048572 [m/s] DF369 | Velocity of Along – Track orbit correction for broadcast ephemeris. Resolution: 0.004 [mm/s] |
| 17 | Dot Delta Cross - Track | Int19 | 19 | ±1.048572 [m/s] DF370 | Velocity of Cross – track orbit correction for broadcast ephemeris. Resolution: 0.004 [mm/s] |
|  | Subtotal (#10 + #17) | 134 bits or 16.75 Bytes |  |  |  |
|  | Total | 65 + 134 x NS |  |  |  |

- SSR I Clock Correction Messages for GPS and GLONASS systems.

- Table 5: SSR GPS Clock Correction Message 1058

| # | Name | Data Type | No. of Bits | Possible range | Contents |
| --- | --- | --- | --- | --- | --- |
| Header Part of the SSR GPS Clock Correction Message 1058 |  |  |  |  |  |
| 1 | Message Number | Uint12 | 12 | 0 – 4095 DF002 | SSR Message Type Number (Value: 1058) |
| 2 | GPS Epoch Time 1s | Uint20 | 20 | 0 – 604799 [s] DF385 | GPS Reference Time (Full seconds since the beginning of the GPS week) |
| 3 | SSR Update Interval | Bit(4) | 4 | 0 – 15 DF391 | The SSR update intervals for this message |
| 4 | Multiple Message Indicator | Bit(1) | 1 | 0 or 1 DF388 | Indicator for transmitting message with the same Message Number and Epoch Time (1: multiple message transmitted) |
| 5 | IOD SSR | Uint4 | 4 | 0 – 15 DF413 | Issue of Data number for SSR in this message |
| 6 | SSR Provider ID | Uint16 | 16 | 0 – 65535 DF414 | SSR Provider ID number |
| 7 | SSR Solution ID | Uint4 | 4 | 0 – 15 DF415 | SSR Solution ID number |
| 8 | No. of Satellites | Uint6 | 6 | 0 – 63 DF387 | Number of Satellites included in the message |
|  | Subtotal (#1 ~ #8) | 67 bits or 8.375 Bytes |  |  |  |
| Satellite Specific Part of the SSR GPS Clock Correction Message 1058 |  |  |  |  |  |
| #9 - #12 are repeated for each satellites (1 ~ NS (No. of Satellites: see #8)) |  |  |  |  |  |
| 9 | GPS Satellite ID | Uint6 | 6 | 0 – 63 DF068 | GPS Satellite ID |
| 10 | Delta Clock C0 | Int22 | 22 | DF376 |  |
| 11 | Delta Clock C1 | Int21 | 21 | DF377 |  |
| 12 | Delta Clock C2 | Int27 | 27 | DF378 |  |
|  | Subtotal (#9 + #12) | 76 bits |  |  |  |
|  | Total | 67 + 76 x NS |  |  |  |

- Table 6: SSR GLONASS Clock Correction Message 1064

| # | Name | Data Type | No. of Bits | Possible range | Contents |
| --- | --- | --- | --- | --- | --- |
| Header Part of the SSR GLONASS Clock Correction Message 1064 |  |  |  |  |  |
| 1 | Message Number | Uint12 | 12 | 0 – 4095 DF002 | SSR Message Type Number (Value: 1064) |
| 2 | GLONASS Epoch Time 1s | Uint20 | 17 | DF386 | GLONASS Reference Time (Full seconds since the beginning of the GLONASS week) |
| 3 | SSR Update Interval | Bit(4) | 4 | 0 – 15 DF391 | The SSR update intervals for this message |
| 4 | Multiple Message Indicator | Bit(1) | 1 | 0 or 1 DF388 | Indicator for transmitting message with the same Message Number and Epoch Time (1: multiple message transmitted) |
| 5 | IOD SSR | Uint4 | 4 | 0 – 15 DF413 | Issue of Data number for SSR in this message |
| 6 | SSR Provider ID | Uint16 | 16 | 0 – 65535 DF414 | SSR Provider ID number |
| 7 | SSR Solution ID | Uint4 | 4 | 0 – 15 DF415 | SSR Solution ID number |
| 8 | No. of Satellites | Uint6 | 6 | 0 – 63 DF387 | Number of Satellites included in the message |
|  | Subtotal (#1 ~ #8) | 64 bits or 8.375 Bytes |  |  |  |
| Satellite Specific Part of the SSR GLONASS Clock Correction Message 1064 |  |  |  |  |  |
| #9 - #13 are repeated for each satellites (1 ~ NS (No. of Satellites: see #8)) |  |  |  |  |  |
| 10 | GLONASS Satellite ID | Uint5 | 5 | DF384 | GLONASS Satellite ID |
| 11 | Delta Clock C0 | Int22 | 22 | DF376 |  |
| 12 | Delta Clock C1 | Int21 | 21 | DF377 |  |
| 13 | Delta Clock C2 | Int27 | 27 | DF378 |  |
|  | Subtotal (#9 + #13) | 75 bits |  |  |  |
|  | Total | 64 + 75 x NS |  |  |  |

- SSR I Code Biases Correction Messages for GPS and GLONASS systems.

Table 7: RTCM SSR message proposed for GPS Code Bias Message 1059

| # | Name | Data Type | No. of Bits | Possible range | Contents |
| --- | --- | --- | --- | --- | --- |
| Header Part of the SSR GPS Code Bias Message 1059 |  |  |  |  |  |
| 1 | Message Number | Uint12 | 12 | 0 – 4095 DF002 | SSR Message Type Number (Value: 1059) |
| 2 | GPS Epoch Time 1s | Uint20 | 20 | 0 – 604799 [s] DF385 | GPS Reference Time (Full seconds since the beginning of the GPS week) |
| 3 | SSR Update Interval | Bit(4) | 4 | 0 – 15 DF391 | The SSR update intervals for this message |
| 4 | Multiple Message Indicator | Bit(1) | 1 | 0 or 1 DF388 | Indicator for transmitting message with the same Message Number and Epoch Time (1: multiple message transmitted) |
| 5 | IOD SSR | Uint4 | 4 | 0 – 15 DF413 | Issue of Data number for SSR in this message |
| 6 | SSR Provider ID | Uint16 | 16 | 0 – 65535 DF414 | SSR Provider ID number |
| 7 | SSR Solution ID | Uint4 | 4 | 0 – 15 DF415 | SSR Solution ID number |
| 8 | No. of Satellites | Uint6 | 6 | 0 – 63 DF387 | Number of Satellites included in the message |
|  | Subtotal (#1 ~ #8) | 67 bits or 8.375 Bytes |  |  |  |
| Satellite Specific Part of the SSR GPS Satellite Code Bias Message 1059 |  |  |  |  |  |
| #9 - #12 are repeated for each satellites (1 ~ NS (No. of Satellites: see #8)) |  |  |  |  |  |
| 9 | GPS Satellite ID | Uint6 | 6 | 1 - 32 DF068 | GPS Satellite ID |
| 10 | No. of Code Biases Processed | Uint5 | 5 | 0 – 31 DF379 | Number of Code Biases for one individual satellite |
|  | Subtotal (#9 + #10) | 11 bits or 1.375 Bytes |  |  |  |
| Code Specific Part of the SSR GPS Satellite Code Bias Message 1059 |  |  |  |  |  |
|  | #11 and #12 are repeated for NCB (No. of Code Biases Processed: see #10) times |  |  |  |  |
| 11 | GPS Signal and Tracking Mode Indicator | Uint5 | 5 | 0 – 31 DF380 | No. of Code Bias Indicator to specify the GPS signal and tracking mode |
| 12 | Code Bias | Int14 | 14 | ±81.91 [m] DF383 | Code Bias for specified Signal. Resolution: 0.01 [m] |
|  | Subtotal (#11 + #12) | 19 bits or 2.375 Bytes |  |  |  |
|  | Total | 67 + 11 x NS + 19 x ΣNCB |  |  |  |

Table 8: RTCM SSR message proposed for GLONASS Code Bias Message 1065

| # | Name | Data Type | No. of Bits | Possible range | Contents |
| --- | --- | --- | --- | --- | --- |
| Header Part of the SSR GLONASS Code Bias Message 1065 |  |  |  |  |  |
| 1 | Message Number | Uint12 | 12 | 0 – 4095 DF002 | SSR Message Type Number (Value: 1065) |
| 2 | GLONASS Epoch Time 1s | Uint17 | 17 | 0 - 86399 DF386 | GLONASS Reference Time (Full seconds since the beginning of the GLONASS week) |
| 3 | SSR Update Interval | Bit(4) | 4 | 0 – 15 DF391 | The SSR update intervals for this message |
| 4 | Multiple Message Indicator | Bit(1) | 1 | 0 or 1 DF388 | Indicator for transmitting message with the same Message Number and Epoch Time (1: multiple message transmitted) |
| 5 | IOD SSR | Uint4 | 4 | 0 – 15 DF413 | Issue of Data number for SSR in this message |
| 6 | SSR Provider ID | Uint16 | 16 | 0 – 65535 DF414 | SSR Provider ID number |
| 7 | SSR Solution ID | Uint4 | 4 | 0 – 15 DF415 | SSR Solution ID number |
| 8 | No. of Satellites | Uint6 | 6 | 0 – 63 DF387 | Number of Satellites included in the message |
|  | Subtotal (#1 ~ #8) | 64 bits or 8 Bytes |  |  |  |
| Satellite Specific Part of the SSR GLONASS Satellite Code Bias Message 1065 |  |  |  |  |  |
| #9 - #12 are repeated for each satellites (1 ~ NS (No. of Satellites: see #8)) |  |  |  |  |  |
| 9 | GLONASS Satellite ID | Uint5 | 5 | 1 - 24 DF384 | GLONASS Satellite ID |
| 10 | No. of Code Biases Processed | Uint5 | 5 | 0 – 31 DF379 | Number of Code Biases for one individual satellite |
|  | Subtotal (#9 + #10) | 10 bits or 1.25 Bytes |  |  |  |
| Code Specific Part of the SSR GLONASS Satellite Code Bias Message 1065 |  |  |  |  |  |
|  | #11 and #12 are repeated for NCB (No. of Code Biases Processed: see #10) times |  |  |  |  |
| 11 | GLONASS Signal and Tracking Mode Indicator | Uint5 | 5 | 0 – 31 DF381 | No. of Code Bias Indicator to specify the GLONASS signal and tracking mode |
| 12 | Code Bias | Int14 | 14 | ±81.91 [m] DF383 | Code Bias for specified Signal. Resolution: 0.01 [m] |
|  | Subtotal (#11 + #12) | 19 bits or 2.375 Bytes |  |  |  |
|  | Total | 64 + 10 x NS + 19 x ΣNCB |  |  |  |

- SSR messages are currently supported for GPS and GLONASS orbits, clocks and code biases. A quick check of the information listed in the above tables reveals the fact that the information/data is essentially the same for GPS and GLONASS (for orbits, clocks and code biases), and these advantage can be used to generalize this common format to support additional/multiple GNSSs (currently also under development in RTCM) by adopting ASN.1 encoding rules.

## 3.2	RTCM SC10403.3 draft messages publicly available in Interface Specifications for MADOCA - SEAD

Although the official standard is not mature enough to support RTK – PPP, additional messages exist in RTCM since 2013 at proposal level [5]. Most of these draft messages, and others not mentioned here or in [3], are adopted and published in [2].

SSR I Orbit Correction Messages for Galileo, QZSS, BDS, SBAS

- Table 10: SSR Galileo Orbit Correction Message 1240

| # | Name | Data Type | No. of Bits | Possible range | Contents |
| --- | --- | --- | --- | --- | --- |
| Header Part of the SSR Galileo Orbit Correction Message 1240 |  |  |  |  |  |
| 1 | Message Number | Uint12 | 12 | 0 – 4095 DF002 | SSR Message Type Number (Value: 1240) |
| 2 | Galileo Epoch Time 1s | Uint20 | 20 | 0 – 604799 s DF458 | Galileo Reference Time (Full seconds since the beginning of the Galileo week) |
| 3 | SSR Update Interval | Bit(4) | 4 | 0 – 15 DF391 | The SSR update intervals for this message |
| 4 | Multiple Message Indicator | Bit(1) | 1 | 0 or 1 DF388 | Indicator for transmitting message with the same Message Number and Epoch Time (1: multiple message transmitted) |
| 5 | Satellite Reference Datum | Bit(1) | 1 | 0 or 1 DF375 | Orbit corrections refer to Satellite Reference Datum (0: ITRF, 1: Regional) |
| 6 | IOD SSR | Uint4 | 4 | 0 – 15 DF413 | Issue of Data number for SSR in this message |
| 7 | SSR Provider ID | Uint16 | 16 | 0 – 65535 DF414 | SSR Provider ID number |
| 8 | SSR Solution ID | Uint4 | 4 | 0 – 15 DF415 | SSR Solution ID number |
| 9 | No. of Satellites | Uint6 | 6 | 0 – 63 DF387 | Number of Satellites included in the message |
|  | Subtotal (#1 ~ #9) | 68 bits or 8.5 Bytes |  |  |  |
| Satellite Specific Part of the SSR Galileo Orbit Correction Message 1240 |  |  |  |  |  |
| #10 - #17 are repeated for each satellites (1 ~ NS (No. of Satellites: see #9)) |  |  |  |  |  |
| 10 | Galileo Satellite ID | Uint6 | 6 | 0 - 63 DF252 | Galileo Satellite ID |
| 11 | Galileo IODnav I/NAV | Uint10 | 10 | 0 - 123 DF459 | IODE value of broadcast ephemeris used for calculation of Correction Differences. |
| 12 | Delta Radial | Int22 | 22 | ±209.7151 DF365 | Real orbit correction for broadcast ephemeris. Resolution: 0.1 [mm] |
| 13 | Delta Along - Track | Int20 | 20 | ±209.7148 DF366 | Along – Track orbit correction for broadcast ephemeris. Resolution: 0.4 [mm] |
| 14 | Delta Cross - Track | Int20 | 20 | ±209.7148 DF367 | Cross – Track orbit correction for broadcast ephemeris. Resolution: 0.4 [mm] |
| 15 | Dot Delta Radial | Int21 | 21 | ±1.048575 [m/s] DF368 | Velocity of Radial orbit correction for broadcast ephemeris. Resolution: 0.001 [mm/s] |
| 16 | Dot Delta Along - Track | Int19 | 19 | ±1.048572 [m/s] DF369 | Velocity of Along – Track orbit correction for broadcast ephemeris. Resolution: 0.004 [mm/s] |
| 17 | Dot Delta Cross - Track | Int19 | 19 | ±1.048572 [m/s] DF370 | Velocity of Cross – track orbit correction for broadcast ephemeris. Resolution: 0.004 [mm/s] |
|  | Subtotal (#10 + #17) | 137 bits or 17.125 Bytes |  |  |  |
|  | Total | 68 + 137 x NS |  |  |  |

- Table 11: SSR QZSS Orbit Correction Message 1246

| # | Name | Data Type | No. of Bits | Possible range | Contents |
| --- | --- | --- | --- | --- | --- |
| Header Part of the SSR QZSS Orbit Correction Message 1246 |  |  |  |  |  |
| 1 | Message Number | Uint12 | 12 | 0 – 4095 DF002 | SSR Message Type Number (Value: 1246) |
| 2 | QZSS Epoch Time 1s | Uint20 | 20 | 0 – 604799 s DF460 | QZSS Reference Time (Full seconds since the beginning of the QZSS week) |
| 3 | SSR Update Interval | Bit(4) | 4 | 0 – 15 DF391 | The SSR update intervals for this message |
| 4 | Multiple Message Indicator | Bit(1) | 1 | 0 or 1 DF388 | Indicator for transmitting message with the same Message Number and Epoch Time (1: multiple message transmitted) |
| 5 | Satellite Reference Datum | Bit(1) | 1 | 0 or 1 DF375 | Orbit corrections refer to Satellite Reference Datum (0: ITRF, 1: Regional) |
| 6 | IOD SSR | Uint4 | 4 | 0 – 15 DF413 | Issue of Data number for SSR in this message |
| 7 | SSR Provider ID | Uint16 | 16 | 0 – 65535 DF414 | SSR Provider ID number |
| 8 | SSR Solution ID | Uint4 | 4 | 0 – 15 DF415 | SSR Solution ID number |
| 9 | No. of Satellites | Uint6 | 6 | 0 – 63 DF387 | Number of Satellites included in the message |
|  | Subtotal (#1 ~ #9) | 68 bits or 8.5 Bytes |  |  |  |
| Satellite Specific Part of the SSR QZSS Orbit Correction Message 1246 |  |  |  |  |  |
| #10 - #17 are repeated for each satellites (1 ~ NS (No. of Satellites: see #9)) |  |  |  |  |  |
| 10 | QZSS Satellite ID | Uint4 | 4 | 1 - 10 DF429 | QZSS Satellite ID |
| 11 | QZSS IODE | Uint8 | 8 | 0 - 255 DF434 | IODE value of broadcast ephemeris used for calculation of Correction Differences |
| 12 | Delta Radial | Int22 | 22 | ±209.7151 DF365 | Radial orbit correction for broadcast ephemeris. Resolution: 0.1 [mm |
| 13 | Delta Along - Track | Int20 | 20 | ±209.7148 DF366 | Along-Track orbit correction for broadcast ephemeris. Resolution: 0.4[mm] |
| 14 | Delta Cross - Track | Int20 | 20 | +209.7148 DF367 | Cross – Track orbit correction for broadcast ephemeris. Resolution: 0.4 [mm] |
| 15 | Dot Delta Radial | Int21 | 21 | ± 1.048575 [m/s] DF368 | Velocity of Radial orbit correction for broadcast ephemeris. Resolution: 0.001 [mm/s] |
| 16 | Dot Delta Along - Track | Int19 | 19 | ± 1.048572 [m/s] DF369 | Velocity of Along - Track orbit correction for broadcast ephemeris. Resolution: 0.004 [mm/s] |
| 17 | Dot Delta Cross - Track | Int19 | 19 | ± 1.048572 [m/s] DF370 | Velocity of Cross - Track orbit correction for broadcast ephemeris. Resolution: 0.004 [mm/s] |
|  | Subtotal (#10 + #17) | 133 bits or 17.125 Bytes |  |  |  |
|  | Total | 68 + 137 x NS |  |  |  |

- Table 12: SSR SBAS Orbit Correction Message 1252

| # | Name | Data Type | No. of Bits | Possible range | Contents |
| --- | --- | --- | --- | --- | --- |
| Header Part of the SSR SBAS Orbit Correction Message 1252 |  |  |  |  |  |
| 1 | Message Number | Uint12 | 12 | 0 – 4095 DF002 | SSR Message Type Number (Value: 1252) |
| 2 | SBAS Epoch Time 1s | Uint20 | 20 | DF462 | SBAS Reference Time (Full seconds since the beginning of the SBAS week) |
| 3 | SSR Update Interval | Bit(4) | 4 | 0 – 15 DF391 | The SSR update intervals for this message |
| 4 | Multiple Message Indicator | Bit(1) | 1 | 0 or 1 DF388 | Indicator for transmitting message with the same Message Number and Epoch Time (1: multiple message transmitted) |
| 5 | Satellite Reference Datum | Bit(1) | 1 | DF375 |  |
| 6 | IOD SSR | Uint4 | 4 | 0 – 15 DF413 | Issue of Data number for SSR in this message |
| 7 | SSR Provider ID | Uint16 | 16 | 0 – 65535 DF414 | SSR Provider ID number |
| 8 | SSR Solution ID | Uint4 | 4 | 0 – 15 DF415 | SSR Solution ID number |
| 9 | No. of Satellites | Uint6 | 6 | 0 – 63 DF387 | Number of Satellites included in the message |
|  | Subtotal (#1 ~ #9) | 68 bits or 8.5 Bytes |  |  |  |
| Satellite Specific Part of the SSR SBAS Orbit Correction Message 1252 |  |  |  |  |  |
| #10 - #17 are repeated for each satellites (1 ~ NS (No. of Satellites: see #9)) |  |  |  |  |  |
| 10 | SBAS Satellite ID | Uint4 | 4 | DF463 | SBAS Satellite ID |
| 11 | SBAS t0 Modulo | Bit(9) | 8 | DF468 |  |
| 12 | SBAS IOD CRC | Bit(24) | 24 | DF469 |  |
| 13 | Delta Radial | Int22 | 22 | DF365 |  |
| 14 | Delta Along - Track | Int20 | 20 | DF366 |  |
| 15 | Delta Cross - Track | Int20 | 20 | DF367 |  |
| 16 | Dot Delta Radial | Int21 | 21 | DF368 |  |
| 17 | Dot Delta Along - Track | Int19 | 19 | DF369 |  |
| 18 | Dot Delta Cross - Track | Int19 | 19 | DF370 |  |
|  | Subtotal (#10 + #17) | 160 bits or 20 Bytes |  |  |  |
|  | Total | 68 + 160 x NS |  |  |  |

- Table 13: SSR BDS Orbit Correction Message 1258

| # | Name | Data Type | No. of Bits | Possible range | Contents |
| --- | --- | --- | --- | --- | --- |
| Header Part of the SSR BDS Orbit Correction Message 1258 |  |  |  |  |  |
| 1 | Message Number | Uint12 | 12 | 0 – 4095 DF002 | SSR Message Type Number (Value: 1258) |
| 2 | BDS Epoch Time 1s | Uint20 | 20 | 0 – 604799 [s] DF465 | BDS Reference Time (Full seconds since the beginning of the BDS week) |
| 3 | SSR Update Interval | Bit(4) | 4 | 0 – 15 DF391 | The SSR update intervals for this message |
| 4 | Multiple Message Indicator | Bit(1) | 1 | 0 or 1 DF388 | Indicator for transmitting message with the same Message Number and Epoch Time (1: multiple message transmitted) |
| 5 | Satellite Reference Datum | Bit(1) | 1 | 0 or 1 DF375 | Orbit corrections refer to Satellite Reference Datum (0: ITRF, 1: Regional) |
| 6 | IOD SSR | Uint4 | 4 | 0 – 15 DF413 | Issue of Data number for SSR in this message |
| 7 | SSR Provider ID | Uint16 | 16 | 0 – 65535 DF414 | SSR Provider ID number |
| 8 | SSR Solution ID | Uint4 | 4 | 0 – 15 DF415 | SSR Solution ID number |
| 9 | No. of Satellites | Uint6 | 6 | 0 – 63 DF387 | Number of Satellites included in the message |
|  | Subtotal (#1 ~ #9) | 68 bits or 8.5 Bytes |  |  |  |
| Satellite Specific Part of the SSR BDS Orbit Correction Message 1258 |  |  |  |  |  |
| #10 - #18 are repeated for each satellites (1 ~ NS (No. of Satellites: see #9)) |  |  |  |  |  |
| 10 | BDS Satellite ID | Uint6 | 6 | 0 - 63 DF466 | BDS Satellite ID |
| 11 | BDS t0 Modulo | Bit(10) | 10 | 0 - 8184 DF470 | BDS ephemeris reference time toe modulo - 8192 |
| 12 | BDS IOD CRC | Bit(24) | 24 | 0 - 1073741823 DF471 | Issue of Data for BDS derived from CRC over broadcasted ephemeris and clock parameters. |
| 13 | Delta Radial | Int22 | 22 | ±209.7151 [m] DF365 | Radial orbit correction for broadcast ephemeris. Resolution: 0.1 [mm] |
| 14 | Delta Along - Track | Int20 | 20 | ±209.7148 [m] DF366 | Along - track orbit correction for broadcast ephemeris. Resolution: 0.4 [mm] |
| 15 | Delta Cross - Track | Int20 | 20 | ±209.7148 [m] DF367 | Cross - track orbit correction for broadcast ephemeris. Resolution: 0.4 [mm] |
| 16 | Dot Delta Radial | Int21 | 21 | ±1.048575 [m/s] DF368 | Velocity of Radial orbit correction for broadcast ephemeris. Resolution: 0.001 [mm/s] |
| 17 | Dot Delta Along - Track | Int19 | 19 | ±1.048572 [m/s] DF369 | Velocity of Along - Track orbit correction for broadcast ephemeris. Resolution: 0.001 [mm/s] |
| 18 | Dot Delta Cross - Track | Int19 | 19 | ±1.048572 [m/s] DF370 | Velocity of Cross - track orbit correction for broadcast ephemeris. Resolution: 0.001 [mm/s] |
|  | Subtotal (#10 + #18) | 161 bits or 20.125 Bytes |  |  |  |
|  | Total | 68 + 161 x NS |  |  |  |

- Analyzing data fields used to define orbit correction messages for standardized and draft RTCM messages the following can be noticed:

- The structure of Orbit Correction messages is GNSS independent and contains two parts: a header and a satellite specific part

- The header part, regardless of the GNSS system, always has 8 common fields and 1 GNSS – dependent field

| - Message Type | - Common fields/GNSS independent | - GNSS – dependent fields | - Total number of fields in the header part of any Orbit correction message |
| --- | --- | --- | --- |
| - One Orbit Correction Message – header part | - 8 fields: Message number, SSR update interval, Multiple Message Indicator, Satellite Reference Datum, IOD SSR, SSR provider ID, SSR solution ID, No of Satellites | - 1 field: this can be one of the GPS Epoch Time, GLONASS Epoch Time, Galileo Epoch Time, QZSS Epoch Time, BDS Epoch Time or SBAS Epoch Time | - 9 fields |
| - Total number of header fields to encode in ASN.1 for full mGNSS support, including QZSS, and SBAS | - 8 + 6 = 14 fields - Note, some of the GNSS – dependent fields basically share the same range and number of bits: except GLONASS Epoch Time all the other system have an allocated field with values in the same range and a size on uint20. |  |

- The satellite specific part, regardless of the GNSS system, always has 8 common fields and 1 GNSS – dependent field

| - Message Type | - Common fields/GNSS independent | - GNSS – dependent fields | - Total number of fields in the satellite specific part of any Orbit correction message |
| --- | --- | --- | --- |
| - One Orbit Correction Message – header part | - 6 fields: Delta Radial, Delta Along – Track, Delta Cross – Track, Dot Delta Radial, Dot Delta Along – Track, Dot Delta Cross – Track, | - 12 field: this can be one of the BDS Satellite ID, BDS t0 Modulo, BDS IOD CRC, GPS Satellite ID, GPS IODS, GLONASS Satellite ID, GLONASS IOD, Galileo Satellite ID, Galileo IODnav, QZSS Satellite ID, QZSS IODE, SBAS Satellite ID, SBAS t0 Modulo, SBAS CRC | - 8 or 9 fields (BDS and SBAS have each 3 GNSS-dependent fields instead of 2 fields like the other satellite constellations). |
| - Total number of satellite specific fields to encode in ASN.1 for full mGNSS support, including QZSS, and SBAS | - 6 + 12 = 18 fields |  |

- Orbit correction for all GNSS, including QZSS and SBAS, could be supported by translating to ASN.1 6 messages and 32 data fields.

- Observation 4	Furthermore a closer look at the structure of orbits, clocks and code biases messages (the last two type of corrections listed only for GPS and GLONASS to keep this contribution at a reasonable size) reveals common header for all messages regardless of the GNSS error source. This means:

- With 14 header fields, mentioned in Observation 3, orbits, clocks and code biases messages for all GNSS, including QZSS and SBAS could be supported. This is true only for the header part of each message. It seems like the header of 18 analyzed RTCM messages (6 constellations times 3 error source – orbits, clocks, code biases) can be defined with 14 fields.

- Some of the fields of Orbits messages (Satellite ID) are found also in Clocks and Code Biases messages.

- Observation 5. 	SSR messages present many common data/information to all GNSS systems, including QZSS and SBAS. This could be very useful to support a speedy standardization of SSR (partially or fully) in Release 15, especially since LPP protocol already has GNSS-ID IE and SBAS-ID that can be re-used or extended.

- Observation 6 	A similar analysis of other messages, including draft messages for Phase II, has to be conducted in order to fully benefit from the similar structure and common information/data fields of SSR messages.

# 4	Conclusion

- SSR messages are the latest RTCM additions and expected to replace OSR techniques in the future. This papers presents a selection of Phase I messages standardized in official releases and a selection of draft messages proposed in RTCM and published in [2].

- SSR messages are currently supported for GPS and GLONASS orbits, clocks and code biases. A quick check of the information listed in the above tables reveals the fact that the information/data is essentially the same for GPS and GLONASS (for orbits, clocks and code biases), and these advantage can be used to generalize this common format to support additional/multiple GNSSs (currently also under development in RTCM) by adopting ASN.1 encoding rules.

- Analyzing data fields used to define orbit correction messages for standardized and draft RTCM messages the following can be noticed:

- The structure of Orbit Correction messages is GNSS independent and contains two parts: a header and a satellite specific part

- The header part, regardless of the GNSS system, always has 8 common fields and 1 GNSS – dependent field

| - Message Type | - Common fields/GNSS independent | - GNSS – dependent fields | - Total number of fields in the header part of any Orbit correction message |
| --- | --- | --- | --- |
| - One Orbit Correction Message – header part | - 8 fields: Message number, SSR update interval, Multiple Message Indicator, Satellite Reference Datum, IOD SSR, SSR provider ID, SSR solution ID, No of Satellites | - 1 field: this can be one of the GPS Epoch Time, GLONASS Epoch Time, Galileo Epoch Time, QZSS Epoch Time, BDS Epoch Time or SBAS Epoch Time | - 9 fields |
| - Total number of header fields to encode in ASN.1 for full mGNSS support, including QZSS, and SBAS | - 8 + 6 = 14 fields - Note, some of the GNSS – dependent fields basically share the same range and number of bits: except GLONASS Epoch Time all the other system have an allocated field with values in the same range and a size on uint20. |  |

- The satellite specific part, regardless of the GNSS system, always has 8 common fields and 1 GNSS – dependent field

| - Message Type | - Common fields/GNSS independent | - GNSS – dependent fields | - Total number of fields in the satellite specific part of any Orbit correction message |
| --- | --- | --- | --- |
| - One Orbit Correction Message – header part | - 6 fields: Delta Radial, Delta Along – Track, Delta Cross – Track, Dot Delta Radial, Dot Delta Along – Track, Dot Delta Cross – Track, | - 12 field: this can be one of the BDS Satellite ID, BDS t0 Modulo, BDS IOD CRC, GPS Satellite ID, GPS IODS, GLONASS Satellite ID, GLONASS IOD, Galileo Satellite ID, Galileo IODnav, QZSS Satellite ID, QZSS IODE, SBAS Satellite ID, SBAS t0 Modulo, SBAS CRC | - 8 or 9 fields (BDS and SBAS have each 3 GNSS-dependent fields instead of 2 fields like the other satellite constellations). |
| - Total number of satellite specific fields to encode in ASN.1 for full mGNSS support, including QZSS, and SBAS | - 6 + 12 = 18 fields |  |

- Orbit correction for all GNSS, including QZSS and SBAS, could be supported by translating to ASN.1 6 messages and 32 data fields.

- Observation 4	Furthermore a closer look at the structure of orbits, clocks and code biases messages (the last two type of corrections listed only for GPS and GLONASS to keep this contribution at a reasonable size) reveals common header for all messages regardless of the GNSS error source. This means:

- With 14 header fields, mentioned in Observation 3, orbits, clocks and code biases messages for all GNSS, including QZSS and SBAS could be supported. This is true only for the header part of each message. It seems like the header of 18 analyzed RTCM messages (6 constellations times 3 error source – orbits, clocks, code biases) can be defined with 14 fields.

- Some of the fields of Orbits messages (Satellite ID) are found also in Clocks and Code Biases messages.

- Observation 5. 	SSR messages present many common data/information to all GNSS systems, including QZSS and SBAS. This could be very useful to support a speedy standardization of SSR (partially or fully) in Release 15, especially since LPP protocol already has GNSS-ID IE and SBAS-ID that can be re-used or extended.

- Observation 6 	A similar analysis of other messages, including draft messages for Phase II, has to be conducted in order to fully benefit from the similar structure and common information/data fields of SSR messages.

# References

- RTCM Standard 10403.3 Differential GNSS Services – Version 3 – RTCM Special Committee No. 104, October 7, 2016

- Interface Specification for MADOCA – SEAD - Japan Aerospace Exploration Agency, February 2017

- RP – 1710536 GNSS Positioning enhancements: ways forward to support SSR concept in Release 15 - ESA.

- RP-171508 UE Positioning Accuracy Enhancements for LTE – 3GPP Work Item Description

- Axel Rulke, Precise Point Positioning at BKG, German Federal Agency, BKG, 2016 https://www.lantmateriet.se/globalassets/kartor-och-geografisk-information/gps-och-matning/nkg2016/precise-point-positioning-at-bkg.pdf