The following modelling alternatives were discussed to replace “NR-UNIT” in NR MAC specifications TS 38.321

Alt 1 - Single terminology - TTI duration

- adapt TTI to accommodate data transmission duration, PDCCH monitoring periodicity for active coreset(s), and PUSCH resource periodicity. meaning depends on the context.

- for procedures that do not involve multiple numerologies (e.g. RA), TTI refers to the slot of the default numerology.

- For scenarios where TTI is ambiguous, use subframe or a reference time unit that has a period in absolute time.

Alt 2 - Multiple terminologies

- use PRACH occasion in RACH procedure

- use TTI duration to refer to PUSCH/PDSCH durations

- use PDCCH occasion (or PDCCH slot) in procedures when referring to the PDCCH monitoring

- Use TTI duration in UL procedures when referring to the PUCCH resource periodicity

- use ms for DRX timers

- subframes to refer to a 1 ms period, with 10 subframes aligned within radio frame boundaries

Alt 3 - Event Driven

- Write the MAC specification in an event-driven manner.

General guidance for all alternatives:

- Minimize using TTI for data transmission durations if possible, for example.

1>          identify the HARQ process(es) associated with this TTI/'NR-UNIT', and for each TB of each identified HARQ process:

1>          identify the HARQ process associated with each TB

# Summary:

- Use PRACH occasion in RACH procedure

- Use PDCCH occasion in procedures when referring to the PDCCH monitoring.

- Subframes to refer to a 1 ms period, with 10 subframes aligned within radio frame boundaries

- FFS: Use reference time interval (e.g. reference slot) in UL procedures when referring to the PUCCH resource periodicity

- FFS: Time restrictions in LCP.