3GPP TSG-RAN WG2 NR Ad Hoc	R2-1700194

Spokane, USA, 17th – 19th January 2017

Source:	CATT

Title:	NR UP stack options for intra-gNB multi-connectivity

Agenda Item:	3.2.1.1

Document for:	Discussion and Decision

# Introduction

In RAN2#95, RAN2 provided a definition for multi-connectivity as follows:

Multi-Connectivity: Mode of operation whereby a multiple Rx/Tx UE in the connected mode is configured to utilise radio resources amongst E-UTRA and NR provided by multiple distinct schedulers connected via a non-ideal backhaul.

In this contribution we focus on intra-gNB multi-connectivity where the distinct schedulers are located in the distributed units (DU) of the gNB and where such DUs are connected to the gNB’s central unit (CU) via a fronthaul. We propose several UP stack improvements in support of the various fronthaul split options identified by RAN3.

# Discussion

## Fronthaul split options

RAN3 identified 8 fronthaul split options as illustrated in Figure 1. Given the above multi-connectivity definition, the candidate fronthaul options addressing multi-connectivity are those where the CU/DU split is above the MAC layer, namely options 1-4. Option 1 reflects the LTE DC option 1A where the bearers are split in the CN, which does not have any impact on the UP stack. From the leftover options involving split in the UP stack, the most relevant options are Options 2 and 3. Although Option 3 is further divided into 3.1 and 3.2, we only discuss Option 3.1 in this contribution. Figure 2 illustrates the multi-connectivity architecture with the 2 split options. We analyze in the following sections the potential improvements enabled by both architectures.

Figure 1: RAN3 fronthaul split options

Figure 2: Multi-connectivity with fronthaul split options 2 and 3.1

## Common improvements

Some potential improvements can be envisioned to the legacy UP stack in support of multi-connectivity as e.g. suggested in [1][2]: support of both RLC AM and UM, introduction of packet duplication in support of URLLC [3][4] as well as leg switching. Such improvements should be studied irrespective of the selected architecture.

Proposal 1: Both AM and UM modes shall be supported in intra-NR multi-connectivity

Proposal 2: Duplication and switching shall be supported on top of split bearer in intra-NR multi-connectivity

## UP-stack with fronthaul split Option 2

This is the legacy DC option 3C where the split occurs in PDCP. The main benefit of this option is that legacy stack can be re-used with potential improvements, as mentioned in Section 2.2. However, as discussed in the following section, we believe it is not best suited for the new challenges brought up in NR, specifically operations in high frequency (HF).

## UP-stack with fronthaul split Option 3.1

As shown in Figure 3 for the DL (UL is similar), this fronthaul split allows 2 options for locating the routing function in the UP-stack:

- Option 1: PDCP is the routing layer between DUs

- Option 2: RLC-high is the routing layer between DUs

Figure 3: Routing options with fronthaul split Option 3.1: in PDCP (left), in RLC (right)

Locating the routing function in the RLC provides a number of benefits elaborated in our companion contributions [5][6]. They can be summarized as follows:

- Can react quicker to HF leg failure than flow control mechanism

- Enables tight flow control without requiring PDCP involvement (so no cross-layer interaction).

- Does not require resetting RLC during cross-leg handover

- RLC SN allocation can be made independently from (and ahead of) the routing decision

- Only one single RLC ARQ and status report process is needed independently of the number of supported legs

- Allows independent routing of retransmissions from previous transmission(s), e.g.:

  - switching legs for retransmissions

  - duplicating retransmissions across legs

- Allows sending status reports in any leg, independently of the receiving leg, including e.g. duplicating reports across legs to improve the ARQ control messages reliability

We believe the above features will be key in addressing the new challenges resulting from operating in HF links which are well known for their abrupt and unpredictable signal losses [4]-[8]. The resulting NR UP stack is as depicted in Figure 4 where a routing function is added in the RLC sub-layer and the ARQ retransmission function is re-located above the segmentation for alignment with RAN3 split option 3.1:

Figure 4: Overall L2 structure for NR

Proposal 4: A routing function including splitting, switching and duplication in the RLC sub-layer shall be studied in support of intra-NR multi-connectivity

Proposal 5: Adopt the text proposal in Annex

# Conclusion

This contribution proposes some new UP stack functions in support of intra-gNB multi-connectivity for the two relevant fronthaul split options defined by RAN3. The associated proposals are:

Proposal 1: It shall be possible to support intra-gNB multi-connectivity over more than 2 DUs

Proposal 2: Both AM and UM modes shall be supported in intra-NR multi-connectivity

Proposal 3: Duplication and switching shall be supported on top of split bearer in intra-NR multi-connectivity

Proposal 4: A routing function including splitting, switching and duplication in the RLC sub-layer shall be studied in support of intra-NR multi-connectivity

Proposal 5: Adopt the text proposal in Annex

# References

- R2-167583, “Support of Multi-Connectivity in NR”, Huawei, HiSilicon

- R2-167665, “Packet Data Convergence Sublayer for NR”, Nokia, Alcatel-Lucent Shanghai Bell

- R2-1700192, “Packet duplication across legs”, CATT, Alcatel-Lucent Shanghai Bell, Nokia

- R2-1700193, “Performance analysis of packet duplication” CATT

- R2-1700195, “Location of RLC ARQ in NR”, CATT

- R2-1700196, “Comparisons of central ARQ and flow control feedback”, CATT

- R2-168423, “Fast Radio Measurement Filtering for Multi-Connectivity”, Nokia, Alcatel-Lucent Shanghai Bell

- R2-166100, “High Frequency NR RRC Mobility”, Samsung Electronics Co., Ltd

- R2-164755, “Implications of High Frequency Bands on Mobility”, Nokia, Alcatel-Lucent Shanghai Bell

- R2-165585, “General aspects for NR HF cell”, Huawei, HiSilicon

- R2-163515, “Implications of High Frequency Bands on Mobility”, Nokia, Alcatel-Lucent Shanghai Bell

# Annex: Text proposal to TR 38.804

### 5.4.1	Overview of Layer 2 functions

Overall layer 2 structure comprised of order and placement of layer 2 functions is illustrated in Figure 5.4.1-1. Each layer 2 function is served by the corresponding layer 2 sublayer described in 5.4.2, 5.4.3 and 5.4.4.

Editor’s note: These are baseline functions and can be changed according to the outcome of this study.

Figure 5.4.1-1:	Overall layer 2 structure for NR

Editor’s note: having concatenation in MAC only is the working assumption.

<< skip unrelated part >>

### 5.4.3	RLC Sublayer

The main services and functions of the RLC sublayer include:

-	Transfer of upper layer PDUs;

-	Error Correction through ARQ (only for AM data transfer)

-	RLC PDU routing (in case of split, duplicated and switch bearers);

-	Segmentation and resegmentation [FFS: of PDU or SDU];

-	Reassembly of SDU;