3GPP TSG-RAN WG2 NR Ad Hoc	R2-1700211

Spokane, USA, 17th – 19th January 2017

Source:	CATT

Title:	Content and Delivery of Minimum SI

Agenda Item:	3.2.2.4

Document for:	Discussion and Decision

# Introduction

Broadcast overhead reduction has been one of the key aspects studied in the NR system information topic. Two main and complementary approaches have been discussed: allowing part of the system information to be acquired by UEs on-demand rather than being permanently broadcasted, and the use of an index on a set of stored configurations rather than delivering the full IE set. The former approach reduces the periodic broadcast to the “minimum system information” required for camping and initial access. In this contribution we analyze the contents of the minimum system information and suggest making use of the index/table technique for broadcasting it.

# Discussion

## Essential SI in LTE

The content of LTE MIB, SIB1 and SIB2 as well as the system information required for camping and initial access is provided in the Annex. This essential system information is broadcasted periodically in LTE. When, for some essential IEs (e.g. plmn-IdentityList, cellSelectionInfo, ue-TimersAndConstant), there are only few configurations being used most of the time in real networks, it costs a large overhead to broadcast the whole IEs periodically. As shown in the Annex, such kind of IEs occupies more than 60% overhead of the total SI needed for camping and initial access (based on LTE SIBs).

Observation: In LTE, the periodic broadcast of some essential IEs in system information can cost more than 60% unnecessary overhead, although these IEs are practically used with few configurations most of the time in real networks.

Therefore, in NR, it is proposed to study the enhancement for the minimum SI to reduce the broadcast overhead.

Proposal 1: In NR, the broadcast of the minimum SI needs to be enhanced, to reduce the overhead caused by the explicit broadcast of some essential IEs.

## Minimum SI in NR

In previous RAN2 meetings, some contributions [1]-[5] suggested to use indexed SI/stored SI mechanism to improve the broadcast of system information. This mechanism can be used for minimum SI as e.g. suggested in [6]. However, considering the specifics of minimum SI, only part of it can use the indexed SI mechanism e.g. SFN needs to be explicit, and some part of minimum SI shall be provisioned to allow UE acquiring a table of minimum SI configurations in first place. Such table can be provided via periodic broadcast [6] or via on-demand. It is expected that such table is valid over a large area so that the UE needs to acquire it very infrequently.

Therefore, the minimum SI could be split into two parts:

Minimum SI configurations Table SI: if periodically broadcasted, this is the table itself, or if available on-demand, this is all (explicit) parameter values necessary for UE to acquire the minimum SI table of configurations, referred to as “Table acquisition SI”. As mentioned above and further discussed in Section 2.3, since this SI is mostly needed at power-up, the latency for acquiring it is not critical and it can be broadcasted with a large period.

Remaining part of minimum SI: index to the minimum SI table + other explicit values (which cannot be coded as an index)

The following table gives the content and the grouping of IEs in minimum SI. A SI is categorized as indexed if only a subset of the total configurations coded in the explicit IE are expected to be used in practice over a given area. The classification is assumed conservative to leave a large flexibility in configuring cells and further indexing could be used (e.g. for RACH, SRS, etc).

Table 1 Content of minimum SI

| SI group | SI | Indexed / explicit |
| --- | --- | --- |
| L1 parameters for camping | SFN | Explicit |
|  | DL bandwidth | Indexed |
|  | Antenna configuration | Indexed |
|  | Duplex mode (FDD/TDD) | Explicit |
|  | TDD cfg | Explicit |
|  | Numerology | Indexed |
| Other parameters for camping | PLMN list | Indexed |
|  | TAC | Explicit |
|  | Cell ID | Explicit |
|  | Barring | Indexed |
|  | Cell selection parameters | Indexed |
|  | Paging parameters | Indexed |
| TRP/beam related parameters [7] | Beam-specific RS exists | Explicit |
|  | Beam-specific RS subbands | Indexed |
|  | Sequence ranges | Indexed |
|  | PDCCH configurations | Indexed |
| RACH parameters | Details FFS | Explicit |
| Uplink L1 parameters | Frequency | Explicit |
|  | Bandwidth | Explicit |
|  | Numerology | Indexed |
|  | PUSCH | Indexed |
|  | PUCCH | Indexed |
|  | SRS | Explicit |
|  | Power control | Indexed |
| Parameters needed to acquire a minimum SI configuration table via on-demand or broadcast | On-demand parameters | Explicit |
|  | Broadcast parameters | Explicit |
| Parameters related to presence/request/delivery of other SI | Other SI availability bitmap | Explicit |
|  | Configuration for requesting other SI (eg. preambles) | Explicit |
|  | Scheduling information for broadcasted SIs | Indexed |
| Value tag | Per SI | Explicit |
| UE-TimersAndConstants, TA timer | T300, T301, N311, TA timer, etc. | Indexed |

From the above table 1, it can be seen that most of the parameters in the minimum SI can use an index-based delivery mechanism. Therefore, it is proposed:

Proposal 2: Use indexed SI mechanism for part of the minimum SI.

Proposal 2a: The details of which parameters in minimum SI should use indexed SI mechanism are left to stage 3.

Proposal 3: The table of minimum SI configurations can be broadcasted periodically or delivered on-demand.

## Delivery of minimum SI

Next, how to deliver the minimum SI needs some further study. An example is shown in figure 1.

Figure 1 how to deliver the minimum SI

NR-PBCH carries cell-common SI [7], and it is split into subsets with different periods. The most frequent NR-PBCH subsets should also be the lightest and carry the minimum SI indexes as well as explicit values of parameters that cannot be coded as an index (e.g. SFN). Infrequent NR-PBCH subsets should contain the largest payloads, e.g. minimum SI Table (in case of periodic broadcast) or Table acquisition SI. The Table acquisition SI repeats in explicit values some of the parameters coded in the NR-PBCH1 indexes (e.g. DL BW, antenna cfg, …). However, the final decision of how to design the PBCH depends on RAN1.

Proposal 4: Two subsets of cell-common physical channel are used to deliver the minimum SI, one subset for the indexes and explicit values, and the other one for the minimum SI Table or Table acquisition SI. The details are left to RAN1.

# Conclusion

This contribution discusses content and delivery of minimum SI. Associated observation and proposals are:

Observation: In LTE, the periodically broadcast of some essential IEs in system information can cost more than 60% unnecessary overhead, when these IEs have only several kinds of configurations in the real network.

Proposal 1: In NR, the broadcast of the minimum SI needs to be enhanced, to reduce the overhead caused by the explicit broadcast of some essential IEs.

Proposal 2: Use indexed SI mechanism for part of the minimum SI.

Proposal 2a: The details of which parameters in minimum SI should use indexed SI mechanism are left to stage 3.

Proposal 3: The table of minimum SI configurations can be broadcasted periodically or delivered on-demand.

Proposal 4: Two subsets of cell-common physical channel are used to deliver the minimum SI, one subset for the indexes and explicit values, and the other one for the minimum SI Table or Table acquisition SI. The details are left to RAN1.

# References

- R2-168086	Comparison of index based approaches	Samsung India	discussion

- R2-168148	Stored System Information for OSI	MediaTek Inc.	discussion

- R2-168286	Index based provisioning of system information in NR	Ericsson	discussion

- R2-168417	SI validity for broadcast SI and on-demand SI	LG Electronics Inc.	discussion

- R2-168530	Area based and index based system information provisioning	Intel Corporation	discussion

- R2-168287  Content and distribution of Minimum SI	Ericsson

- R2-1700209  Cell-common and TR/beam-specific system information	CATT

# Annex

Table 2 Content and classification of MIB/SIB1/SIB2

| Information Element |  | Maximum Length (bits) | Required for Camping and Initial access (minimum SI) |  | Only few configurations in real networks |  |
| --- | --- | --- | --- | --- | --- | --- |
| MIB |  |  |  |  |  |  |
| dl-Bandwidth |  | 3 | 3 |  | Y |  |
| phich-Config |  | 3 | 3 |  | Y |  |
| >phich-Duration |  | 1 |  |  |  |  |
| >phich-Resource |  | 2 |  |  |  |  |
| systemFrameNumber |  | 8 | 8 |  |  |  |
| Sum |  | 14 | 14 |  | 6 |  |
| SIB1 |  |  |  |  |  |  |
| cellAccessRelatedInfo | 224 |  |  | 224 |  |  |
| >plmn-IdentityList | 6*25=180 |  |  |  |  | Y |
| >trackingAreaCode | 16 |  |  |  |  |  |
| >cellIdentity | 28 |  |  |  |  |  |
| >cellBarred | 1 |  |  |  |  |  |
| >intraFreqReselection | 1 |  |  |  |  |  |
| >csg-Indication | 1 |  |  |  |  |  |
| >csg-Identity | 27 |  |  |  |  |  |
| cellSelectionInfo | 9 |  |  | 9 |  | Y |
| >q-RxLevMin | 6 |  |  |  |  |  |
| >q-RxLevMinOffset | 3 |  |  |  |  |  |
| p-Max | 6 |  |  | 6 |  | Y |
| freqBandIndicator | 5 |  |  | 5 |  |  |
| schedulingInfoList | 416 |  |  |  |  |  |
| tdd-Config | 7 |  |  | 7 |  |  |
| si-WindowLength | 3 |  |  |  |  |  |
| systemInfoValueTag | 5 |  |  |  |  |  |
| Sum | 672 |  |  | 251 |  | 195 |
| SIB2 |  |  |  |  |  |  |
| ac-BarringInfo | 25 |  |  | 25 |  |  |
| >ac-BarringForEmergency | 1 |  |  |  |  |  |
| >ac-BarringForMO-Signalling | 12 |  |  |  |  |  |
| >ac-BarringForMO-Data | 12 |  |  |  |  |  |
| radioResourceConfigCommon | 149 |  |  |  |  |  |
| >rach-ConfigCommon | 37 |  |  | 37 |  |  |
| >bcch-Config | 2 |  |  |  |  |  |
| >pcch-Config | 5 |  |  |  |  |  |
| >prach-Config | 13 |  |  | 13 |  |  |
| >pdsch-ConfigCommon | 9 |  |  | 9 |  | Y |
| >pusch-ConfigCommon | 21 |  |  | 21 |  | Y |
| >pucch-ConfigCommon | 23 |  |  |  |  |  |
| >soundingRS-UL-ConfigCommon | 10 |  |  |  |  |  |
| >uplinkPowerControlCommon | 28 |  |  |  |  |  |
| >ul-CyclicPrefixLength | 1 |  |  | 1 |  |  |
| ue-TimersAndConstant | 18 |  |  |  |  |  |
| >t300 | 3 |  |  | 3 |  | Y |
| >t301 | 3 |  |  |  |  |  |
| >t310 | 3 |  |  |  |  |  |
| >n310 | 3 |  |  |  |  |  |
| >t311 | 3 |  |  |  |  |  |
| >n311 | 3 |  |  |  |  |  |
| freqInfo | 24 |  |  | 24 |  |  |
| >ul-CarrierFreq | 16 |  |  |  |  |  |
| >ul-Bandwidth | 3 |  |  |  |  | Y |
| >additionalSpectrumEmission | 5 |  |  |  |  | Y |
| mbsfn-SubframeConfigList | 8*36 |  |  |  |  |  |
| timeAlignmentTimerCommon | 3 |  |  | 3 |  | Y |
| Sum | 507 |  |  | 113 |  | 44 |

The IEs in the right-most column are those IEs required for camping and initial access and that are typically reduced to a few sets of configurations in real network deployments, so are typical candidate for being delivered as an index on a set of configurations. When broadcasted explicitly as in LTE, they are shown to occupy more than 60% overhead of the camping and initial access SI [(6+195+44)/(14+251+113)=64%].