3GPP TSG-RAN WG2 #AH	Tdoc R2-1700427

# Spokane, Washington, USA, 17th – 19th January 2017

Agenda Item:	3.2.1.2

Source:	Ericsson (Rapporteur)

Title:	Email discussion (29) report for TP on QoS

Document for:	Decision

# Introduction

This paper provides a text proposal to TR 38.804 on QoS for NR.

# History of RAN2 Agreements:

RAN2#94:

Agreements

1: The "data radio bearer" (DRB) defines the Over-The-Air packet treatments in the RAN.

2: A DRB serves a set of packets requiring the same packet forwarding treatment, e.g. reliability, target delay, etc.

3: A separate DRB is defined for each different packet forwarding treatment required.

RAN2#95:

Agreements

1	For DL for a non-GBR flow, the eNB sees an indication over NG-u and based on the indication the eNB maps the packet to a DRB of an appropriate QoS.

RAN2 understanding of SA2 agreements is that eNB has a QoS profile associated with the indication.

FFS whether there is a requirement for every different QoS indication to be mapped to a different radio bearer.

2	Functionality is required to differentiate flows from different PDN-connections over the radio interface (e.g. by using separate DRBs or by an explicit indication in a header)

3	For DL, the eNB establishes DRBs for the UE taking the QoS profiles in to account.

FFS how the DRB is established in the first packet is an UL packet.

=>	FFS whether there is a requirement for GBR flows and non GBR flows to be mapped to different DRBs.

RAN2#95bis:

Agreements

1:	RAN determines the mapping relationship between QoS flow (as determine by the UE in UL or marked by the CN in DL) and DRB for UL and DL.

1a	RAN can map multiple QoS flows to a DRB.

2	Specification will not forbid a GBR flow and non-GBR flow to be mapped to the same DRB, but we will not introduce mechanisms to optimise this case.

3	Specification will not forbid more than one GBR flow to be mapped to the same DRB, but we will not introduce mechanisms to optimise this case.

FFS: Whether traffic from different PDU sessions can be mapped to one DRB or not.

Agreements

1	Default DRB is established by eNB at PDU session establishment (or an existing DRB may be used if mapping of more than one session to a DRB is allowed)

2. 	If the first packet of the flow is UL packet, if no mapping rule is configured in the UE, the packet is sent through default DRB to the network.

FFS How and when the network can remap the flow to more appropriate DRB.

FFS the first packet is handled in the case that pre-authorised QoS is configured

FFS whether the pre-authorised QoS applies to RAN or only to the UE.

FFS whether there is a single level of mapping from UL TFT (5 tuple) to DRB, or whether there is a 2 level mapping from UL TFT to QoS flow and then from QoS flow to DRB.

RAN2#96:

Agreement

1:	Traffic from different PDU sessions are mapped to different DRBs

2:	In DL we have a 2-step mapping of IP flows, in which NAS is responsible for the IPflow->QOSflow mapping, and AS is responsible for the QOSflow->DRB mapping (confirmation of SA2 agreement status).

3:	In UL we have a 2-step mapping of IP flows, in which NAS is responsible for the IPflow->QOSflow mapping, and AS is responsible for the QOSflow->DRB mapping.

4	DL packets over Uu are marked in band with QOS-flow-id for the purposes of reflective QoS

5	UL packets over Uu are marked in band with QOS-flow-id for the purposes of marking forwarded packets to the CN.

FFS for bullets 4 and 5 whether it can be semi-statically configured to not include the QOS flow ID in some cases.

FFS for bullets 4 and 5 whether it might be possible to use a shorter id over the radio compared to that received from the CN. This is a stage 3 issue.

Agreements

1	For reflective QoS, the UE determines QoS Flow ID to DRB mapping in the uplink based on the downlink packets received within a DRB and applies those filters for mapping uplink Flows to DRBs.

2	The UE "continuously" monitors the QoS Flow ID in downlink PDCP packets and updates the reflective QoS Flow ID to DRB mapping in the uplink accordingly.

3	RRC can configure an uplink mapping

FFS The precedence of the RRC configured mapping and reflective QoS (e.g. can reflective QoS update an RRC configured mapping)

Working assumption:

If an incoming UL packet does not match a QoS Flow ID to DRB mapping (neither a configured nor a determined via reflective QoS), the UE shall map that packet to the default DRB of the PDU session.

=>	FFS whether the QoS field is added by PDCP or a new protocol layer above PDCP.

# TP on QoS for TR

# 8	QoS control

## 8.1	QoS architecture in NR and NextGen Core

Editor’s note:	 intended to capture overall architecture in NR and NextGen Core something like sub-clause 13.1 in TS 36.300.

The CN may indicate to the RAN one or more PDU Sessions that have been established. The RAN maps packets belonging to different PDU sessions to different DRBs. Hence, the RAN establishes at least one default DRB for each PDU Session indicated by the CN upon PDU Session establishment.

5G-CN and 5G-RAN ensure quality of service (e.g. reliability and target delay) by appropriate packet treatment. In NR, the data radio bearer (DRB) defines the packet treatment on the radio interface (Uu).

A DRB serves packets with the same packet forwarding treatment. Separate DRBs may be established for QoS flows requiring different packet forwarding treatment.

For both uplink and downlink, the RAN determines the mapping of QoS flows to DRBs, whereas NAS is responsible for the mapping of IP-flows to QoS flows. Hence there is a 2-step mapping of IP-flows to QoS flows (NAS) and from QoS flows to DRBs (Access Stratum).

Within each PDU session, how to map multiple QoS flows to a DRB is up to RAN. The RAN may map a GBR flow and a non-GBR flow, or more than one GBR flow to the same DRB, but mechanisms to optimise these cases are not within the scope of standardization.

In the downlink, the mapping of packets on DRBs is done by the RAN based on NGU3 marking and QoS profiles.

In the uplink, the RAN may control the mapping in two different ways:

- Reflective QoS: for each DRB, the UE monitors the QoS flow ID(s) of the downlink packets and applies the same mapping in the uplink; that is, for a DRB, the UE maps the uplink packets belonging to the QoS flows(s) corresponding to the QoS flow ID(s) spotted in the downlink for that DRB. To enable this reflective QoS mapping, the RAN marks downlink packets over Uu with QoS flow ID. The UE marks uplink packets over Uu with the QoS flow ID for the purposes of marking forwarded packets to the CN;

Editor’s Note: It is FFS whether the marking with a QoS flow ID can be semi-statically configured (to not include the QOS flow ID when not needed).

Editor’s Note: It is FFS whether the QoS Flow ID field is added by PDCP or a new protocol layer above PDCP

- Explicit Configuration: besides the reflective QoS mapping, the RAN may configure by RRC an uplink “QoS Flow ID to DRB mapping”.

Editor’s Note: The precedence of the RRC configured mapping and reflective QoS is FFS (can reflective QoS update and thereby override an RRC configured mapping? Or does a configured QoS Flow ID to DRB mapping always take precedence over a reflective mapping?)

[Working assumption:] If an incoming UL packet matches neither an RRC configured nor a reflective “QoS Flow ID to DRB mapping”, the UE shall map that packet to the default DRB of the PDU session.

--- End Text Proposal --

# Conclusion

Adopt the text proposal in section 2 as the outcome to the email discussion.

# References

[1]	Chairman’s note, RAN2 Chairman, RAN2#94-96